| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0481 |
| --- | --- |

| Title: | AHG 17: On output layers and output layer sets. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin Karsten Sühring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to add output layers and output layer sets to VVC for layered video and subpictures.

Syntax to bundle layers in the VPS is proposed in addition to a decoding mode signalling for each output layer set to differentiate three modes of decoder operation when encountering layered bitstreams. The decoding mode specifically allows to differentiate that:

- only NAL units of one layer of the layers belonging to a layer group are decoded per access unit when decoding the layer group

- NAL units all layers of the layers belonging to the a layer group are decoded per access unit when decoding the layer group

# Introduction

With the introduction of subpictures and layers to the VVC specification, it is asserted that a concept as output layer and output layer sets, as defined in HEVC, would be very useful.

It is asserted that with output layer sets, conformance can be defined for a set of layers that do not necessarily follow a straightforward dependency path, which is always the case for subpictures if contained in separate layers in the same bitstream and might be the case for some multi-view use cases, e.g., where the left and right views are independent from each other but dependent on the same base layer.

It further allows to indicate the layers that are output per output layer set, since it might be the case that for one output layer set one layer is output while in another output layer set, to which the same layer belongs, the same layer is not an output layer.

Although, it is asserted that it is beneficial to provide a solution that uses single-layer decoding for subpictures (e.g., straight-forward output synchronization), in immersive use-cases the number of subpictures might be too high to be included within the same layer if limits are set to the number of subpictures during profile level discussions. Therefore, separating the subpictures into layers might be a potential solution. However, it would be valuable to know whether several subpictures can be decoded as a single-layer decoding process.

In addition, in many cases, subpictures may share the same parameter sets and therefore, they could be stored in a separate layer that is applicable to some or all subpicture, while decoding the layer with parameter sets and the subpicture layer as if they were the same layer.

Also, for multi-resolution scenarios with RPR, one could envision the same video to be present within the same bitstream at different resolutions. In SHVC this was achieved by resampling the base layer at a given picture as illustrated in the following picture where the blue pictures are L0 and the red pictures L1, the blue bounding box subsumes a pure L0 decoding path and the red bounding box subsumes a decoding path transitioning from L0 to L1.

For VVC with RPR, there is no need to have a picture where both the base and enhancement layer are present as shown in the following where a lower layer picture is used as upsampled reference.

The scenarios illustrated above could happen in a DASH system with Open GOP switch.

During the last JVET meeting JVET-O0497 was discussed, were a grouping mechanism was proposed to indicate that single-layer decoding could be achieved for several layers indicating whether:

- only NAL units of one layer of the layers belonging to the i-th Layer Group are decoded per access unit when decoding the i-th Layer Group

- NAL units all layers of the layers belonging to the i-th Layer Group are decoded per access unit when decoding the i-th Layer Group

In this proposal the concept described in JVET-O0497 is completed and merged with output sets.

# Proposal

It is proposed to add the following definitions:

output layer: A layer of an output layer set that is output when TargetOlsIdx is equal to the index of the output layer set.

output layer set (OLS): A set of layers consisting of the layers of the specified layer sets, where one or more layers in the set of layers are indicated to be output layers.

Layers of the CVS are bundled into output layer sets and a syntax element vps_decode_layers_mode[ i ] is added to indicate three different operations on the OLS.vps_decode_layers_mode[ i ] equal to 0 corresponds to the typical multi-layer case, where layers are decoded with or without inter-layer dependency but each of the layers has a nuh_layer_id at the NUH that is used for decoding the multi-layer bitstream.

vps_decode_layers_mode[ i ] equal to 1 corresponds to the a resolution change use case, using RPR, where two layers are available. In the example below 2 layer are depicted L0 and L1. At picture#4, there is a resolution change and since with vps_decode_layers_mode[ i ] equal to 1 NALUs with nuh_layer_id L0 or L1 are interpreted as if they had a common nuh_layer_id LX, it can be decoded using a single layer decoder using RPR. Note that only a value of layer_id is decoded per AU. E.g., for picture#4 only L1 is decoded and not L0.

vps_decode_layers_mode[ i ] equal to 2 corresponds to a sub-picture use case where each sub-picture is stored as a separate layer. In the following figure for instance Output Layer Set OS1 would consist of Layers L0, L1, L4 and L5. All these layers could be decoded with a single decoder given that they are properly encoded (i.e. they are MCTS or sub-pictures). Note also that the OS1 should additionally include a nuh_layer_id that contains the right parameter sets that correspond to the “merged” 4 sub-pictures into a single picture.

# Proposed specification text changes

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(6) |
| if( vps_max_layers_minus1 > 0 ) |  |
| vps_all_independent_layers_flag | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(6) |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |
| vps_independent_layer_flag[ i ] | u(1) |
| if( !vps_independent_layer_flag[ i ] ) |  |
| for( j = 0; j < i; j++ ) |  |
| vps_direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| if( vps_max_layers_minus1 > 0 ) |  |
| vps_num_layer_sets_minus1 | ue(v) |
| for( i = 0; i <= vps_num_layer_sets_minus1; i++ ) { |  |
| vps_decode_layers_mode[ i ] | u(2) |
| vps_output_layers_mode[ i ] | u(2) |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) { |  |
| layer_idx_included_flag[ i ][ j ] | u(1) |
| if( vps_output_layers_mode = = 2 && layer_idx_included_flag[ i ][ j ]) |  |
| vps_output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| vps_constraint_info_present_flag | u(1) |
| vps_reserved_zero_7bits | u(7) |
| if( vps_constraint_info_present_flag ) |  |
| general_constraint_info( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_decode_layers_mode[ i ] equal to 0 specifies that a decoder decoding the i-th output layer set of the bitstream should decode all NAL units belonging to the i-th output layer set obeying the indicated nuh_layer_id values. vps_decode_layers_mode[ i ] equal to 1 specifies that a decoder decoding the i-th output layer set of the bitstream should only decode, within an AU, all NAL units of a single value of nuh_layer_id belonging to the i-th output layer set and interpreting the nuh_layer_id values of the decoded NAL units as equal to 0. vps_decode_layers_mode[ i ] equal to 2 specifies that a decoder decoding the i-th output layer set of the bitstream should decode, within an AU, all NAL units belonging to the i-th output layer set and interpreting the nuh_layer_id values of the decoded NAL units as equal to 0.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).