| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0482 |
| --- | --- |

| Title: | AHG8: On reference picture resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin Karsten Suehring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes two aspects related to reference picture resampling:

- Implicit and signalled constraints for reference picture resampling.

- Region of interest (RoI) and extended scalability support.

# Introduction

Reference Picture Resampling (RPR) allows an encoder to change the coded picture size within the coded video sequence, e.g. in live communication scenarios without RAPs or in streaming scenarios at RAPs with leading pictures. In such a case reference pictures are to be resampled to the correct resolution, i.e. the resolution of the current picture.

However, carrying out RPR in addition to normal inter-prediction incurs a processing burden compared to “normal” inter-prediction without RPR. An implementor would have to significantly overprovision its decoder hardware in light of unrestricted RPR to perform simultaneous resampling at any arbitrary current picture (every picture) of all pictures in the DPB from various resolutions and so on. Therefore, it is clearly desirable to limit the implementation complexity of RPR to cover the most useful use cases for RPR.

For video conferencing scenarios, RPR would typically apply at time instants at which a change on the available throughput happens or at the beginning of the session where a RAP needs to be transmitted. In that cases, the intra-coded RAP can be transmitted/encoded at a reasonable size in term of bytes with a lower resolution than the target resolution desired for the service.

For streaming scenarios, RPR would be typically used to support Open GOP switching points when different resolutions are used for bitrate adaptation.

In any of the cases, it is envisioned that by not using a lot of reference pictures at a different resolution than the current picture, RPR would already provide a big benefit. This assertion leads to determining that the number of reference pictures with a different resolution to the current picture should be limited, as well as the potential number of different scaling factors for a single picture with respect to its reference pictures to keep complexity at a reasonable level.

In addition to the constraints that ensure practicability/feasibility of RPR, it is asserted that the current approach of RPR does not support RoI scalability and extended scalability, illustrated in the following figure, which is considered a valuable feature:

# Proposal

The proposal is twofold:

## Aspect 1: Add constraints to RPR

  - If a the current picture uses a reference picture with a scaling factor F different than 1, the scaling factor for all other reference pictures of the current picture shall be  equal to 1 or equal to F.

  - Add a constraint in the VUI to limit the number of reference pictures that are to be resampled:

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = 255 ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coeffs | u(8) |
| full_range_flag | u(1) |
| } |  |
| field_seq_flag | u(1) |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| if( !field_seq_flag ) { |  |
| chroma_sample_loc_type_frame | ue(v) |
| else { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| max_num_resampled_ref_pics | ue(v) |
| } |  |
| } |  |

max_num_resampled_ref_pics indicates the maximum number of resampled reference pictures that might be required for any coded picture in the CVS.

Additionally, a level constraint should be added to limit the maximum value of max_num_resampled_ref_pics.

## Aspect 2: Add syntax that allows for RoI and Extended scalability with the RPR functionality

An important difference needs to be considered in case of RoI scalability compared to the approach in SHVC. In SHVC, RoI scalability applied within the same AU, which means that the corresponding content used for prediction was simply the collocated samples in the reference picture of a reference layer. However, for RPR, the reference picture is at a different time instance. So the corresponding content can lay outside the corresponding samples in the reference layer (depicted in the following figure) and only move into the region of interest at the resolution change. Thus it would be beneficial to allow an extended reference region to be used for prediction.

In this proposal the RoI region is signalled as for SHVC so that the scaling factor can be computed properly. In addition, syntax is added to indicate and additional bounding box from which samples are allowed to be used for prediction.

| pic_parameter_set( ) { | Descriptor |
| --- | --- |
| … |  |
| ref_region_offset_present_flag | u(1) |
| if( ref_region_offset_present_flag ) { |  |
| ref_region_left_offset | ue(v) |
| ref_region_top_offset | ue(v) |
| ref_region_right_offset | ue(v) |
| ref_region_bottom_offset | ue(v) |
| } |  |
| additional_roi_offset_present_flag | u(1) |
| if( additional_roi_offset_present_flag) { |  |
| additional_roi_left_offset | ue(v) |
| additional_roi_top_offset | ue(v) |
| additional_roi_right_offset | ue(v) |
| additional_roi_bottom_offset | ue(v) |
| } |  |

In order to support extended scalability the syntax existing in SHVC is suggested.

| pic_parameter_set( ) { | Descriptor |
| --- | --- |
| … |  |
| scaled_ref_offset_present_flag | u(1) |
| if( scaled_ref_offset_present_flag ) { |  |
| scaled_ref_left_offset | ue(v) |
| scaled_ref_top_offset | ue(v) |
| scaled_ref_right_offset | ue(v) |
| scaled_ref_bottom_offset | ue(v) |
| } |  |
| } |  |

Note that the ref_region_X_offset and scaled_ref_X_offset syntax elements have been changed compared to SHVC to ue(v), as we could not find justification for coding them as se(v).

The computation of the scaling factors is changed as follow:

- If ref_region_offset_present_flag is equal to 1, the scaling factor is computed taking into account the cropping window in the current picture and the reference region offsets in the reference picture.

- If scaled_ref_offset_present_flag is equal to 1, the scaling factor is computed taking into account the scaled_ref_X_offset values for the current picture and the reference picture without the cropping window.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).