| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0485 |
| --- | --- |

| Title: | AHG18: Performance of CE7-1.2d and CE7-2.2 on lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Alican Nalci, Hongtao Wang, Muhammed Coban, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the transform skip modification proposed in CE7-1.2d and the transform skip binarization proposed in CE7-2.2a for lossless. The two methods are implemented on top of method 1 in JVET-P0082, which enables transform skip mode and BDPCM for lossless coding. The results show:

Method 1:

-2.72% AI, -1.28% RA, xxx% LB

Method 2:

-3.30% AI, -1.69% RA, xxx% LB

# Proposed Method

## Method 1

In CE7-1.2d [1], a method of Rice Golomb coding is applied to encode the transform skip residuals after the context coded bin limit is reached. If there’s no context bin that can be used for any of the component of a coefficient, the abs(level) is coded using Rice-Golomb coding and followed by a bypass coded sign flag. The rice parameter is derived as follows for this case:

auiGoRicePars[std::min(sum, 31)]

Otherwise, if one or more bins for a coefficient in the 1st pass is context coded but there’s no context bin for any component in for the coefficient in the 2nd pass. abs(level) – 1 is coded using Rice-Golomb coding. The rice parameter is derived as follows for this case:

auiGoRicePars[std::min(sum + 4, 31)]

If one or more bins for a coeffcient in the 2nd path is context coded, the remainder (if gt9 flag is 1) is coded using Rice-Golomb coding, where the rice parameter is derived as follows:

auiGoRicePars[ std::min(std::max(sum -8, 0), 31) ]

In this proposal, the method of CE7-1.2d-alternative is tested on top of the lossless coding proposed in JVET-P0082 [3], where transform skip mode and BDPCM are enabled for lossless.

## Method 2

In CE7-2.2a [2], a new binarization is proposed for transform skip residual coding, where the binarization is adjusted depending on the dynamic range of the coefficient. The binarization is also tested on top of the Rice-Golomb coding after bypass switch proposed in CE7-1.2d (CE7-2.2a-alternative). In this proposal, CE7-2.2a-alternative is tested on top of the lossless coding proposed in JVET-P0082 [3], where transform skip mode and BDPCM are enabled for lossless.

# Simulation Results

In the simulation, the anchor is AHG18 VTM-6.0 Lossless branch. The following settings are used to run both the anchor and test: TransquantBypassEnable : 1, CUTransquantBypassFlagForce : 1, CostMode : lossless, InternalBitDepth : 0. QP value was set as 0. Additionally, BDPCM is set to be 1 for the test. Table 1 and 2 shows the results of method 1 and 2.

Table 1: Results of method 1 (CE7-1.2d-alternative on lossless coding)

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | Anchor | Method1 |  | Anchor | Method1 |  | Anchor | Method1 |  |
| Class A1 | 2.2 | 2.3 | -1.03% | 2.3 | 2.3 | -0.42% |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.74% | 1.9 | 1.9 | -0.42% |  |  |  |
| Class B | 2.1 | 2.1 | -1.57% | 2.3 | 2.3 | -0.29% | 2.3 | 0.0 | -500.00% |
| Class C | 2.1 | 2.1 | -1.81% | 2.6 | 2.6 | -0.38% | 2.6 | 2.6 | -0.40% |
| Class D | 1.9 | 2.0 | -3.21% | 2.8 | 2.8 | -0.62% | 2.7 | 2.8 | -0.53% |
| Class E | 2.9 | 3.0 | -2.89% |  |  |  | 3.3 | 3.4 | -1.01% |
| Class F | 5.5 | 5.8 | -4.51% | 39.4 | 41.5 | -3.10% | 60.7 | 84.1 | -37.24% |
| TGM | 12.3 | 12.9 | -4.66% | 112.6 | 116.7 | -3.52% | 142.4 | 148.1 | -3.61% |
| Overall | 2.2 | 2.2 | -2.72% | 2.3 | 2.3 | -1.28% | 2.6 | 1.7 | -26.37% |
| Enc Time[%] | 108% |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 94% |  |  | #NUM! |  |  | #NUM! |  |  |

Table 2: Results of method 1 (CE7-2.2a-alternative on lossless coding)

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | Anchor | Method 2 |  | Anchor | Method 2 |  | Anchor | Method 2 |  |
| Class A1 | 2.2 | 2.3 | -1.00% | 2.3 | 2.3 | -0.36% |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.73% | 1.9 | 1.9 | -0.37% |  |  |  |
| Class B | 2.1 | 2.1 | -1.62% | 2.3 | 2.3 | -0.29% | 2.3 | 0.0 | -500.00% |
| Class C | 2.1 | 2.1 | -1.73% | 2.6 | 2.6 | -0.34% | 2.6 | 2.6 | -0.36% |
| Class D | 1.9 | 2.0 | -3.10% | 2.8 | 2.8 | -0.56% | 2.7 | 2.8 | -0.47% |
| Class E | 2.9 | 3.0 | -2.73% |  |  |  | 3.3 | 3.4 | -0.99% |
| Class F | 5.5 | 5.8 | -5.01% | 39.4 | 42.0 | -3.46% | 60.7 | 84.7 | -37.59% |
| TGM | 12.3 | 13.5 | -8.80% | 112.6 | 120.7 | -6.14% | 142.4 | 130.6 | -40.41% |
| Overall | 2.2 | 2.2 | -3.30% | 2.3 | 2.3 | -1.69% | 2.6 | 1.7 | -30.85% |
| Enc Time[%] | 110% |  |  | 111% |  |  | #NUM! |  |  |
| Dec Time[%] | 96% |  |  | 100% |  |  | #NUM! |  |  |

# Reference

- M. Karczewicz, H. Wang, Y.-H. Chao, M. Coban, “CE7: Modifications to transform-skip residual coding (CE7-1.2)”, JVET-P0079, 15th meeting, Gothenburg, SE, July 2019.

- M. Karczewicz, H. Wang, Y.-H. Chao, M. Coban, “CE7-2.2: QP dependent binarization in TS residual coding”, JVET-P0076, 15th meeting, Gothenburg, SE, July 2019.

- A. Nalci, H. Wang, H. E. Egilmez, M. Coban, M. Karczewicz, “AHG18: BDPCM for Lossless”, JVET-P0082, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)