| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0495-v2 |
| --- | --- |

| Title: | Non-CE6: On implicit MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang, Hongbin Liu, Li Zhang, Kai Zhang, Zhipin Deng, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | wangyang.cs@bytedance.com liuhongbin.01@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

In the current design of VVC, implicit MTS may be applied on ISP-coded blocks, SBT-coded blocks and normal-intra coded blocks with different rules. Implicit MTS is controlled together for all the three cases. When sps_mts_enabled_flag is equal to 0, only DCT2 can be used in all the three cases, which is inflexible and deteriorates the coding performance of ISP and SBT. In this contribution, two aspects are proposed to control implicit MTS in three cases separately. Two flags are signalled in SPS to indicate whether non-DCT2 transform kernels are allowed for ISP and SBT, respectively. Simulation results reportedly show BD-rate changes of -0.37%/-0.35%/-0.34% in AI/RA/LB configurations on average, compared to VTM-6.0 with MTS disabled.

Introduction

With Multiple Transform Selection (MTS), transform kernels can be selected from DCT2/DST7/DCT8 adaptively. MTS could be enabled in either explicit or implicit ways. For the explicit MTS, intra and inter MTS are controlled separately. When it is enabled for a TU, an index (tu_mts_idx) is signalled to specify which transform kernels are selected. For the implicit MTS, transform kernels are selected without signalling tu_mts_idx. In VVC D6, implicit MTS may be applied in three cases with different rules:

- Case #1: On ISP-coded blocks.

- Case #2: On SBT-coded blocks.

- Case #3: On normal-intra coded blocks.

In the current design, implicit MTS is controlled together for all the three cases. When sps_mts_enabled_flag is equal to 0, only DCT2 can be used in all the three cases, which is inflexible and deteriorates the coding performance of ISP and SBT.

Proposed methods

Two aspects are proposed to control implicit MTS in three cases separately.

Aspect #1: A flag is signalled in SPS to indicate whether non-DCT2 transform kernels are allowed for ISP, independent on sps_mts_enabled_flag.

Aspect #2: A flag is signalled in SPS to indicate whether non-DCT2 transform kernels are allowed for SBT, independent on sps_mts_enabled_flag.

With the proposed method, implicit MTS can be controlled more precisely. And the coding gain of ISP and SBT can be kept when we do not want to apply MTS on normal-intra coded blocks.

Simulation results

The proposed methods have been implemented on top of VTM6.0 [1]. Simulations are performed following the common test conditions [2].

Performance of ISP

Following two tests are performed to verify the coding performance of ISP with different sets of transforms:

- Test #1: ISP using DCT2 transform kernels only.

Tested:   VTM6.0 with “--MTS=0”

Anchor:  VTM6.0 with “--MTS=0, ISP=0”

- Test #2: ISP using multiple transform kernels.

Tested:   Aspect #1 with “--MTS=0”

Anchor:  VTM6.0 with “--MTS=0”

The test results are provided in Table 1 and Table 2, respectively. As can be seen, ISP can achieve -0.32% luma BD-rate gain on average if only DCT2 transform kernels are used, and additional -0.37% luma BD-rate gain can be achieved if multiple transform kernels are used. That is, using multiple transforms could double the coding gain of ISP with minor running time increase.

Table 1: Coding performance of ISP with DCT2 (Test #1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM-6.0 --MTS=0 over VTM-6.0 --MTS=0 --ISP=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.15% | 0.11% | 116% | 101% |
| Class A2 | -0.15% | 0.01% | 0.03% | 117% | 102% |
| Class B | -0.33% | -0.10% | -0.14% | 119% | 102% |
| Class C | -0.52% | -0.33% | -0.16% | 120% | 101% |
| Class E | -0.48% | -0.22% | -0.03% | 120% | 104% |
| Overall | -0.32% | -0.11% | -0.06% | 119% | 102% |
| Class D | -0.50% | -0.43% | -0.43% | 125% | 103% |
| Class F (optional) | -0.58% | -0.11% | -0.05% | 108% | 103% |

Table 2: Coding performance of ISP with DCT2/DST7/DCT8 (Test #2)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Proposed over VTM-6.0 --MTS=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.08% | 0.08% | 100% | 99% |
| Class A2 | -0.34% | -0.50% | -0.55% | 103% | 101% |
| Class B | -0.38% | -0.51% | -0.45% | 103% | 99% |
| Class C | -0.32% | -0.32% | -0.55% | 103% | 101% |
| Class E | -0.66% | -0.80% | -0.84% | 103% | 100% |
| Overall | -0.37% | -0.44% | -0.47% | 102% | 100% |
| Class D | -0.28% | -0.26% | -0.35% | 102% | 102% |
| Class F (optional) | -0.15% | -0.30% | -0.22% | 102% | 102% |

Performance of SBT

Following two tests are performed to verify the coding performance of SBT:

- Test #3: SBT using DCT2 transform kernels only.

Tested:   VTM6.0 with “--MTS=0”

Anchor:  VTM6.0 with “--MTS=0, SBT=0”

- Test #4: SBT using multiple transform kernels.

Tested:   Aspect #2 with “--MTS=0”

Anchor:  VTM6.0 with “--MTS=0”

The test results are provided in Table 3 and Table 4, respectively. As can be seen, SBT can achieve -0.2%/-0.35 % luma BD-rate gain for RA/LDB on average if only DCT2 transform kernels are used, and additional -0.19%/-0.29% luma BD-rate gain for RA/LDB can be achieved if multiple transform kernels are used. That is, using multiple transforms could double the coding gain of SBT without increasing running time.

Table 3: Coding performance of SBT with DCT2 (Test #3)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM-6.0 --MTS=0 over VTM-6.0 --MTS=0 --SBT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.11% | -0.07% | 104% | 100% |
| Class A2 | -0.25% | -0.26% | -0.23% | 104% | 96% |
| Class B | -0.24% | 0.00% | -0.06% | 107% | 101% |
| Class C | -0.24% | 0.08% | -0.02% | 106% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.05% | -0.08% | 105% | 100% |
| Class D | -0.30% | -0.01% | -0.22% | 107% | 107% |
| Class F (optional) | -0.11% | -0.09% | -0.14% | 104% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | VTM-6.0 --MTS=0 over VTM-6.0 --MTS=0 --SBT=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.38% | -0.22% | -0.10% | 108% | 99% |
| Class C | -0.38% | -0.24% | -0.26% | 107% | 98% |
| Class E | -0.27% | 0.25% | 0.37% | 103% | 99% |
| Overall | -0.35% | -0.11% | -0.04% | 106% | 99% |
| Class D | -0.37% | -0.22% | -0.63% | 108% | 102% |
| Class F (optional) | -0.17% | -0.19% | 0.30% | 104% | 100% |

Table 4: Coding performance of SBT with DCT2/DST7/DCT8 (Test #4)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Proposed over VTM-6.0 --MTS=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.07% | 0.14% | 99% | 99% |
| Class A2 | -0.26% | 0.06% | 0.08% | 100% | 101% |
| Class B | -0.20% | -0.01% | 0.07% | 99% | 98% |
| Class C | -0.20% | 0.02% | 0.04% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | 0.03% | 0.08% | 99% | 98% |
| Class D | -0.17% | 0.07% | -0.01% | 99% | 97% |
| Class F (optional) | -0.04% | 0.12% | 0.10% | 99% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Proposed over VTM-6.0 --MTS=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.37% | 0.05% | 0.20% | 100% | 101% |
| Class C | -0.26% | 0.20% | 0.14% | 99% | 99% |
| Class E | -0.20% | 0.35% | 0.11% | 99% | 96% |
| Overall | -0.29% | 0.17% | 0.16% | 99% | 99% |
| Class D | -0.23% | 0.35% | 0.51% | 99% | 96% |
| Class F (optional) | -0.17% | 0.03% | -0.61% | 100% | 97% |

Performance of ISP and SBT

Following test is performed to verify the coding performance of ISP and SBT:

- Test #5: ISP and SBT using multiple transform kernels.

Tested:   Aspect #1 + Aspect #2 with “--MTS=0”

Anchor:  VTM6.0 with “--MTS=0”

The test results are provided in Table 5. As can be seen, the proposed method can achieve -0.37%/-0.35%/-0.34% in AI/RA/LB configurations on average, compared to VTM-6.0 with MTS disabled.

Table 5: Coding performance of Aspect #1 and Aspect #2 (Test #5)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Proposed over VTM-6.0 --MTS=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.08% | 0.08% | 100% | 99% |
| Class A2 | -0.34% | -0.50% | -0.55% | 103% | 101% |
| Class B | -0.38% | -0.51% | -0.45% | 103% | 99% |
| Class C | -0.32% | -0.32% | -0.55% | 103% | 101% |
| Class E | -0.66% | -0.80% | -0.84% | 103% | 100% |
| Overall | -0.37% | -0.44% | -0.47% | 102% | 100% |
| Class D | -0.28% | -0.26% | -0.35% | 102% | 102% |
| Class F (optional) | -0.15% | -0.30% | -0.22% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Proposed over VTM-6.0 --MTS=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.03% | -0.01% | 101% | 100% |
| Class A2 | -0.39% | -0.11% | -0.07% | 100% | 100% |
| Class B | -0.40% | -0.39% | -0.32% | 101% | 101% |
| Class C | -0.35% | -0.31% | -0.32% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | -0.24% | -0.21% | 101% | 100% |
| Class D | -0.32% | -0.05% | -0.07% | 100% | 98% |
| Class F (optional) | -0.21% | -0.07% | -0.19% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Proposed over VTM-6.0 --MTS=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | -0.15% | -0.07% | 101% | 100% |
| Class C | -0.32% | 0.06% | 0.05% | 101% | 101% |
| Class E | -0.22% | 0.64% | 0.13% | 99% | 97% |
| Overall | -0.34% | 0.12% | 0.02% | 100% | 100% |
| Class D | -0.33% | 0.22% | 0.52% | 101% | 103% |
| Class F (optional) | -0.14% | 0.08% | -0.51% | 100% | 100% |

Conclusions

It is proposed to enable implicit MTS for ISP and SBT when sps_mts_enabled_flag is equal to 0. It is recommended adopting the proposed method to the next version of VVC.

Proposed changes to working draft

The suggested specification changes on top of JVET-O2001_vE is described as follows: deleted text is marked with strikethrough, and newly added text are highlighted in green.

7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_isp_enabled_flag | u(1) |
| if (sps_isp_enabled_flag) |  |
| sps_isp_non_dct2_enabled_flag | u(1) |
| … |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag ) { |  |
| sps_sbt_max_size_64_flag | u(1) |
| sps_sbt_non_dct2_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

8.7.4 Transformation process for scaled transform coefficients

8.7.4.1 General

…

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- sps_isp_non_dct2_enabled_flag is equal to 1 and IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- sps_sbt_non_dct2_enabled_flag is equal to 1 and cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_mts_enabled_flag is equal to 1 and sps_explicit_mts_intra_enabled_flag is equal to 0 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA and lfnst_idx[ x0 ][ y0 ] is equal to 0 and intra_mip_flag[ x0 ][ y0 ] is equal to 0

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT or sps_explicit_mts_intra_enabled_flag is equal to 0 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA, trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4 && nTbW <= 16 ) ? 1 : 0	(8-975)

trTypeVer = ( nTbH >= 4 && nTbH <= 16 ) ? 1 : 0	(8-976)

- Otherwise (cu_sbt_flag is equal to 1), trTypeHor and trTypeVer are specified in Table 8-15 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-14 depending on tu_mts_idx[ xTbY ][ yTbY ].

…

Reference

- VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- JVET-N1010, “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).