| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0497 |
| --- | --- |

| Title: | AHG17: On LMCS Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chiao Yang Chih-Yu Teng Po-Han Lin | Tel: Email: | yuciaoyang@fginnov.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

Parameters of luma mapping with chroma scaling (LMCS) piecewise linear model is described in the LMCS APS. The delta sign flag is one of parameters and has signalling redundancy due to the valid range of delta codeword. This contribution proposes two methods to reduce the signalling of LMCS APS. In the first method, a condition check is added to remove the redundancy in delta sign flag of delta codeword. In the second method, two additional reference flags are signalled in the LMCS APS to indicate that the sign of all the delta codewords are the same, instead of signalling the delta sign flag (lmcs_delta_sign_cw_flag[ i ]) of each delta codeword.

Introduction

The signalling of piecewise linear model of luma mapping is described in JVET-O2001-vE [1] as follows:

| lmcs_data () { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| for ( i = lmcs_min_bin_idx; i <= LmcsMaxBinIdx; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if ( lmcs_delta_abs_cw[ i ] > 0 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| } |  |

lmcs_min_bin_idx specifies the minimum bin index used in the luma mapping with chroma scaling construction process. The value of lmcs_min_bin_idx shall be in the range of 0 to 15, inclusive.

lmcs_delta_max_bin_idx specifies the delta value between 15 and the maximum bin index LmcsMaxBinIdx used in the luma mapping with chroma scaling construction process. The value of lmcs_delta_max_bin_idx shall be in the range of 0 to 15, inclusive. The value of LmcsMaxBinIdx is set equal to 15 − lmcs_delta_max_bin_idx. The value of LmcsMaxBinIdx shall be greater than or equal to lmcs_min_bin_idx.

lmcs_delta_cw_prec_minus1 plus 1 specifies the number of bits used for the representation of the syntax lmcs_delta_abs_cw[ i ]. The value of lmcs_delta_cw_prec_minus1 shall be in the range of 0 to BitDepthY − 2, inclusive.

lmcs_delta_abs_cw[ i ] specifies the absolute delta codeword value for the ith bin.

lmcs_delta_sign_cw_flag[ i ] specifies the sign of the variable lmcsDeltaCW[ i ] as follows:

- If lmcs_delta_sign_cw_flag[ i ] is equal to 0, lmcsDeltaCW[ i ] is a positive value.

- Otherwise ( lmcs_delta_sign_cw_flag[ i ] is not equal to 0 ), lmcsDeltaCW[ i ] is a negative value.

When lmcs_delta_sign_cw_flag[ i ] is not present, it is inferred to be equal to 0.

The variable OrgCW is derived as follows:

OrgCW = (1 << BitDepthY ) / 16	(7-52)

The variable lmcsDeltaCW[ i ], with i = lmcs_min_bin_idx..LmcsMaxBinIdx, is derived as follows:

lmcsDeltaCW[ i ] = ( 1 − 2 * lmcs_delta_sign_cw_flag[ i ] ) * lmcs_delta_abs_cw[ i ]	(7-53)

The variable lmcsCW[ i ] is derived as follows:

- For i = 0.. lmcs_min_bin_idx − 1, lmcsCW[ i ] is set equal 0.

- For i = lmcs_min_bin_idx..LmcsMaxBinIdx, the following applies:

lmcsCW[ i ] = OrgCW + lmcsDeltaCW[ i ]	(7-54)

The value of lmcsCW[ i ] shall be in the range of (OrgCW>>3) to (OrgCW<<3 − 1), inclusive.

Proposed

In the current VVC draft, the delta codeword is in the range of ((OrgCW>>3) – OrgCW) to ((OrgCW<<3 − 1) – OrgCW), inclusive, where OrgCW is corresponding to the luma bit-depth. Therefore, the maximum valid range of delta codeword can be derived in the range of -3584 to 28761, inclusive, because maximum luma bit-depth supported in VVC is 16-bit.

In the current approach, the delta sign flag is signalled in each of delta codeword. However, the delta codeword can be directly derived as a positive value if the absolute delta codeword is greater than a threshold. In the first method, a condition check is added to remove the signalling redundancy. The threshold in the condition check can be derived based on the minimum delta codeword:

Threshold = Abs( ((OrgCW>>3) – OrgCW) )

= Abs( 2(BitDepthY-7) – 2(BitDepthY-4) )

The threshold depends on the luma bit-depth parsed in the SPS. However, LMCS APS should be independent of SPS. Therefore, considering the maximum luma bit-depth, the threshold is set equal to 3584.

On the other hand, in the current approach of piecewise linear model, there are 16 delta sign flags at most can be signalled in the LMCS APS. Moreover, the probability of case that delta sign flag of delta codeword are all the same is high under the AI/RA test conditions, and the probability is reported in Table 1. In the second method, this contribution provides two additional reference flags to achieve adaptive signalling in LMCS APS if all the delta sign flags are represented as positive or negative.

Table 1. Probability of the delta sign flag direction in LMCS APS

| Test Condition | Total number of LMCS APS | Probability of sign direction |  |  |
| --- | --- | --- | --- | --- |
|  |  | All positive | All negative | Others |
| All Intra | 6628 | 86.72% | 13.28% | 0.00% |
| Random Access | 1460 | 97.12% | 0.82% | 2.05% |

Proposed Syntax Change on LMCS

Method 1

The proposed condition check in the first method is added in the LMCS APS as follows (highlighted in yellow):

| lmcs_data () { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| for ( i = lmcs_min_bin_idx; i <= LmcsMaxBinIdx; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if ( lmcs_delta_abs_cw[ i ] > 0 && lmcs_delta_abs_cw[ i ] <= 3584 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| } |  |

Method 2

The proposed two reference sign flags in the second method is added in the LMCS APS as follows (highlighted in yellow):

| lmcs_data () { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| lmcs_reference_sign_signalled_flag | u(1) |
| if (lmcs_reference_sign_signalled_flag ) |  |
| lmcs_reference_sign_flag | u(1) |
| for ( i = lmcs_min_bin_idx; i <= LmcsMaxBinIdx; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if ( lmcs_delta_abs_cw[ i ] > 0 && !lmcs_reference_sign_signalled_flag ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| } |  |

lmcs_min_bin_idx specifies the minimum bin index used in the luma mapping with chroma scaling construction process. The value of lmcs_min_bin_idx shall be in the range of 0 to 15, inclusive.

lmcs_delta_max_bin_idx specifies the delta value between 15 and the maximum bin index LmcsMaxBinIdx used in the luma mapping with chroma scaling construction process. The value of lmcs_delta_max_bin_idx shall be in the range of 0 to 15, inclusive. The value of LmcsMaxBinIdx is set equal to 15 − lmcs_delta_max_bin_idx. The value of LmcsMaxBinIdx shall be greater than or equal to lmcs_min_bin_idx.

lmcs_delta_cw_prec_minus1 plus 1 specifies the number of bits used for the representation of the syntax lmcs_delta_abs_cw[ i ]. The value of lmcs_delta_cw_prec_minus1 shall be in the range of 0 to BitDepthY − 2, inclusive.

lmcs_reference_sign_signalled_flag equal to 1 indicates the lmcs_reference_sign_flag is signalled. lmcs_reference_sign_signalled_flag equal to 0 indicates the lmcs_reference_sign_flag is not signalled.

lmcs_reference_sign_flag specifies the reference sign of all the variable lmcs_delta_abs_cw[i] if the lmcs_reference_sign_signalled_flag is equal to 1.

- If lmcs_reference_sign_flag is equal to 0, all of the variable lmcsDeltaCW[ i ] are positive values.

- Otherwise ( lmcs_reference_sign_flag is not equal to 0 ), all of the variable lmcsDeltaCW[ i ] are negative values.

lmcs_delta_abs_cw[ i ] specifies the absolute delta codeword value for the ith bin.

lmcs_delta_sign_cw_flag[ i ] specifies the sign of the variable lmcsDeltaCW[ i ] as follows:

- If lmcs_delta_sign_cw_flag[ i ] is equal to 0, lmcsDeltaCW[ i ] is a positive value.

- Otherwise ( lmcs_delta_sign_cw_flag[ i ] is not equal to 0 ), lmcsDeltaCW[ i ] is a negative value.

When lmcs_delta_sign_cw_flag[ i ] is not present, it is inferred to be equal to 0.

The variable OrgCW is derived as follows:

OrgCW = (1 << BitDepthY ) / 16	(7-52)

The variable lmcsDeltaCW[ i ], with i = lmcs_min_bin_idx..LmcsMaxBinIdx, is derived as follows:

- If the lmcs_reference_sign_signalled_flag is equal to 0, the following applies:

lmcsDeltaCW[ i ] = ( 1 − 2 * lmcs_delta_sign_cw_flag[ i ] ) * lmcs_delta_abs_cw[ i ]	(7-53)

- Otherwise (lmcs_reference_sign_signalled_flag is equal to 1), the following applies

lmcsDeltaCW[ i ] = ( 1 − 2 * lmcs_reference_sign_flag ) * lmcs_delta_abs_cw[ i ]	(7-54)

The variable lmcsCW[ i ] is derived as follows:

- For i = 0.. lmcs_min_bin_idx − 1, lmcsCW[ i ] is set equal 0.

- For i = lmcs_min_bin_idx..LmcsMaxBinIdx, the following applies:

lmcsCW[ i ] = OrgCW + lmcsDeltaCW[ i ]	(7-55)

The value of lmcsCW[ i ] shall be in the range of (OrgCW>>3) to (OrgCW<<3 − 1), inclusive.

Experimental result

The test condition described in JVET-N1010 [2] is used in the simulation. There is no experimental result is provided in the first method as there is no 16-bit test sequence in the current CTC. Table 2 shows the simulation results of the second method.

Table 2. Summary of experimental results (Second method over VTM-6.0)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class E | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.02% | -0.02% | -0.02% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |

Conclusion

This contribution proposes two methods on LMCS signalling. In the first method, a condition check is added to remove the signalling redundancy. In the second method, additional reference flags can achieve adaptive signalling when all of the delta sign flag are the same. Experimental results show that no overall YUV-BD rate change in AI, but -0.02 YUV-BD rate change in class D and -0.01 YUV-BD rate change in class C/E. It is suggested to adopt proposed two methods into the next VVC working draft.

Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, Gothenburg, SE, July 2019.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March 2019.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).