| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0504-v4 |
| --- | --- |

| Title: | CE5-related: Harmonization of Hadamard filter, SAO and ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Victor Stepin Sergey Ikonin Alexander Karabutov | Tel: Email: | +7-495-660-44-59 stepin.victor@huawei.com sergey.ikonin@huawei.com karabutov.alexander@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution reports results of the combination Hadamard transform domain filter HTDF (JVET-M0468) and SAO in-loop filter when ALF is turned on and off.

The simulations over VTM-6.0 show following results:

- -0.40%/-0.41/-0.20% of the luma BD-rate change with 107%/103%/102% of encoding time and 105%/101%/101% of decoding time for AI/RA/LDB configurations correspondingly when HTDF is on, SAO is on, ALF is on (sequential filter connection is used )

- 0.40%/-0.36/-0.09% of the luma BD-rate change with 107%/103%/102% of encoding time and 103%/101%/100% of decoding time for AI/RA/LDB configurations correspondingly when HTDF is on, SAO is off, ALF is on (sequential filter connection is used)

The simulations over VTM-6.0 with SAO off and ALF off show following results:

- -0.32%/-0.79%/-0.95%/-2.59% of the luma BD-rate change with 100%/100%/100%/100% of encoding time and 104%/102%/102%/102% of decoding time for AI/RA/LDB/LDP configurations correspondingly, when, SAO is on, HTDF is off, ALF is off (sequential filter connection is used)

- -0.93%/-1.35%/-1.34%/-2.64% of the luma BD-rate change with 108%/104%/102%/104% of encoding time and 107%/103%/102%/103% of decoding time for AI/RA/LDB/LDP configurations correspondingly, when SAO is off, HTDF is on, ALF is off (sequential filter connection is used)

- -1.02%/-1.64%/-1.59%/-3.48% of the luma BD-rate change with 108%/103%/102%/104% of encoding time and 109%/104%/104%/104% of decoding time for AI/RA/LDB configurations correspondingly, when SAO is on, HTDF is on, ALF is off (sequential filter connection is used)

- -0.99%/-1.50%/-1.48%/-3.27% of the luma BD-rate change with 109%/104%/102%/104% of encoding time and 111%/107%/106%/107% of decoding time for AI/RA/LDB configurations correspondingly ,when  SAO is on, HTDF is on, ALF is off (parallel SAO and HTDF filters connection is used, as in CE5-3.2)

Based on test result it is proposed to disable SAO when De-blocking, HTDF and ALF filter are used for in-loop filtering and enable SAO and HTDF filter when ALF filter is off.

# Introduction

This contribution is related to CE5-3.2 (JVET-P0078). CE5-3.2 proposes the combination of HTDF filter and SAO. As modification of CE5-3.2, this contribution proposes to disable SAO when ALF is enabled and enable HTDF and SAO sequentially connected when ALF is disabled. The figures below illustrate proposed loop filter configurations. Also results of parallel connection of HTDF and SAO are presented for case when ALF is disabled. In this test SAO&HTDF integration fixes described in JVET-P0412 are incorporated.

Figure 1 In-loop filters chain when ALF on

Figure 2 In-loop filters chain when ALF off (sequential connection HTDF and SAO)

Figure 3 In-loop filter chain when ALF off (parallel connection HTDF and SAO)

The following changing in syntax are proposed

Before changing:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |

sps_sao_enabled_flag equal to 1 specifies that the sample adaptive offset process is applied to the reconstructed picture after the deblocking filter process. sps_sao_enabled_flag equal to 0 specifies that the sample adaptive offset process is not applied to the reconstructed picture after the deblocking filter process.

After changing:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_alf_enabled_flag | u(1) |
| If(!sps_alf_enabled_flag) |  |
| sps_sao_enabled_flag | u(1) |

sps_sao_enabled_flag equal to 1 specifies that the sample adaptive offset process is applied to the reconstructed picture after the deblocking filter process. sps_sao_enabled_flag equal to 0 specifies that the sample adaptive offset process is not applied to the reconstructed picture after the deblocking filter process. When not present, the value of sps_sao_enabled_flag is inferred to be equal to 0.

# Test results

Simulations were carried out on top of VTM-6.0 software under common test conditions described in JVET-M1010.

## Test1: HTDF on, SAO on, ALF on (sequential processing)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | 0.48% | 0.44% | 107% | 105% |
| Class A2 | -0.42% | 0.34% | 0.28% | 107% | 104% |
| Class B | -0.32% | 0.45% | 0.35% | 106% | 105% |
| Class C | -0.52% | 0.14% | 0.26% | 107% | 105% |
| Class E | -0.43% | 0.26% | 0.23% | 107% | 106% |
| Overall | -0.40% | 0.34% | 0.31% | 107% | 105% |
| Class D | -0.47% | 0.39% | 0.24% | 107% | 104% |
| Class F (optional) | -0.47% | 0.14% | 0.02% | 104% | 107% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | 0.23% | 0.13% | 103% | 102% |
| Class A2 | -0.61% | 0.01% | -0.07% | 103% | 101% |
| Class B | -0.39% | 0.20% | 0.19% | 104% | 101% |
| Class C | -0.46% | 0.19% | 0.24% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.41% | 0.16% | 0.14% | 103% | 101% |
| Class D | -0.54% | -0.13% | 0.05% | 103% | 101% |
| Class F (optional) | -0.38% | -0.09% | 0.11% | 102% | 102% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | 0.63% | 0.45% | 103% | 102% |
| Class C | -0.38% | 0.30% | 0.48% | 102% | 101% |
| Class E | -0.17% | 0.92% | 1.02% | 100% | 100% |
| Overall | -0.20% | 0.59% | 0.60% | 102% | 101% |
| Class D | -0.57% | 0.74% | 0.66% | 103% | 100% |
| Class F (optional) | -0.06% | 0.45% | 0.64% | 101% | 101% |
|  | Low delay P |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | 0.56% | 0.73% | 104% | 101% |
| Class C | -0.38% | 0.27% | 0.30% | 103% | 102% |
| Class E | -0.26% | 0.28% | 0.74% | 101% | 100% |
| Overall | -0.28% | 0.39% | 0.59% | 103% | 101% |
| Class D | -0.54% | 0.33% | 0.41% | 103% | 101% |
| Class F (optional) | -0.24% | 0.13% | 0.32% | 102% | 101% |

## Test2: HTDF on, SAO off, ALF on (sequential processing)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | 0.70% | 0.55% | 107% | 103% |
| Class A2 | -0.40% | 0.66% | 0.46% | 107% | 102% |
| Class B | -0.30% | 0.53% | 0.55% | 107% | 103% |
| Class C | -0.56% | 0.48% | 0.71% | 107% | 104% |
| Class E | -0.42% | 0.24% | 0.24% | 107% | 104% |
| Overall | -0.40% | 0.52% | 0.52% | 107% | 103% |
| Class D | -0.48% | 0.51% | 0.38% | 107% | 104% |
| Class F (optional) | -0.16% | 0.55% | 0.59% | 104% | 104% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | 0.37% | 0.37% | 103% | 102% |
| Class A2 | -0.47% | 0.41% | 0.30% | 103% | 101% |
| Class B | -0.34% | 0.42% | 0.72% | 104% | 100% |
| Class C | -0.44% | 0.45% | 0.62% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | 0.42% | 0.54% | 103% | 101% |
| Class D | -0.64% | 0.00% | 0.04% | 103% | 100% |
| Class F (optional) | 0.09% | 0.39% | 0.61% | 103% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 1.33% | 2.27% | 103% | 100% |
| Class C | -0.34% | 0.96% | 1.25% | 102% | 100% |
| Class E | 0.00% | 0.73% | 1.56% | 101% | 100% |
| Overall | -0.09% | 1.06% | 1.75% | 102% | 100% |
| Class D | -0.72% | 1.40% | 0.40% | 103% | 100% |
| Class F (optional) | 0.60% | 0.70% | 1.36% | 101% | 101% |
|  | Low delay P |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 1.30% | 2.37% | 105% | 100% |
| Class C | -0.34% | 0.79% | 1.26% | 104% | 101% |
| Class E | -0.09% | 0.50% | 0.96% | 102% | 100% |
| Overall | -0.16% | 0.93% | 1.65% | 104% | 100% |
| Class D | -0.76% | 0.92% | 0.82% | 104% | 100% |
| Class F (optional) | 0.41% | 0.78% | 1.94% | 103% | 101% |

## Test3: SAO on, HTDF off, ALF off (CE5-3.3.2 NO ALF for reference)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0,SAO off, ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.60% | -1.84% | 100% | 104% |
| Class A2 | -0.48% | -0.56% | -0.44% | 100% | 105% |
| Class B | -0.29% | -0.21% | -0.42% | 100% | 104% |
| Class C | -0.30% | -0.58% | -0.92% | 100% | 103% |
| Class E | -0.38% | 0.05% | -0.01% | 100% | 103% |
| Overall | -0.32% | -0.37% | -0.70% | 100% | 104% |
| Class D | -0.21% | -0.29% | -0.45% | 101% | 102% |
| Class F (optional) | -0.97% | -0.78% | -1.09% | 100% | 103% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0,SAO off, ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | -0.46% | -0.54% | 100% | 102% |
| Class A2 | -1.39% | -1.28% | -0.91% | 100% | 102% |
| Class B | -1.00% | -0.89% | -0.69% | 100% | 102% |
| Class C | -0.40% | -0.58% | -0.74% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.79% | -0.80% | -0.72% | 100% | 102% |
| Class D | 0.05% | 0.10% | 0.08% | 100% | 101% |
| Class F (optional) | -1.27% | -0.89% | -1.32% | 100% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0,SAO off, ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.99% | -1.94% | -2.65% | 99% | 103% |
| Class C | -0.97% | -2.52% | -4.46% | 100% | 103% |
| Class E | -0.87% | -0.26% | -0.91% | 100% | 101% |
| Overall | -0.95% | -1.71% | -2.82% | 100% | 102% |
| Class D | -0.15% | -1.31% | -1.37% | 100% | 103% |
| Class F (optional) | -2.47% | -3.32% | -3.74% | 99% | 102% |
|  | Low delay P |  |  |  |  |
|  | Over VTM-6.0,SAO off, ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.12% | -3.44% | -3.55% | 99% | 103% |
| Class C | -2.08% | -3.36% | -4.84% | 100% | 103% |
| Class E | -2.36% | -2.03% | -2.23% | 100% | 102% |
| Overall | -2.59% | -3.06% | -3.65% | 100% | 102% |
| Class D | -0.89% | -1.16% | -1.44% | 100% | 103% |
| Class F (optional) | -2.62% | -3.38% | -3.83% | 99% | 103% |

## Test4: SAO off, HTDF on, ALF off (CE5-3.2.2 NO ALF for reference)

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0,SAO off, ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.82% | 0.56% | 0.59% | 108% | 107% |
| Class A2 | -0.97% | 0.42% | 0.34% | 108% | 107% |
| Class B | -0.76% | 0.54% | 0.44% | 108% | 107% |
| Class C | -1.01% | 0.27% | 0.36% | 108% | 106% |
| Class E | -1.21% | 0.36% | 0.31% | 107% | 107% |
| Overall | -0.93% | 0.43% | 0.41% | 108% | 107% |
| Class D | -0.83% | 0.51% | 0.35% | 107% | 106% |
| Class F | -1.10% | 0.21% | 0.14% | 105% | 107% |
|  |  |  | Random access |  |  |
|  | Over VTM-6.0,SAO off, ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.83% | 0.23% | 0.19% | 103% | 103% |
| Class A2 | -2.04% | -0.17% | -0.18% | 104% | 103% |
| Class B | -1.58% | 0.16% | 0.12% | 105% | 102% |
| Class C | -0.95% | 0.14% | 0.32% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -1.35% | 0.10% | 0.13% | 104% | 103% |
| Class D | -0.71% | 0.00% | 0.08% | 103% | 101% |
| Class F | -1.07% | 0.13% | 0.25% | 103% | 102% |
|  |  |  | Low delay B |  |  |
|  |  | Over VTM-6.0,SAO off, ALF off |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.34% | 1.08% | 0.91% | 103% | 103% |
| Class C | -1.45% | 0.56% | 0.60% | 103% | 102% |
| Class E | -1.21% | 1.47% | 1.00% | 101% | 101% |
| Overall | -1.34% | 1.00% | 0.83% | 102% | 102% |
| Class D | -1.00% | 1.13% | 0.63% | 104% | 101% |
| Class F | -1.42% | 0.52% | -0.14% | 102% | 101% |
|  |  |  | Low delay P |  |  |
|  |  | Over VTM-6.0,SAO off, ALF off |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.91% | 0.82% | 1.57% | 106% | 104% |
| Class C | -2.40% | 0.52% | 0.82% | 105% | 102% |
| Class E | -2.53% | 0.79% | 0.67% | 103% | 102% |
| Overall | -2.65% | 0.71% | 1.10% | 104% | 103% |
| Class D | -1.98% | 1.52% | 0.97% | 105% | 101% |
| Class F | -1.64% | -0.08% | 0.11% | 103% | 103% |

## Test5: SAO on, HTDF on, ALF off (sequential processing)

This test also incorporates integration fixes described in JVET-P0412.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0, SAO off, ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.84% | -0.01% | -1.36% | 108% | 110% |
| Class A2 | -1.15% | -0.25% | -0.20% | 107% | 110% |
| Class B | -0.85% | 0.25% | -0.03% | 108% | 109% |
| Class C | -1.05% | -0.41% | -0.66% | 108% | 107% |
| Class E | -1.32% | 0.26% | 0.28% | 108% | 110% |
| Overall | -1.02% | -0.02% | -0.37% | 108% | 109% |
| Class D | -0.83% | 0.05% | -0.23% | 108% | 106% |
| Class F (optional) | -1.71% | -0.58% | -0.89% | 106% | 110% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0, SAO off, ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.91% | -0.32% | -0.30% | 103% | 104% |
| Class A2 | -2.62% | -1.18% | -0.99% | 103% | 104% |
| Class B | -1.96% | -0.82% | -0.66% | 104% | 104% |
| Class C | -1.05% | -0.56% | -0.60% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -1.64% | -0.72% | -0.64% | 103% | 104% |
| Class D | -0.56% | 0.08% | 0.34% | 103% | 102% |
| Class F (optional) | -1.94% | -0.69% | -1.03% | 103% | 103% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0, SAO off, ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.72% | -1.16% | -2.27% | 102% | 103% |
| Class C | -1.71% | -2.02% | -3.91% | 103% | 104% |
| Class E | -1.20% | 0.44% | -0.01% | 100% | 102% |
| Overall | -1.59% | -1.05% | -2.25% | 102% | 103% |
| Class D | -0.95% | -0.36% | -0.61% | 104% | 104% |
| Class F (optional) |  | -3.02% |  | -3.14% |  | -3.87% |  | 101% |  | 103% |  |
|  | Low delay P |  |  |  |  |
|  | Over VTM-6.0, SAO off, ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.09% | -2.50% | -2.84% | 105% | 104% |
| Class C | -3.09% | -2.98% | -4.52% | 104% | 104% |
| Class E | -2.97% | -0.76% | -0.78% | 102% | 103% |
| Overall | -3.48% | -2.23% | -2.88% | 104% | 104% |
| Class D | -2.04% | -0.17% | -0.73% | 104% | 104% |
| Class F (optional) | -3.50% | -2.81% | -3.75% | 102% | 104% |

## Test6: SAO on, HTDF on, ALF off (parallel processing)

This test also incorporates integration fixes described in JVET-P0412.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 SAO off, ALF off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.86% | 0.17% | -0.67% | 109% | 114% |
| Class A2 | -1.12% | 0.21% | 0.14% | 108% | 113% |
| Class B | -0.80% | 0.45% | 0.33% | 109% | 112% |
| Class C | -1.04% | 0.00% | 0.08% | 109% | 108% |
| Class E | -1.21% | 0.28% | 0.37% | 109% | 111% |
| Overall | -0.99% | 0.23% | 0.08% | 109% | 111% |
| Class D | -0.82% | 0.40% | 0.27% | 108% | 107% |
| Class F | -1.30% | -0.04% | -0.21% | 106% | 111% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-6.0 SAO off, ALF off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.86% | 0.05% | 0.00% | 103% | 109% |
| Class A2 | -2.32% | -0.43% | -0.48% | 103% | 108% |
| Class B | -1.72% | -0.24% | -0.23% | 104% | 108% |
| Class C | -1.08% | -0.27% | -0.24% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | -1.50% | -0.22% | -0.23% | 104% | 107% |
| Class D | -0.62% | 0.18% | 0.25% | 103% | 103% |
| Class F | -1.56% | -0.28% | -0.53% | 103% | 106% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-6.0 SAO off, ALF off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.52% | 0.03% | -1.13% | 103% | 106% |
| Class C | -1.69% | -0.92% | -1.71% | 103% | 105% |
| Class E | -1.17% | 0.83% | 0.23% | 100% | 106% |
| Overall | -1.49% | -0.09% | -0.98% | 102% | 106% |
| Class D | -0.99% | 1.33% | 0.82% | 104% | 105% |
| Class F |  |  |  |  | -2.58% |  |  |  |  | -2.45% |  |  |  |  | -3.61% |  |  |  |  | 101% |  |  |  |  | 108% |  |  |  |  |
|  |  |  | Low delay P |  |  |
|  |  |  | Over VTM-6.0 SAO off, ALF off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  | -3.75% |  |  |  |  | -1.38% |  |  |  |  | -1.44% |  |  |  |  | 105% |  |  |  |  | 108% |  |  |  |  |
| Class C | -2.90% | -1.45% | -2.18% | 104% | 105% |
| Class E | -2.95% | -0.08% | -1.06% | 102% | 108% |
| Overall |  |  |  |  | -3.27% |  |  |  |  | -1.08% |  |  |  |  | -1.59% |  |  |  |  | 104% |  |  |  |  | 107% |  |  |  |  |
| Class D | -2.06% | 1.21% | 0.42% | 105% | 103% |
| Class F |  |  |  |  | -2.83% |  |  |  |  | -2.23% |  |  |  |  | -3.66% |  |  |  |  | 102% |  |  |  |  | 108% |  |  |  |  |

# Summary

The tables below summarise simulation results on loop filters configuration in presence of ALF. Effect of SAO based on JVET-P0045 are also given for comparison.

Table 1. Simulation summary over VTM6.0

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| SAO tool-off (CE5-3.3.2) |  | 0.01% | 0.14% | 0.17% | 100%* | 99%* |  | 0.08% | 0.28% | 0.35% | 100%* | 98%* |
| HTDF (CE5-3.2.2, no SAO) |  | -0.42% | 0.53% | 0.53% | 107% | 103% |  | -0.38% | 0.39% | 0.51% | 104% | 100% |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| SAO tool-off (CE5-3.3.2) |  | 0.07% | 0.58% | 1.24% | 101%* | 101%* |  | 0.14% | 0.41% | 1.14% | 100%* | 100% |
| HTDF (CE5-3.2.2, no SAO) |  | -0.17% | 1.05% | 1.54% | 102% | 99% |  | -0.19% | 1.00% | 1.77% | 104% | 99% |

The tables below summarise simulation results on loop filters configuration in absence of ALF. Effect of SAO based on JVET-P0045 are also given for comparison.

Table 2. Simulation summary over VTM6.0.noSAO.noALF

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| SAO (CE5-3.3.2) |  | -0.32% | -0.37% | -0.70% | 100% | 104% |  | -0.79% | -0.80% | -0.72% | 100% | 102% |
| HTDF (CE5-3.2.2) |  | -0.93% | 0.43% | 0.41% | 108% | 107% |  | -1.35% | 0.10% | 0.13% | 104% | 103% |
| SAO + HTDF parallel |  | -0.99% | 0.23% | 0.08% | 109% | 111% |  | -1.50% | -0.22% | -0.23% | 104% | 107% |
| SAO + HTDF sequential |  | -1.02% | -0.02% | -0.37% | 108% | 109% |  | -1.64% | -0.72% | -0.64% | 103% | 104% |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| SAO (CE5-3.3.2) |  | -0.95% | -1.71% | -2.82% | 100% | 102% |  | -2.59% | -3.06% | -3.65% | 100% | 102% |
| HTDF (CE5-3.2.2) |  | -1.34% | 1.00% | 0.83% | 102% | 102% |  | -2.65% | 0.71% | 1.10% | 104% | 103% |
| SAO + HTDF parallel |  | -1.49% | -0.11% | -1.01% | 102% | 106% |  | -3.27% | -1.08% | -1.59% | 104% | 107% |
| SAO + HTDF sequential |  | -1.59% | -1.05% | -2.25% | 102% | 104% |  | -3.48% | -2.23% | -2.88% | 104% | 104% |

# Conclusion

From CE5 result it can be concluded that coding gain of SAO and ALF filter is significantly overlapped. Therefore, this contribution proposes to disable SAO from in-loop filtering processing chain when ALF is enabled (as in CE5-3.2.2) with keeping same number of in-loop filtering stages as in anchor. When ALF is disabled it is proposed to replace parallel connection HTDF and SAO by sequential connection of these filters. Both configurations allows to keep same numbers of in-loop filtering stages as in anchor.

It is suggested to adopt proposed loop filter configuration into VVC.

We would like to thank Ericsson for cross-checking.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).