| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0505 |
| --- | --- |

| Title: | Non-CE5: On non-linear ALF clipping values |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yixin Du, Xin Zhao, Xiang Li and Shan Liu | Email: | {yixindu, xinzzhao, xlxiangli, shanl}@tencent.com |  |
| Source: | Tencent |  |  |  |

_____________________________

# Abstract

In VTM-6.0, the clipping values used in non-linear ALF are derived for luma and chroma components with separate equations. In this contribution, a method is proposed to derive ALF clipping values which are proportional to 2bitDepth. It is reported that, when InternalBitDepth is set equal to 8, average -0.01%, -0.06% and -0.10% luma BD-rates are obtained under AI, RA and LDB configurations, respectively, and average -0.19%, -0.29% and -0.33% luma BD-rates are obtained for TGM sequences under AI, RA and LDB configurations, respectively.

# Introduction

The clipping values of non-linear ALF are dependent on the internal bit depth in VTM-6.0. More precisely, the table of clipping values are obtained for luma and chroma components, respectively, by the following formulas:

AlfClipL=round2BN-n+1N  for n∈1..N, 				(1-1)

AlfClipC=round2B-8+8(N-n)N-1 for n∈1..N			(1-2)

with B equal to the internal bit depth and N equal to 4 which is the number of allowed clipping values. Table 1 provides clipping values derived by equation (1-1).

Table 1 – Specification AlfClip depending on bitDepth and clipIdx

| bitDepth | clipIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 8 | 255 | 64 | 16 | 4 |
| 9 | 511 | 108 | 23 | 5 |
| 10 | 1023 | 181 | 32 | 6 |
| 11 | 2047 | 304 | 45 | 7 |
| 12 | 4095 | 512 | 64 | 8 |
| 13 | 8191 | 861 | 91 | 10 |
| 14 | 16383 | 1448 | 128 | 11 |
| 15 | 32767 | 2435 | 181 | 13 |
| 16 | 65535 | 4096 | 256 | 16 |

# Proposed Solution

In order to derive the clipping values which are proportional to 2bitDepth, the following one equation is proposed to derive ALF clipping values for both luma and chroma:

AlfClip=round2B-*n  for n∈0..N-1, 			(2-1)

where AlfClip is clipping value, B refers to internal bit depth or the bit depth of input samples for ALF,  are pre-defined constant values. In this contribution,  is set as 2.35 and 2.70 for luma and chroma, respectively.

# Experimental Results

The proposed method was implemented on VTM-6.0 reference software [1] and simulations are performed using Common Test Conditions [2]. The results of setting InternalBitDepth equal to 8 and 10 are provided in the following tables separately. The anchor is VTM-6.0 and the test is the proposed method.

Table 1: Results under CTC with InternalBitDepth equal to 8.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.02% | 100% |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% |  |
| Class B | 0.00% | -0.01% | -0.01% | 100% |  |
| Class C | 0.00% | -0.01% | -0.01% | 100% |  |
| Class E | -0.02% | -0.01% | 0.00% | 100% |  |
| Overall | -0.01% | -0.01% | -0.01% | 100% |  |
| Class D | 0.00% | -0.01% | -0.01% | 100% |  |
| Class F | -0.02% | 0.01% | 0.00% | 100% |  |
| Class TGM | -0.19% | -0.01% | 0.00% | 100% |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.10% | 0.08% | 100% |  |
| Class A2 | -0.12% | 0.08% | -0.05% | 100% |  |
| Class B | -0.05% | -0.07% | 0.02% | 101% |  |
| Class C | -0.06% | 0.02% | -0.05% | 101% |  |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.02% | 0.00% | 101% |  |
| Class D | 0.00% | -0.16% | -0.03% | 101% |  |
| Class F | -0.14% | 0.05% | 0.03% | 101% |  |
| Class TGM | -0.29% | 0.02% | 0.03% | 101% |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | 0.02% | -0.08% | 101% |  |
| Class C | -0.06% | -0.23% | -0.31% | 100% |  |
| Class E | -0.17% | -0.51% | -0.37% | 101% |  |
| Overall | -0.10% | -0.20% | -0.23% | 101% |  |
| Class D | -0.01% | 0.02% | 0.00% | 100% |  |
| Class F | -0.14% | -0.12% | 0.27% | 100% |  |
| Class TGM | -0.33% | 0.17% | 0.11% | 101% |  |

Table 2: Results under CTC with InternalBitDepth equal to 10.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.02% | 100% |  |
| Class A2 | 0.00% | 0.00% | -0.01% | 100% |  |
| Class B | -0.01% | 0.00% | -0.01% | 100% |  |
| Class C | 0.00% | -0.01% | -0.01% | 100% |  |
| Class E | 0.00% | 0.00% | 0.00% | 100% |  |
| Overall | 0.00% | 0.00% | -0.01% | 100% |  |
| Class D | 0.00% | 0.01% | 0.01% | 101% |  |
| Class F | -0.03% | 0.00% | 0.00% | 100% |  |
| Class TGM | -0.01% | 0.00% | 0.00% | 100% |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | -0.09% | 100% |  |
| Class A2 | 0.02% | 0.05% | 0.05% | 100% |  |
| Class B | -0.02% | 0.09% | 0.06% | 101% |  |
| Class C | 0.00% | 0.04% | -0.02% | 101% |  |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.05% | 0.01% | 101% |  |
| Class D | -0.05% | -0.02% | 0.02% | 100% |  |
| Class F | -0.04% | 0.03% | 0.05% | 100% |  |
| Class TGM | 0.00% | -0.03% | -0.04% | 101% |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.03% | -0.14% | 100% |  |
| Class C | -0.02% | 0.15% | 0.08% | 100% |  |
| Class E | -0.05% | 0.71% | 1.06% | 100% |  |
| Overall | -0.03% | 0.22% | 0.23% | 100% |  |
| Class D | -0.01% | -0.10% | 0.18% | 100% |  |
| Class F | -0.05% | -0.10% | 0.33% | 100% |  |
| Class TGM | -0.03% | 0.01% | 0.03% | 101% |  |

# Proposed Spec Changes

The following proposed text modifications are based on VVC Draft 6 [1]. The added texts are highlighted in green, and deleted texts are marked by red strikethrough.

In 7.4.3.14 “Adaptive loop filter data semantics”, following changes are proposed.

alf_luma_clip_idx[ sfIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the signalled luma filter indicated by sfIdx. It is a requirement of bitstream conformance that the values of alf_luma_clip_idx[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1 and j = 0..11 shall be in the range of 0 to 3, inclusive.

The luma filter clipping values AlfClipL[ adaptation_parameter_set_id ] with elements AlfClipL[ adaptation_parameter_set_id ][ filtIdx ][ j ], with filtIdx = 0..NumAlfFilters − 1 and j = 0..11 are derived as specified in Table 7-4 depending on bitDepth set equal to BitDepthY and clipIdx set equal to alf_luma_clip_idx[ alf_luma_coeff_delta_idx[ filtIdx ] ][ j ].

alf_chroma_num_alt_filters_minus1 plus 1 specifies the number of alternative filters for chroma components.

alf_chroma_clip_flag[ altIdx ] equal to 0 specifies that linear adaptive loop filtering is applied on chroma components when using the chroma filter with index altIdx; alf_chroma_clip_flag[ altIdx ] equal to 1 specifies that non-linear adaptive loop filtering is applied on chroma components when using the chroma filter with index altIdx. When not present, alf_chroma_clip_flag[ altIdx ] is inferred to be equal to 0.

alf_chroma_coeff_abs[ altIdx ][ j ] specifies the absolute value of the j-th chroma filter coefficient for the alternative chroma filter with index altIdx. When alf_chroma_coeff_abs[ altIdx ][ j ] is not present, it is inferred to be equal 0. It is a requirement of bitstream conformance that the values of alf_chroma_coeff_abs[ altIdx ][ j ] shall be in the range of 0 to 27 − 1, inclusive.

The order k of the exp-Golomb binarization uek(v) is set equal to 3.

alf_chroma_coeff_sign[ altIdx ][ j ] specifies the sign of the j-th chroma filter coefficient for the alternative chroma filter with index altIdx as follows:

- If alf_chroma_coeff_sign[ altIdx ][ j ] is equal to 0, the corresponding chroma filter coefficient has a positive value.

- Otherwise (alf_chroma_coeff_sign[ altIdx ][ j ] is equal to 1), the corresponding chroma filter coefficient has a negative value.

When alf_chroma_coeff_sign[ altIdx ][ j ] is not present, it is inferred to be equal to 0.

The chroma filter coefficients AlfCoeffC[ adaptation_parameter_set_id ][ altIdx ] with elements AlfCoeffC[ adaptation_parameter_set_id ][ altIdx ][ j ], with altIdx = 0..alf_chroma_num_alt_filters_minus1, j = 0..5 are derived as follows:

AlfCoeffC[ adaptation_parameter_set_id ][ altIdx ][ j ] = alf_chroma_coeff_abs[ altIdx ][ j ] * 	(7-51)
	( 1 − 2 * alf_chroma_coeff_sign[ altIdx ][ j ])

It is a requirement of bitstream conformance that the values of AlfCoeffC[ adaptation_parameter_set_id ][ altIdx ][ j ] with altIdx = 0..alf_chroma_num_alt_filters_minus1, j = 0..5 shall be in the range of −27 − 1 to 27 − 1, inclusive.

alf_chroma_clip_idx[ altIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the alternative chroma filter with index altIdx. It is a requirement of bitstream conformance that the values of alf_chroma_clip_idx[ altIdx ][ j ] with altIdx = 0..alf_chroma_num_alt_filters_minus1, j = 0..5 shall be in the range of 0 to 3, inclusive.

The chroma filter clipping values AlfClipC[ adaptation_parameter_set_id ][ altIdx ] with elements AlfClipC[ adaptation_parameter_set_id ][ altIdx ][ j ], with altIdx = 0..alf_chroma_num_alt_filters_minus1, j = 0..5 are derived as specified in Table 7-4Table 7-5 depending on bitDepth set equal to BitDepthC and clipIdx set equal to alf_chroma_clip_idx[ altIdx ][ j ].

Table 7-4 – Specification AlfClipL depending on bitDepth and clipIdx

| bitDepth | clipIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 8 | 255 | 6452 | 1611 | 42 |
| 9 | 511 | 108104 | 2321 | 54 |
| 10 | 1023 | 181208 | 3242 | 69 |
| 11 | 2047 | 304416 | 4584 | 717 |
| 12 | 4095 | 512832 | 64169 | 834 |
| 13 | 8191 | 8611663 | 91338 | 1069 |
| 14 | 16383 | 14483327 | 128676 | 11137 |
| 15 | 32767 | 24356654 | 1811351 | 13274 |
| 16 | 65535 | 409613308 | 2562702 | 16549 |

Table 7-5 – Specification AlfClipC depending on bitDepth and clipIdx

| bitDepth | clipIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 8 | 255 | 42 | 7 | 1 |
| 9 | 511 | 84 | 14 | 2 |
| 10 | 1023 | 169 | 28 | 5 |
| 11 | 2047 | 338 | 56 | 9 |
| 12 | 4095 | 676 | 111 | 18 |
| 13 | 8191 | 1351 | 223 | 37 |
| 14 | 16383 | 2702 | 446 | 74 |
| 15 | 32767 | 5405 | 891 | 147 |
| 16 | 655335 | 10809 | 1783 | 294 |

# Conclusion

This contribution proposed an alternative method to derive non-linear ALF clipping values. It derives the clipping values which are proportional to 2bitDepth. It is reported that when InternalBitDepth is set to 8, average -0.01%, -0.06% and -0.10% luma BD-rates are obtained under AI, RA and LDB configurations, respectively, and average -0.19%, -0.29% and -0.33% are obtained for TGM sequences under AI, RA and LDB configurations, respectively.

# References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-6.0.

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).