| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0506-v2 |
| --- | --- |

| Title: | AGH 17: On max QT/BT/TT size signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yixin Du, Xiang Li, Xin Zhao, Byeongdoo Choi, Stephan Wenger, and Shan Liu | Email: | {yixindu, xlxiangli, xinzzhao, bdchoi, swenger, shanl}@tencent.com |  |
| Source: | Tencent |  |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to signal max QT/BT/TT sizes relative to CTU size instead of min QT size in SPS and slice header to be more bit-efficient. It is reported that -0.01% for RA and -0.04% for LDB coding gain can be obtained using the proposed method. It is reported that -2.46% for AI, -1.10% for RA, and -2.07% for LDB bits saving can be obtained in SPS, -5.14% for RA and -5.65% for LDB bits saving can be obtained in slice header.

# Introduction

In VVC Draft 6 [1], there are some syntax elements which specify the sizes of QT/BT/TT, as shown in highlighted area in Table 1 and 2. Each of the highlighted syntaxes specifies the default difference between the base 2 logarithm of two numbers.

Table 1. JVET-O2001-vE “Sequence parameter set RBSP syntax”.

| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| --- | --- |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_slices != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if ( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |

Table 2. JVET-O2001-vE “General slice header syntax”.

| if( slice_max_mtt_hierarchy_depth_luma != 0 ) |  |
| --- | --- |
| slice_log2_diff_max_bt_min_qt_luma | ue(v) |
| slice_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_chroma | ue(v) |
| slice_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_chroma | ue(v) |
| slice_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |

# Proposed Solution and Experimental Results

When signaling max QT/BT/TT size in SPS and slice header, it is proposed to change the base from min QT to CTU. For example, sps_log2_diff_max_bt_min_qt_intra_slice_luma can be changed to sps_log2_diff_ctu_max_bt_intra_slice_luma. Spec text changes are provided in Section 3.

Simulations are performed using VTM-6.0. The results are shown in the following tables following Common Test Condition [2]. The anchor is VTM-6.0, and the test is proposed method.

Table 3. Experimental results using proposed method.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 104% |
| Class E | -0.01% | -0.01% | -0.01% | 99% | 104% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class D | -0.01% | -0.01% | -0.01% | 100% | 106% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.01% | 99% | 101% |
| Class A2 | -0.01% | -0.01% | -0.01% | 99% | 101% |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 102% |
| Class C | -0.02% | -0.02% | -0.02% | 100% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 103% |
| Class D | -0.06% | -0.06% | -0.06% | 100% | 104% |
| Class F | -0.01% | -0.01% | -0.01% | 100% | 103% |
| Class TGM | -0.02% | -0.01% | -0.01% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.01% | 99% | 102% |
| Class C | -0.02% | -0.02% | -0.02% | 100% | 101% |
| Class E | -0.12% | -0.11% | -0.11% | 99% | 100% |
| Overall | -0.04% | -0.04% | -0.04% | 100% | 101% |
| Class D | -0.05% | -0.05% | -0.05% | 100% | 103% |
| Class F | -0.08% | -0.07% | -0.07% | 100% | 102% |
| Class TGM | -0.02% | -0.02% | -0.02% | 100% | 104% |

Table 4 shows the average bits saving in SPS and slice header. It is reported that -2.46% for AI, -1.10% for RA, and -2.07% for LDB bits saving can be obtained in SPS, -5.14% for RA and -5.65% for LDB bits saving can be obtained in slice header.

Table 4. Average bits saving in SPS and slice header using the proposed method.

|  | AI | RA | LDB |
| --- | --- | --- | --- |
| SPS | -2.46% | -1.10% | -2.07% |
| Slice header | 0.00% | -5.14% | -5.65% |

# Spec Text Change

The following proposed text modifications are based on VVC Draft 6 [1]. The added texts are highlighted in green, and deleted texts are marked by red strikethrough.

In syntax table 7.3.2.3 “Sequence parameter set RBSP syntax”, change the following content.

| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| --- | --- |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_ctu_max_bt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_ctu_max_tt_intra_slice_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_slices != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_ctu_max_bt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_ctu_max_tt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_log2_diff_ctu_min_qt_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if ( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_ctu_max_bt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_ctu_max_tt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |

In syntax table 7.3.6.1 “General slice header syntax”, change the following content.

| if( slice_max_mtt_hierarchy_depth_luma != 0 ) |  |
| --- | --- |
| slice_log2_diff_max_bt_min_qt_luma | ue(v) |
| slice_log2_diff_ctu_max_bt_luma | ue(v) |
| slice_log2_diff_max_tt_min_qt_luma | ue(v) |
| slice_log2_diff_ctu_max_tt_luma | ue(v) |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_chroma | ue(v) |
| slice_log2_diff_ctu_min_qt_chroma | ue(v) |
| slice_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_chroma | ue(v) |
| slice_log2_diff_ctu_max_bt_chroma | ue(v) |
| slice_log2_diff_max_tt_min_qt_chroma | ue(v) |
| slice_log2_diff_ctu_max_tt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |

In 7.4.3.3 “Sequence parameter set RBSP semantics”, changes are provided as follows.

sps_log2_diff_ctu_max_bt_min_qt_intra_slice_luma specifies the default difference between the base 2 logarithm of the ctu size and the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_ flag is equal to 1, the default difference can be overridden by slice_log2_diff_ctu_max_bt_min_qt_luma present in the slice header of the slices referring to the SPS. The value of sps_log2_diff_ctu_max_bt_min_qt_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraY, inclusive. When sps_log2_diff_ctu_max_bt_min_qt_intra_slice_luma is not present, the value of sps_log2_diff_ctu_max_bt_min_qt_intra_slice_luma is inferred to be equal to 0.

sps_log2_diff_ctu_max_tt_min_qt_intra_slice_luma specifies the default difference between the base 2 logarithm of the ctu size and the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_ flag is equal to 1, the default difference can be overridden by slice_log2_diff_ctu_max_tt_min_qt_luma present in the slice header of the slices referring to the SPS. The value of sps_log2_diff_ctu_max_tt_min_qt_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraY, inclusive. When sps_log2_diff_ctu_max_tt_min_qt_intra_slice_luma is not present, the value of sps_log2_diff_ctu_max_tt_min_qt_intra_slice_luma is inferred to be equal to 0.

sps_log2_diff_ctu_max_bt_min_qt_inter_slice specifies the default difference between the base 2 logarithm of the ctu size and the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 0 (B) or 1 (P) referring to the SPS. When partition_constraints_override_ flag is equal to 1, the default difference can be overridden by slice_log2_diff_ctu_max_bt_min_qt_luma present in the slice header of the slices referring to the SPS. The value of sps_log2_diff_ctu_max_bt_min_qt_inter_slice shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeInterY, inclusive. When sps_log2_diff_ctu_max_bt_min_qt_inter_slice is not present, the value of sps_log2_diff_ctu_max_bt_min_qt_inter_slice is inferred to be equal to 0.

sps_log2_diff_ctu_max_tt_min_qt_inter_slice specifies the default difference between the base 2 logarithm of the ctu size and the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 0 (B) or 1 (P) referring to the SPS. When partition_constraints_override_ flag is equal to 1, the default difference can be overridden by slice_log2_diff_ctu_max_tt_min_qt_luma present in the slice header of the slices referring to the SPS. The value of sps_log2_diff_ctu_max_tt_min_qt_inter_slice shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeInterY, inclusive. When sps_log2_diff_ctu_max_tt_min_qt_inter_slice is not present, the value of sps_log2_diff_ctu_max_tt_min_qt_inter_slice is inferred to be equal to 0.

sps_log2_diff_ctu_max_bt_min_qt_intra_slice_chroma specifies the default difference between the base 2 logarithm of the ctu size and the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_flag is equal to 1, the default difference can be overridden by slice_log2_diff_ctu_max_bt_min_qt_chroma present in the slice header of the slices referring to the SPS. The value of sps_log2_diff_ctu_max_bt_min_qt_intra_slice_chroma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraC, inclusive. When sps_log2_diff_ctu_max_bt_min_qt_intra_slice_chroma is not present, the value of sps_log2_diff_ctu_max_bt_min_qt_intra_slice_chroma is inferred to be equal to 0.

sps_log2_diff_ctu_max_tt_min_qt_intra_slice_chroma specifies the default difference between the base 2 logarithm of the ctu size and the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_flag is equal to 1, the default difference can be overridden by slice_log2_diff_ctu_max_tt_min_qt_chroma present in the slice header of the slices referring to the SPS. The value of sps_log2_diff_ctu_max_tt_min_qt_intra_slice_chroma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeIntraC, inclusive. When sps_log2_diff_ctu_max_tt_min_qt_intra_slice_chroma is not present, the value of sps_log2_diff_ctu_max_tt_min_qt_intra_slice_chroma is inferred to be equal to 0.

In 7.4.7.1 “General slice header semantics”, change the following content.

slice_log2_diff_ctu_max_bt_min_qt_luma specifies the difference between the base 2 logarithm of the ctu size and the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in the current slice. The value of slice_log2_diff_ctu_max_bt_min_qt_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeY, inclusive. When not present, the value of slice_log2_diff_ctu_max_bt_min_qt_luma is inferred as follows:

- If slice_type equal to 2 (I), the value of slice_log2_diff_max_bt_min_qt_luma is inferred to be equal to sps_log2_diff_ctu_max_bt_min_qt_intra_slice_luma

- Otherwise (slice_type equal to 0 (B) or 1 (P)), the value of slice_log2_diff_max_bt_min_qt_luma is inferred to be equal to sps_log2_diff_ctu_max_bt_min_qt_inter_slice.

slice_log2_diff_ctu_max_tt_min_qt_luma specifies the difference between the base 2 logarithm of the ctu size and the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in in the current slice. The value of slice_log2_diff_ctu_max_tt_min_qt_luma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeY, inclusive. When not present, the value of slice_log2_diff_ctu_max_tt_min_qt_luma is inferred as follows:

- If slice_type equal to 2 (I), the value of slice_log2_diff_max_tt_min_qt_luma is inferred to be equal to sps_log2_diff_ctu_max_tt_min_qt_intra_slice_luma

Otherwise (slice_type equal to 0 (B) or 1 (P)), the value of slice_log2_diff_ctu_max_tt_min_qt_luma is inferred to be equal to sps_log2_diff_ctu_max_tt_min_qt_inter_slice.

slice_log2_diff_ctu_max_bt_min_qt_chroma specifies the difference between the base 2 logarithm of the ctu size and the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in the current slice. The value of slice_log2_diff_ctu_max_bt_min_qt_chroma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeC, inclusive. When not present, the value of slice_log2_diff_ctu_max_bt_min_qt_chroma is inferred to be equal to sps_log2_diff_ctu_max_bt_min_qt_intra_slice_chroma

slice_log2_diff_ctu_max_tt_min_qt_chroma specifies the difference between the base 2 logarithm of the ctu size and the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in the current slice. The value of slice_log2_diff_ctu_max_tt_min_qt_chroma shall be in the range of 0 to CtbLog2SizeY − MinQtLog2SizeC, inclusive. When not present, the value of slice_log2_diff_ctu_max_tt_min_qt_chroma is inferred to be equal to sps_log2_diff_ctu_max_tt_min_qt_intra_slice_chroma

The variables MinQtLog2SizeY, MinQtLog2SizeC, MinQtSizeY, MinQtSizeC, MaxBtSizeY, MaxBtSizeC, MinBtSizeY, MaxTtSizeY, MaxTtSizeC, MinTtSizeY, MaxMttDepthY and MaxMttDepthC are derived as follows:

MinQtLog2SizeY  =  MinCbLog2SizeY + slice_log2_diff_min_qt_min_cb_luma	(7-99)

MinQtLog2SizeC  =  MinCbLog2SizeY + slice_log2_diff_min_qt_min_cb_chroma	(7-100)

MinQtSizeY  =  1  <<  MinQtLog2SizeY		(7-101)

MinQtSizeC  =  1  <<  MinQtLog2SizeC		(7-102)

MaxBtSizeY  =  1  <<  ( MinQtLog2SizeY +CtbSizeY - slice_log2_diff_ctu_max_bt_min_qt_luma )	(7-103)

# Conclusion

It is proposed to change the base of syntaxes regarding max QT/BT/TT size from min QT to CTU. It is reported that -0.01% for RA and -0.04% for LDB coding gain can be obtained using the proposed method on top of VTM-6.0 using Common Test Condition. It is reported that -2.46% for AI, -1.10% for RA, and -2.07% for LDB bits saving can be obtained in SPS, -5.14% for RA and -5.65% for LDB bits saving can be obtained in slice header.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, ISO/IEC JTC1/SC29/WG11 JVET-O2001-vE, July 2019.

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).