| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0511_r1 |
| --- | --- |

| Title: | Non-CE3: ISP extension to avoid small Chroma intra blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xiang Li, Xin Zhao, Shan Liu | Tel: Email: | {leolzhao, xlxiangli, xinzzhao, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In VVC Draft 6, Small Chroma Intra Prediction Unit (SCIPU) has been adopted, which avoids 2x2, 2x4, and 4x2 small chroma intra blocks in single tree. However, it creates a local dual tree in single tree. It is reported that it leaded a lot of software and spec text changes. In this contribution, an alternative method to avoid small chroma intra blocks is proposed, which needs less software and spec text changes. It includes two steps. In the first step, 4x4, 4x8, and 8x4 intra CU is disallowed in single tree so that 2x2, 2x4, and 4x2 chroma intra CBs is avoided. In the second step, ISP is extended to support binary tree split (BT) for 8x8, 4x16, and 16x4 CUs, and one flag is added to indicate whether current luma CU is split into 2 partitions or 4 partitions. Same as ISP in VTM-6, the extended ISP is only applied to luma component. It is reported that the proposed method achieves 0.01% and 0.09% luma BD-rate changes for AI and RA configurations, respectively.

Introduction

In HEVC, the smallest intra CU is with 8x8 luma samples. The luma component of the smallest intra CU can be further split into four 4x4 luma intra prediction units (PUs), but the chroma components of the smallest intra CU cannot be further split. Therefore, the worst-case hardware processing throughput occurs when 4x4 chroma intra blocks or 4x4 luma intra blocks are processed.

In VTM5.0, in single coding tree, since chroma partitions always follow luma and the smallest intra CU is 4x4 luma samples, the smallest chroma intra CB is 2x2. The worst-case hardware processing throughput for VVC decoding is only 1/4 of that for HEVC decoding.

In VTM6.0, Small Chroma Intra Prediction Unit (SCIPU) is adopted [1]. SCIPU aims to disallow chroma intra CBs smaller than 16 chroma samples by creating a local dual tree in single tree. Before entering local dual tree, chroma partitions has the same split with the associated luma partitions. Every CU has the flexibility to decide whether to be intra coded, inter coded or IBC coded. After entering local dual tree, all the CUs should be inter coded or non-inter coded (intra or IBC). When current CU is inter coded, chroma component also follows the same split with luma CU. Otherwise, when current CU is non-inter coded, luma CU can be further split whereas the associated chroma intra CU cannot be further split. The condition for determining whether current CU is the parent node of local dual tree or not is listed below:

- CU size is 64 and current CU is further QT split

- CU size is 64 and current CU is further TT split

- CU size is 32 and current CU is further BT split

- CU size is 64 and current CU is further BT split

- CU size is 128 and current CU is further TT split

When one of the above conditions is true, current CU is marked as the parent node of local dual tree. In addition, when condition a), b) or c) is satisfied, the CUs within this area is constrained to be all intra coded or all IBC coded. When condition d) or e) is satisfied, one more flag needs to be signaled to indicate whether all the CUs within this area is inter-coded or non-inter coded.

Proposed method

In this contribution, an alternative method to avoid 2x2, 2x4, and 4x4 single tree small chroma blocks is proposed. It includes two steps. In the first step, 4x4, 4x8, and 8x4 intra CU is disallowed in single tree so that 2x2, 2x4, and 4x2 chroma intra CBs is avoided. In the second step, ISP is extended to support binary tree split (BT) for 8x8, 4x16, and 16x4 CUs, and one flag is added to indicate whether current luma CU is split into 2 partitions or 4 partitions.

Same as ISP in VTM-6, the extended ISP is only applied to luma component. One difference between ISP in VTM-6 and extended ISP is that, each sub-partition can have its own luma intra prediction mode when the extended ISP is selected.

Experimental results

The proposed method in this contribution is implemented on top of VTM6.0 software. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-N1010 [2]. One test is conducted to evaluate the performance of the proposed method.

Table 3-1: Results of the proposed method over VTM-6.0.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.03% | 97% | 101% |
| Class A2 | 0.01% | -0.01% | 0.01% | 97% | 101% |
| Class B | 0.01% | 0.01% | -0.03% | 98% | 102% |
| Class C | 0.00% | -0.12% | 0.04% | 97% | 99% |
| Class E | 0.00% | 0.00% | 0.01% | 97% | 101% |
| Overall | 0.01% | -0.03% | 0.01% | 97% | 101% |
| Class D | -0.02% | 0.02% | -0.03% | 97% | 96% |
| Class F | 0.27% | 0.04% | 0.09% | 100% | 102% |

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.01% | 101% | 101% |
| Class A2 | 0.04% | -0.09% | 0.00% | 101% | 101% |
| Class B | 0.07% | 0.05% | -0.04% | 102% | 101% |
| Class C | 0.23% | 0.09% | 0.03% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.01% | 0.00% | 102% | 101% |
| Class D | 0.16% | -0.18% | -0.01% | 102% | 101% |
| Class F | 0.21% | 0.20% | 0.27% | 103% | 100% |

Conclusion

This contribution proposes an alternative method to avoid small chroma intra blocks in single tree, by extending ISP to support binary tree split (BT) for 8x8, 4x16, and 16x4 CUs. It is reported that the proposed method achieves 0.01% and 0.09% BD-rate changes for AI and RA configurations, respectively.

References

- Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei, Yin Zhao, Haitao Yang, “CE3-2.1.1 and CE3-2.1.2: Removing 2x2, 2x4, and 4x2 chroma CBs,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-O0050, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).