| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0516 |
| --- | --- |

| Title: | Non-CE8: On palette mode signaling binarization |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Hong-Jheng, Jhu, Xianglin Wang (Kwai) Yung-Hsuan Chao, Chao-Hsiung Hung, Wei-Jung Chien, Marta Karczewicz (Qualcomm) | Tel: Email: | +1-858-7052600 yiwenchen@kwai.com |  |
| Source: | Kwai Inc., Qualcomm Inc. |  |  |  |

_____________________________

# Abstract

In current VVC, the PLT mode flag (pred_mode_plt_flag) is signaled when the pred_mode_flag is signaled as 1. However, there is one exception when only intra, inter and PLT modes are allowed (i.e. IBC mode is disabled). The PLT mode flag (pred_mode_plt_flag) is signaled when the pred_mode_flag is signaled as 0. Such an exception not only breaks the physical meaning of the flag of pred_mode_flag, it also complicates the signaling conditions of pred_mode_plt_flag. In this contribution, it is proposed to signal pred_mode_plt_flag when pred_mode_flag is signaled as 1 under all conditions.

With the proposed fix, the signaling conditions for pred_mode_plt_flag is simplified and the results on YUV4:4:4 sequences show minor coding performance changes under CE8 CTC with IBC disabled compared to VTM6.0. The proposed modification introduces 0.00% AI, 0.01% RA, -0.08% LB BD-rate change under dual-tree off condition and 0.00% AI, -0.03% RA, -0.04% LB BD-rate change under dual-tree on condition.

# Introduction

In current VVC, the IBC mode flag (pred_mode_ibc_flag) is signaled when the pred_mode_flag is signaled as 0 which implies the current CU is inter mode alike. The PLT mode flag (pred_mode_plt_flag) is signaled when the pred_mode_flag is signaled as 1 which implies the current CU is intra mode alike. However, there is one exception when only intra, inter and PLT modes are allowed (i.e. IBC mode is not allowed). As shown in Table 1, The PLT mode flag (pred_mode_plt_flag) is signaled when the pred_mode_flag is signaled as 0. Such an exception not only breaks the physical meaning of the flag of pred_mode_flag, it also complicates the signaling conditions of pred_mode_plt_flag because different signaling conditions are used depending on whether IBC mode is allowed or not.

Table 1. Signaling of the prediction modes in current VVC when only intra, inter and PLT modes are allowed

| Pred_Modes | pred_mode_flag | pred_mode_ibc_flag | pred_mode_plt_flag |
| --- | --- | --- | --- |
| INTER | 0 | - | 0 |
| PLT | 0 | - | 1 |
| INTRA | 1 | - | - |

# Proposal

In this proposal, it is proposed to signal pred_mode_plt_flag when pred_mode_flag is signaled as 1 under all conditions. The proposed modification only introduce change under the condition when only inter, intra and PLT modes are allowed. The corresponding codewords are illustrated in Table 2 which is different from Table 1. Moreover, the signaling conditions of pred_mode_plt_flag can be simplified as highlighted in Table 3.

Table 2. Signaling of modes when only intra, inter and PLT modes are allowed

| Pred_Modes | pred_mode_flag | pred_mode_ibc_flag | pred_mode_plt_flag |
| --- | --- | --- | --- |
| INTER | 0 | - | - |
| PLT | 1 | - | 1 |
| INTRA | 1 | - | 0 |

Table 3. Signaling of the modes when only intra, inter and PLT modes are allowed

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ¦ ¦ sps_palette_enabled_flag) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( ( ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ modeType = = MODE_TYPE_INTRA ) && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && cbWidth <= 64 && cbHeight <= 64 && modeType != MODE_TYPE_INTER && sps_ibc_enabled_flag && treeType != DUAL_TREE_CHROMA ) |  |
| pred_mode_ibc_flag | ae(v) |
| if( ( ( ( slice_type = = I ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ sps_ibc_enabled_flag ) && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) ¦ ¦ ( slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && !sps_ibc_enabled_flag && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) ) && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_INTER ) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |

# Simulation Results

The proposed method is evaluated according to the CE8 description [1]. The HashME, BDPCM, PLT are turned on for all Classes of 444 sequences and IBC is turned off. The results with DualITree OFF and DualITree ON are shown in Table 4 and Table 5.

Table 4 Results of the proposed modification (IBC=0, DualITree = 0)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Animation | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.05% | 0.07% | 0.06% | 100% | 101% |
| TGM 720p | -0.02% | 0.02% | 0.01% | 100% | 100% |
| Animation | 0.10% | 0.09% | 0.13% | 100% | 101% |
| Mixed content | -0.05% | -0.03% | -0.05% | 101% | 100% |
| Camera-Captured content | -0.07% | 0.05% | -0.05% | 100% | 99% |
| Overall | 0.01% | 0.04% | 0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.17% | -0.08% | -0.08% | 99% | 100% |
| TGM 720p | -0.21% | -0.17% | -0.25% | 100% | 99% |
| Animation | 0.07% | 0.08% | 0.12% | 99% | 100% |
| Mixed content | 0.06% | -0.13% | -0.27% | 100% | 99% |
| Camera-Captured content | -0.07% | 0.06% | 0.02% | 102% | 100% |
| Overall | -0.08% | -0.06% | -0.11% | 100% | 100% |

Table 5 Results of the proposed modification (IBC=0, DualITree = 1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Animation | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.01% | 0.00% | 0.01% | 99% | 99% |
| TGM 720p | -0.01% | -0.01% | 0.07% | 99% | 100% |
| Animation | 0.02% | 0.04% | 0.04% | 100% | 99% |
| Mixed content | -0.09% | -0.04% | -0.08% | 100% | 99% |
| Camera-Captured content | -0.11% | 0.01% | -0.13% | 101% | 100% |
| Overall | -0.03% | 0.00% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.02% | 0.13% | 0.11% | 100% | 100% |
| TGM 720p | -0.07% | -0.40% | -0.17% | 101% | 99% |
| Animation | -0.08% | 0.06% | 0.04% | 100% | 100% |
| Mixed content | 0.03% | 0.14% | 0.26% | 99% | 101% |
| Camera-Captured content | -0.10% | -0.08% | 0.03% | 102% | 102% |
| Overall | -0.04% | -0.04% | 0.05% | 100% | 100% |

# Conclusions

In this proposal, it is proposed to signal pred_mode_plt_flag when pred_mode_flag is signaled under all conditions. With the proposed fix, the signaling conditions for pred_mode_plt_flag is simplified and the results on YUV4:4:4 sequences show minor coding performance changes under CE8 CTC with IBC disabled compared to VTM6.0.

# Reference

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).