| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0520 |
| --- | --- |

| Title: | Non-CE3: Spec fix for the smallest chroma intra prediction unit (SCIPU) |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | +1-858-7052600 yiwenchen@kwai.com |  |
| Source: | Kwai Inc. |  |  |  |

_____________________________

# Abstract

When the smallest chroma intra prediction unit (SCIPU) was adopted into VVC, the design only considered the chroma format 4:2:0 and is disabled for chroma format 4:4:4. The other chroma formats are not considered and the design therefore imposed over-restrict rules to the size of intra chroma blocks for the chroma formats 4:2:2 and monochrome. To resolve the over-restrict issue on 4:2:2 and monochrome, it is first proposed to disable SCIPU for monochrome. Second, the derivation of the modeTypeCondition for SCIPU is also modified according to the ratio between the luma and chroma samples when the chroma format is 4:2:2. The proposed changes on the VVC specification draft version 6 are also provided in this contribution to solve the issues of SCIPU on chroma format 4:2:2 and monochrome.

# Introduction

When smallest chroma intra prediction unit (SCIPU) was adopted into VVC [1], the design only considered the chroma format 4:2:0. During the integration of VTM6.0, the SCIPU was agreed to be disabled for 4:4:4 in VTM6.0 (https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/456#comment:1) but is not yet reflected in the VVC draft text. However, the other chroma formats (e.g. 4:2:2 and monochrome) are not considered and the design therefore imposed over-restrict rules to the size of intra chroma blocks for 4:2:2 and monochrome color formats.

# Proposal

First, in this contribution, it is proposed to disable SCIPU for monochrome. Second, the modeTypeCondition derivation for SCIPU is also changed to consider the chroma format 4:2:2. The derivation of the modeTypeCondition considering the chroma format monochrome, 4:2:2 and 4:4:4 based on the VVC draft text JVET-O2001-vE is given in Table 1.

TABLE 1 The spec of the proposed method to consider chroma format monochrome, 4:2:0, 4:2:2 and 4:4:4

The variable modeTypeCondition is derived as follows:

- If one of the following conditions is true, modeTypeCondition is set equal to 0

- slice_type = = I  and  qtbtt_dual_tree_intra_flag is equal to 1

- modeTypeCurr is not equal to MODE_ALL

- chroma_format_idc is equal to 0

- chroma_format_idc is equal to 3

- Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1

- cbWidth * cbHeight is equal to 64 and split_qt_flag is equal to 1

- cbWidth * cbHeight is equal to 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER

- cbWidth * cbHeight is equal to 32 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER

- Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1 + (slice_type  !=  I ? 1 : 0)

- cbWidth * cbHeight is equal to 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER and chroma_format_idc is equal to 1

- cbWidth * cbHeight is equal to 128 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER and chroma_format_idc is equal to 1

- Otherwise, modeTypeCondition is set equal to 0

.

# Conclusions

In current VVC, SCIPU only considers 4:4:4 and 4:2:0. In this proposal, it is proposed to modify the SCIPU to consider all different chroma formats such as 4:4:4, 4:2:2, 4:2:0 and monochrome. Specifically, SCIPU is disabled for monochrome and 4:4:4. Moreover, the derivation of the modeTypeCondition is modified accordingly for the chroma format 4:2:2.

# Reference

- Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, Y. Zhao, H. Yang, “CE3-2.1.1 and CE3-2.1.2: Removing 2x2, 2x4, and 4x2 chroma CBs”, JVET-O0050, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).