| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0521 |
| --- | --- |

| Title: | AHG18: Disabling DMVR and BDOF for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-chuan Ma, Yi-wen Chen, Xiaoyu Xiu, Hong-jheng Jhu, Xianglin Wang, 8910 University Center Dr., Suite 400 San Diego, CA-92122, USA | Email: | {tsung-chuanma, yiwenchen, xiaoyuxiu, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution is proposed to disable the DMVR and BDOF under lossless coding. The result shows that the coding loss is 0.03%, 0.00% for RA, and LDB, respectively

# Introduction

In current VVC [1], the DMVR and BDOF are applied in decoder side to refine the decoded picture. Since the lossless coded is perfectly reconstructed, it is proposed to disable DMVR and BDOF under lossless coding in order to reduce the runtime complexity.

# Experimental results

Tests were carried out by using the VTM6.0 reference software released by AHG18 for lossless coding as an anchor [2].

|  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | TEST |  | AHG18 | TEST |  |
| Class A1 | 2.3 | 2.3 | 0.00% |  |  |  |
| Class A2 | 1.9 | 1.9 | 0.01% |  |  |  |
| Class B | 2.3 | 2.3 | 0.01% | 2.3 | 2.3 | 0.00% |
| Class C | 2.6 | 2.6 | 0.07% | 2.6 | 2.4 | 0.00% |
| Class D | 2.8 | 2.8 | 0.16% | 2.7 | 2.7 | 0.00% |
| Class E |  |  |  | 0.0 | 0.0 | 0.00% |
| Class F | 39.4 | 39.4 | 0.01% | 60.6 | 60.6 | 0.00% |
| TGM | 112.6 | 112.6 | -0.02% | 142.4 | 142.4 | 0.00% |
| Overall | 2.3 | 2.3 | 0.03% | 2.6 | 2.6 | 0.00% |
| Enc Time[%] | 96% |  |  | 101% |  |  |
| Dec Time[%] | 95% |  |  | 101% |  |  |

Table 1: Simulation result

# Working Draft Text

The change is highlight. The proposed changes are listed below. For enabling VVC lossless coding, please refer to JVET-P0514 [3].

### 8.5.1 General decoding process for coding units coded in inter prediction mode

…

The decoding process for coding units coded in inter prediction mode consists of the following ordered steps:

- The variable dmvrFlag is set equal to 0 and the variable hpelIfIdx is set equal to 0.

- The motion vector components and reference indices of the current coding unit are derived as follows:

- If MergeTriangleFlag[ xCb ][ yCb ], inter_affine_flag[ xCb ][ yCb ] and merge_subblock_flag[ xCb ][ yCb ] are all equal to 0, the following applies:

- The derivation process for motion vector components and reference indices as specified in clause 8.5.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], the half sample interpolation filter index hpelIfIdx, and the bi-prediction weight index bcwIdx as outputs.

- When all of the following conditions are true, dmvrFlag is set equal to 1:

- cu_transquantbypass_flag is set equal to 0

- sps_dmvr_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flag is equal to 0

- general_merge_flag[ xCb ][ yCb ] is equal to 1

- both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

- mmvd_merge_flag[ xCb ][ yCb ] is equal to 0

- ciip_flag[ xCb ][ yCb ] is equal to 0

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- BcwIdx[ xCb ][ yCb ] is equal to 0

- Both luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0

- cbWidth is greater than or equal to 8

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 128

- For X being each of 0 and 1, the pic_width_in_luma_samples and pic_height_in_luma_samples of the reference picture refPicLX associated with the refIdxLX are equal to the pic_width_in_luma_samples and pic_height_in_luma_samples of the current picture, respectively.

8.5.6.1 General

…

Let predSamplesL0L, predSamplesL1L and predSamplesIntraL be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr and predSamplesL1Cr, predSamplesIntraCb, and predSamplesIntraCr be (cbWidth / SubWidthC)x(cbHeight / SubHeightC) arrays of predicted chroma sample values.

  - The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - cu_transquantbypass_flag is set equal to 0

      - sps_bdof_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flag is equal to 0.

      - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

      - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

      - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

      - sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

      - ciip_flag[ xCb ][ yCb ] is equal to 0.

      - BcwIdx[ xCb ][ yCb ] is equal to 0.

      - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

      - cbWidth is greater than or equal to 8.

      - cbHeight is greater than or equal to 8.

      - cbHeight * cbWidth is greater than or equal to 128.

      - For X being each of 0 and 1, the pic_width_in_luma_samples and pic_height_in_luma_samples of the reference picture refPicLX associated with the refIdxLX are equal to the pic_width_in_luma_samples and pic_height_in_luma_samples of the current picture, respectively.

      - cIdx is equal to 0.

    - Otherwise, bdofFlag is set equal to FALSE.

# Conclusion

This contribution presents to disable DMVR and BDOF in lossless mode. It is asserted that these two decoder side tools may not be beneficial for lossless coding. Therefore, based on the result of this proposal, it is recommended to disable both DMVR and BDOF under lossless coding.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-O1010, the 15th JVET meeting, July 2019, Gothenburg, SE

[2] AHG18 reference software in at https://vcgit.hhi.fraunhofer.de/jvet-o-ahg-18/VVCSoftware_VTM, branch VTM-6.0-Lossless.

[3] T.-C. Ma, Y.-W. Chen, X. Xiu, H.-J. Jhu, X. Wang, “AHG18: Enable lossless coding for VVC”, JVET-P0514, the 15th JVET meeting, Oct 2019, Geneva, CH

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).