| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0522 |
| --- | --- |

| Title: | Non-CE8: On palette mode syntax |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | +1-858-7052600 yiwenchen@kwai.com |  |
| Source: | Kwai Inc. |  |  |  |

_____________________________

# Abstract

There exist some redundant bits in PLT (palette) mode signaling in current VVC. When the index map of current PLT block contains only one index, signaling of the palette_transpose_flag and the copy_above_indices_for_final_run_flag is redundant. In this proposal, it is proposed to omit the signaling of palette_transpose_flag and copy_above_indices_for_final_run_flag when num_palette_indices_minus1 is signaled as 0.

Moreover, in current palette design, there is no constraint on num_palette_indices_minus1. When num_palette_indices_minus1 is signaled as a number larger than the size of current block, an undefined behavior may occur at the decoder. It is proposed to constrain the num_palette_indices_minus1 to be equal to or less than the size of the PLT mode coded block.

The proposed two modifications are integrated into VTM6.0 and tested on 444 sequences under CE8 CTC. The first modification which removes the redundant signaling introduces negligible coding difference (0.0% AI, 0.0%RA) while the second modification introduce no coding performance change.

# Introduction

In current platte (PLT) mode of VVC, for coding the palette index map, the indices can be coded using either horizontal or vertical traverse scans indicated by an explicitly signalled syntax palette_transpose_flag. However, when the index map of current PLT block contains only one index, the coded index maps are the same no matter it is horizontal or vertical traverse scan order. In other words, the signaling of palette_transpose_flag flag in this case could be omitted. In current design, the palette_transpose_flag is still signaled when the index map contains only one index (e.g. num_palette_indices_minus1 is signaled as 0) which is a redundancy.

The copy_above_indices_for_final_run_flag is also signaled when the index map contains only one index (e.g. num_palette_indices_minus1 is signaled as 0). But when the index map contains only one index, there is no need to signal the copy_above_indices_for_final_run_flag. Signaling of the flag is also redundant in this case.

Moreover, num_palette_indices_minus1 is signaled to indicate the number of indices in the index map of current PLT block. However, it is obvious that the maximum number of the indices should be equal to or smaller than the size of the current block (e.g. block width multiplies block height). In current palette design, there is no constraint on the signaled value of num_palette_indices_minus1. When num_palette_indices_minus1 is signaled as a number larger than the size of current block, an undefined behavior may occur at the decoder.

# Proposal

Two modifications are proposed in this contribution to solve the issues of PLT syntax. First, to remove the redundant signaling, it is proposed to signal the palette_transpose_flag and num_palette_indices_minus1 when the number of indices in the index map of current PLT block is larger than 1. The modified syntax signaling is illustrated in the table below.

| if( MaxPaletteIndex > 0 ) { |  |
| --- | --- |
| num_palette_indices_minus1 | ae(v) |
| adjust = 0 |  |
| for( i = 0; i <= num_palette_indices_minus1; i++ ) { |  |
| if( MaxPaletteIndex − adjust > 0 ) { |  |
| palette_idx_idc | ae(v) |
| PaletteIndexIdc[ i ] = palette_idx_idc |  |
| } |  |
| adjust = 1 |  |
| } |  |
| if(num_palette_indices_minus1>0) { |  |
| copy_above_indices_for_final_run_flag | ae(v) |
| palette_transpose_flag | ae(v) |
| } |  |
| } |  |

In the second modification, it is proposed to constrain the num_palette_indices_minus1 to be equal to or less than the size of the current block. An example based on the semantics of the num_palette_indices_minus1 in the VVC specification is given below：

num_palette_indices_minus1 plus 1 is the number of palette indices explicitly signalled or inferred for the current block. When num_palette_indices_minus1 is not present, it is inferred to be equal to 0. The value of num_palette_indices_minus1 shall be in the range of 0 to cbWidth* cbHeight − 1 inclusive.

# Simulation Results

The proposed method is evaluated according to the CE8 description [1]. The IBC, HashME, BDPCM, PLT are turned on for all Classes of 444 sequences. The results show that the proposed first modifications introduce minor coding performance change with DualITree OFF and DualITree ON which are illustrated in Table 1 and Table 2, respectively. It is also noted that, the second modification introduces no coding performance change since current VTM encoder does not encode a block as PLT mode with the number of PLT indices larger than the block size.

Table 1 Results of the proposed modification (DualITree = 0)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 99% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Animation | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 98% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 101% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Animation | 0.00% | -0.02% | -0.03% | 100% | 100% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.01% | 0.00% | -0.01% | 99% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Animation | -0.01% | 0.01% | -0.01% | 101% | 101% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 2 Results of the proposed modification (DualITree = 1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Animation | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.01% | 0.00% | 101% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Animation | 0.01% | 0.00% | 0.01% | 99% | 100% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | -0.01% | 100% | 100% |
| TGM 720p | -0.01% | 0.00% | 0.02% | 100% | 99% |
| Animation | 0.02% | 0.02% | 0.04% | 100% | 99% |
| Mixed content | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 100% |

# Conclusions

In this proposal, two methods are proposed to solve the issues of PLT syntax. First, to remove the signaling redundancy, the palette_transpose_flag and num_palette_indices_minus1 are signaled only when the number of indices in the index map of current PLT block is larger than 1. Second, to avoid undefined decoder behavior, it is proposed to constrain the num_palette_indices_minus1 to be equal to or less than the size of the PLT mode coded block. The simulation results show that both modifications introduce no coding performace change on 444 sequences under the CE8 CTC.

# Reference

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).