| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0523 |
| --- | --- |

| Title: | Non-CE8: Context coded bin constraint for palette mode signaling |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | +1-858-7052600 yiwenchen@kwai.com |  |
| Source: | Kwai Inc. |  |  |  |

_____________________________

# Abstract

To limit the maximum context coded bin (CCB), current limitation 1.75 CCB per sample is used for each transform block and residual block coding. However, when coding the index map of a PLT coded block, around 2 CCB per sample could be coded by signaling copy_above_palette_indices_flag and palette_run_prefix for each sample. Since the worst-case CCB per sample of palette_run_prefix is 1, it is proposed to only apply the CCB limit to the coding of the syntax element copy_above_palette_indices_flag with the limit of 0.75 CCB per sample. With the proposed modification, the CCB per sample for the index map coding of PLT mode can be limited below 1.75 and the simulation results show that the proposed limitation introduce minor coding performance change for all the 444 sequences under CE8 common test condition. Under dual-tree off condition, it introduces 0.00%AI, -0.02% RA BD-rate changes; under dual-tree on condition, it introduces 0.00%AI, -0.01% RA BD-rate changes.

# Introduction

To limit the maximum context coded bin (CCB) per sample for each transform block and residual block, current limitation is 1.75 CCB per sample for each block. However, when coding the index map of a PLT block, around 2 CCB per sample are coded which is larger than the CCB limitation on the transform block and residual block.

Taking the PLT mode in VTM6.0 as an example, when all the samples in a 64x64 PLT block are coded as “INDEX” mode and the run is “1”, there are 4032 (64x63) CCBs coded to signal that the “copy_above_palette_indices_flag”=0 used for all the samples except for the first row or column of samples; there are 4096(64x64) CCBs coded to signal that the “palette_run_prefix” equal to 0. In total it requires (4032+4096)/(4096) ≈1.98 CCB for each sample to code the index map which is larger than the criterion used for transform and residual block coding.

# Proposal

To be aligned with the CCB limitation of the residual coding in current VVC, 1.75 CCB per sample is also used as a criterion to the syntax elements copy_above_palette_indices_flag and palette_run_prefix which are related to the index map coding for the PLT mode. Because for the syntax palette_run_prefix, the worst-case average number of CCB per sample is 1, the maximum number of CCB needed for coding all the palette_run_prefix syntaxes of a PLT block is equal to the number of samples in the block. Therefore, the overall CCB budget for the coding of copy_above_palette_indices_flag can be set to be:

block_size*1.75 – block_size = block_size*0.75

Here block_size represents the number of samples in the block. As a result, for coding a PLT mode block, the CCB budget is initialized to be the number of samples in the block multiplied by 0.75. During PLT mode syntax coding, CCB budget is checked and updated only when coding the syntax copy_above_palette_indices_flag. Whenever copy_above_palette_indices_flag is coded, the CCB budget is decreased by one. When the CCB budget is equal to or less than 0, the remaining copy_above_palette_indices_flag syntaxes of the block are coded as bypass bins. The draft text of the proposed modification is illustrated in Table1.

Table 1. Draft text of the proposed modification

| palette_coding( x0, y0, cbWidth, cbHeight, startComp, numComps ) { | Descriptor |
| --- | --- |
| palettePredictionFinished = 0 |  |
| NumPredictedPaletteEntries = 0 |  |
| for( predictorEntryIdx = 0; predictorEntryIdx < PredictorPaletteSize[ startComp ] && !palettePredictionFinished && NumPredictedPaletteEntries[ startComp ] < palette_max_size; predictorEntryIdx++ ) { |  |
| palette_predictor_run | ae(v) |
| if( palette_predictor_run != 1 ) { |  |
| if( palette_predictor_run > 1 ) |  |
| predictorEntryIdx += palette_predictor_run − 1 |  |
| PalettePredictorEntryReuseFlags[ predictorEntryIdx ] = 1 |  |
| NumPredictedPaletteEntries++ |  |
| } else |  |
| palettePredictionFinished = 1 |  |
| } |  |
| if( NumPredictedPaletteEntries < palette_max_size ) |  |
| num_signalled_palette_entries | ae(v) |
| for( cIdx = startComp; cIdx < ( startComp + numComps); cIdx++ ) |  |
| for( i = 0; i < num_signalled_palette_entries; i++ ) |  |
| new_palette_entries[ cIdx ][ i ] | ae(v) |
| if( CurrentPaletteSize[ startComp ] > 0 ) |  |
| palette_escape_val_present_flag | ae(v) |
| if( MaxPaletteIndex > 0 ) { |  |
| num_palette_indices_minus1 | ae(v) |
| adjust = 0 |  |
| for( i = 0; i <= num_palette_indices_minus1; i++ ) { |  |
| if( MaxPaletteIndex − adjust > 0 ) { |  |
| palette_idx_idc | ae(v) |
| PaletteIndexIdc[ i ] = palette_idx_idc |  |
| } |  |
| adjust = 1 |  |
| } |  |
| copy_above_indices_for_final_run_flag | ae(v) |
| palette_transpose_flag | ae(v) |
| ccbBudge= (cbWidth* cbHeight*3)>>2 |  |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && palette_escape_val_present_flag ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| if( treeType != DUAL_TREE_LUMA && palette_escape_val_present_flag ) { |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded ) { |  |
| cu_chroma_qp_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag ) |  |
| cu_chroma_qp_offset_idx | ae(v) |
| } |  |
| } |  |
| remainingNumIndices = num_palette_indices_minus1 + 1 |  |
| PaletteScanPos = 0 |  |
| log2CbWidth = Log2( cbWidth ) |  |
| log2CbHeight = Log2( cbHeight ) |  |
| while( PaletteScanPos < cbWidth*cbHeightt ) { |  |
| xC = x0 + TraverseScanOrder[ log2CbWidth ][ log2CbHeight ][ PaletteScanPos ][ 0 ] |  |
| yC = y0 + TraverseScanOrder[ log2CbWidth ][ log2CbHeight ][ PaletteScanPos ][ 1 ] |  |
| if( PaletteScanPos > 0 ) { |  |
| xcPrev = x0 + TraverseScanOrder[ log2CbWidth ][ log2CbHeight ][ PaletteScanPos − 1 ][ 0 ] |  |
| ycPrev = y0 + TraverseScanOrder[ log2CbWidth ][ log2CbHeight ][ PaletteScanPos − 1 ][ 1 ] |  |
| } |  |
| PaletteRunMinus1 = cbWidth * cbHeight − PaletteScanPos − 1 |  |
| RunToEnd = 1 |  |
| CopyAboveIndicesFlag[ xC ][ yC ] = 0 |  |
| if( MaxPaletteIndex > 0 ) |  |
| if( ( ( !palette_transpose_flag && yC > 0 ) ¦ ¦ ( palette_transpose_flag && xC > 0 ) ) && CopyAboveIndicesFlag[ xcPrev ][ ycPrev ] = = 0 ) |  |
| if( remainingNumIndices > 0 && PaletteScanPos < cbWidth* cbHeight − 1 ) { |  |
| copy_above_palette_indices_flag | ae(v) |
| ccbBudge-- |  |
| CopyAboveIndicesFlag[ xC ][ yC ] = copy_above_palette_indices_flag |  |
| } else { |  |
| if( PaletteScanPos = = cbWidth * cbHeight − 1 && remainingNumIndices > 0 ) |  |
| CopyAboveIndicesFlag[ xC ][ yC ] = 0 |  |
| else |  |
| CopyAboveIndicesFlag[ xC ][ yC ] = 1 |  |
| } |  |
| if ( CopyAboveIndicesFlag[ xC ][ yC ] = = 0 ) { |  |
| currNumIndices = num_palette_indices_minus1 + 1 − remainingNumIndices |  |
| PaletteIndexMap[ xC ][ yC ] = PaletteIndexIdc[ currNumIndices ] |  |
| } |  |
| if( MaxPaletteIndex > 0 ) { |  |
| if( CopyAboveIndicesFlag[ xC ][ yC ] = = 0 ) |  |
| remainingNumIndices − = 1 |  |
| if( remainingNumIndices > 0 ¦ ¦ CopyAboveIndicesFlag[ xC ][ yC ] != copy_above_indices_for_final_run_flag ) { |  |
| PaletteMaxRunMinus1 = cbWidth * cbHeight − PaletteScanPos − 1 − remainingNumIndices − copy_above_indices_for_final_run_flag |  |
| RunToEnd = 0 |  |
| if( PaletteMaxRunMinus1 > 0 ) { |  |
| palette_run_prefix | ae(v) |
| if( ( palette_run_prefix > 1 ) && ( PaletteMaxRunMinus1 != ( 1 << ( palette_run_prefix – 1 ) ) ) ) |  |
| palette_run_suffix | ae(v) |
| } |  |
| } |  |
| } |  |
| runPos = 0 |  |
| while ( runPos <= PaletteRunMinus1 ) { |  |
| xR = x0 + TraverseScanOrder[ log2CbWidth ][ log2CbHeight ][ PaletteScanPos ][ 0 ] |  |
| yR = y0 + TraverseScanOrder[ log2CbWidth ][ log2CbHeight ][ PaletteScanPos ][ 1 ] |  |
| if( CopyAboveIndicesFlag[ xC ][ yC ] = = 0 ) { |  |
| CopyAboveIndicesFlag[ xR ][ yR ] = 0 |  |
| PaletteIndexMap[ xR ][ yR ] = PaletteIndexMap[ xC ][ yC ] |  |
| } else { |  |
| CopyAboveIndicesFlag[ xR ][ yR ] = 1 |  |
| if ( !palette_transpose_flag ) |  |
| PaletteIndexMap[ xR ][ yR ] = PaletteIndexMap[ xR ][ yR − 1 ] |  |
| else |  |
| PaletteIndexMap[ xR ][ yR ] = PaletteIndexMap[ xR − 1 ][ yR ] |  |
| } |  |
| runPos++ |  |
| PaletteScanPos ++ |  |
| } |  |
| } |  |
| if( palette_escape_val_present_flag ) { |  |
| for( cIdx = startComp; cIdx < ( startComp + numComps ); cIdx++ ) |  |
| for( sPos = 0; sPos < cbWidth* cbHeight; sPos++ ) { |  |
| xC = x0 + TraverseScanOrder[ log2CbWidth][ log2CbHeight ][ sPos ][ 0 ] |  |
| yC = y0 + TraverseScanOrder[ log2CbWidth][ log2CbHeight ][ sPos ][ 1 ] |  |
| if( PaletteIndexMap[ cIdx ][ xC ][ yC ] = = MaxPaletteIndex ) { |  |
| palette_escape_val | ae(v) |
| PaletteEscapeVal[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| } |  |
| } |  |
| } |  |

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| copy_above_palette_indices_flag | ccbBudge>0?0:bypass | na | na | na | na | na |

# Simulation Results

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, BDPCM, PLT are turned on for all Classes. The results with DualITree OFF and DualITree ON are shown in Table 1 and Table 2.

Table 1: Results of the proposed modification (DualITree = 0)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.02% | -0.02% | -0.03% | 100% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Animation | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | -0.01% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.02% | -0.02% | -0.02% | 100% | 100% |
| TGM 720p | -0.05% | -0.02% | -0.05% | 100% | 99% |
| Animation | -0.01% | 0.03% | 0.02% | 99% | 101% |
| Mixed content | 0.01% | 0.00% | -0.01% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | -0.02% | 0.00% | -0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p |  |  |  |  |  |
| TGM 720p | 0.01% | -0.01% | 0.00% | 100% | 99% |
| Animation |  |  |  |  |  |
| Mixed content | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Camera-Captured content |  |  |  |  |  |
| Overall |  |  |  |  |  |

Table 2: Results of the proposed modification (DualITree = 1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | -0.01% | 0.00% | 100% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.01% | 100% | 99% |
| Animation | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.03% | -0.01% | -0.01% | 100% | 100% |
| TGM 720p | 0.00% | 0.01% | -0.02% | 100% | 100% |
| Animation | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | -0.01% | 0.00% | -0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p |  |  |  |  |  |
| TGM 720p | -0.02% | 0.01% | 0.11% | 100% | 100% |
| Animation |  |  |  |  |  |
| Mixed content |  |  |  |  |  |
| Camera-Captured content |  |  |  |  |  |
| Overall |  |  |  |  |  |

# Conclusions

In this proposal, it is proposed to apply 0.75 CCB limit to the coding of copy_above_palette_indices_flag. With the proposed limitation, it is guaranteed that 1.75 CCB per sample limitation is also applied to the PLT index coding (including the syntax elements copy_above_palette_indices_flag and palette_run_prefix) which aligned with the CCB limit imposed to the transform and residual coding. The results show that the proposed modification introduces minor coding performance change to the 444 sequences under CE8 CTC.

# Reference

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).