| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0526-v2 |
| --- | --- |

| Title: | Non-CE8: Palette encoder improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu Yi-Wen Chen Xiaoyu Xiu Tsung-Chuan Ma Xianglin Wang | Email: | jhuhong-jheng@kwai.com yiwenchen@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In this contribution, two encoder improvements are proposed to enhance the Palette mode (PLT) efficiency of the VTM-6.0 encoder, including 1) fixing the mis-alignment of rate and distortion during palette coding rate-distortion analysis; 2) using different weights on luma component cost and chroma component cost. The proposed method is implemented and evaluated with VTM6 by following CE8 test conditions. Results are reported as below:

Dual tree OFF: -4.04% AI, -3.04% RA, -2.32% LB

Dual tree ON: -1.60% AI, -1.39% RA, -1.40% LB

For version 2, update the result for bug fix.

Introduction

VTM6 [1] supports the palette mode to be used for screen content coding in 4:4:4 color format. On the encoder side, it is necessary to derive the appropriate palette to be used with that CU.

For the derivation of the palette for lossy coding, a modified k-means clustering algorithm is used. The first sample of the block is added to the palette. Then, for each subsequent sample from the block, the sum of absolute distances (SAD) in 8-bit precision between the sample and each of the current palette color is calculated. If the distortion for each of the components is less than a threshold value for the palette entry corresponding to the minimum SAD, the sample is added to the cluster belonging to the palette entry. Otherwise, the sample is added as a new palette entry. When the number of samples mapped to a cluster exceeds a threshold, a centroid for that cluster is updated and becomes the palette entry of that cluster.

In the next step, the clusters are sorted in a descending order of usage. Then, the palette entry corresponding to each entry is updated. Normally, the cluster centroid is used as the palette entry. But a rate-distortion analysis is performed to analyze whether any entry from the palette predictor may be more suitable to be used as the updated palette entry instead of the centroid when the cost of coding the palette entries is taken into account. Note the precision of the distortion part is 8-bit precision. This process is continued till all the clusters are processed or the maximum palette size is reached. Finally, if a cluster has only a single sample and the corresponding palette entry is not in the palette predictor, the sample is converted to an escape symbol. Additionally, duplicate palette entries are removed and their clusters are merged.

After palette derivation, each sample in the block is assigned the index of the nearest (in SAD and 8-bit precision) palette entry.

Proposal

To further improve the palette coding efficiency, the following non-normative changes are proposed in the VTM-6.0:

- Firstly, it is proposed to fix the mis-alignment of rate and distortion during palette coding RD decision. In current CE8 test conditions, the palette distortion is scaled down to 8 bit scale. However, the Lagrangian cost parameter Lambda being used in calculating the bitCost is still determined by the internal coding bit depth which is 10. It is reasonable to use 10 bits precision to calculate the distortion used in encoder side, including SAD and rate-distortion analysis.

- Secondly, it is proposed to use different weights on luma component cost and chroma component cost. Luma component is more important than chroma component. It is reasonable to reduce the chroma distortion used in encoder side, including SAD and rate-distortion analysis.

Experimental results

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, BDPCM, PLT are turned on for all Classes. The results with DualITree OFF and DualITree ON are shown in Table 1 and Table 2.

Table 1: Results of proposed modification, DualITree = 0 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -7.42% | -8.85% | -9.50% | 100% | 98% |
| TGM 720p | -5.37% | -5.82% | -8.45% | 103% | 101% |
| Animation | -0.68% | -2.21% | -2.37% | 102% | 96% |
| Mixed content | -3.80% | -10.01% | -10.82% | 100% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 97% | 97% |
| Overall | -4.04% | -5.96% | -6.96% | 101% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -5.27% | -6.53% | -7.14% | 103% | 100% |
| TGM 720p | -4.67% | -6.06% | -7.83% | 100% | 99% |
| Animation | 0.02% | -1.00% | -1.19% | 99% | 99% |
| Mixed content | -2.99% | -8.80% | -9.30% | 98% | 98% |
| Camera-Captured content | 0.00% | 0.00% | -0.01% | 103% | 101% |
| Overall | -3.04% | -4.98% | -5.71% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -3.99% | -5.37% | -5.78% | 101% | 101% |
| TGM 720p | -3.47% | -4.91% | -6.31% | 99% | 99% |
| Animation | 0.06% | -0.82% | -0.82% | 101% | 104% |
| Mixed content | -2.49% | -8.32% | -9.94% | 101% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | -2.32% | -4.28% | -5.04% | 101% | 100% |

Table 2: Results of proposed modification, DualITree = 1 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2.28% | -2.15% | -2.45% | 100% | 98% |
| TGM 720p | -3.11% | -0.25% | -0.57% | 98% | 100% |
| Animation | -0.10% | -0.69% | -0.54% | 101% | 100% |
| Mixed content | -1.24% | -1.19% | -2.14% | 99% | 94% |
| Camera-Captured content | 0.00% | 0.01% | -0.01% | 107% | 104% |
| Overall | -1.60% | -0.95% | -1.26% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2.51% | -2.96% | -2.90% | 99% | 98% |
| TGM 720p | -2.42% | -1.03% | -1.26% | 101% | 100% |
| Animation | 0.32% | -0.30% | -0.36% | 98% | 97% |
| Mixed content | -1.17% | -0.72% | -1.82% | 100% | 98% |
| Camera-Captured content | -0.02% | 0.00% | -0.01% | 101% | 100% |
| Overall | -1.39% | -1.19% | -1.45% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2.86% | -4.04% | -4.03% | 98% | 98% |
| TGM 720p | -1.97% | -1.92% | -3.30% | 100% | 102% |
| Animation | 0.29% | -0.29% | -0.51% | 97% | 92% |
| Mixed content | -1.29% | -0.83% | -2.20% | 98% | 97% |
| Camera-Captured content | 0.00% | -0.07% | -0.07% | 100% | 93% |
| Overall | -1.40% | -1.71% | -2.35% | 99% | 97% |

References

- VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, 15th meeting, Gothenburg, SE, July 2019.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).