| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0529-v2 |
| --- | --- |

| Title: | Non-CE8: QP dependent binarization for palette escape |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu Yi-Wen Chen Xiaoyu Xiu Tsung-Chuan Ma Xianglin Wang | Email: | jhuhong-jheng@kwai.com yiwenchen@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, three methods are proposed for Palette mode (PLT), including 1) a minimum allowed quantization parameter used in escape mode coding. To be consistent with Transform skip minimum Qp; 2) a new binarization method for palette escape coding, where the binarization of quantized escape samples is adjusted based on the dynamic range of the quantized values; 3) QP remapping is introduced. The proposed methods is implemented and evaluated with VTM6 by following CE8 test conditions. Results are reported as below:

Test 1: minimum QP used in escape mode coding to be consistent with Transform skip minimum QP

Dual tree OFF:

Stnadard QP (22, 27, 32, 37): 0.00 % AI, 0.00 % RA, 0.00 % LB

Dual tree ON:

Stnadard QP (22, 27, 32, 37): 0.00% AI, 0.00% RA, 0.00 % LB

Test 2: Test 1 + a new binarization method for palette escape coding

Dual tree OFF:

Stnadard QP (22, 27, 32, 37): 0.01% AI, -0.09 % RA, 0.10 % LB

Low QP (2, 7, 12, 17): -1.15% AI, -1.12% RA, -0.53% LB

Dual tree ON:

Stnadard QP (22, 27, 32, 37): -0.04% AI, 0.00% RA, -0.02 % LB

Low QP (2, 7, 12, 17): -0.85% AI, -0.76% RA, -0.51% LB

Test 3: Test 2 + QP remapping

Dual tree OFF:

Stnadard QP (22, 27, 32, 37): -0.12% AI, -0.12% RA, 0.03 % LB

Low QP (2, 7, 12, 17): -1.46% AI, -1.19% RA, -0.70% LB

Dual tree ON:

Stnadard QP (22, 27, 32, 37): -0.06% AI, -0.01% RA, 0.02 % LB

Low QP (2, 7, 12, 17): -1.05% AI, -0.85% RA, -0.55% LB

# Introduction

In current VVC [1], the binarization of escape samples is derived by invoking the third order Exp-Golomb binarization process. In the proposal, it proposes to limit the minimum QP used in escape mode coding to be consistent with Transform skip minimum QP. And a new binarization method is proposed for palette escape coding which adjust the binarization of quantized escape samples based on the corresponding QP value.

# Proposal

First, it proposes to limit the minimum QP used in escape mode coding to be consistent with Transform skip minimum QP.

Second, it is proposed to use the fix-length binarization process for escape samples. The length of fix-length binarization process is derived as follows:

length = max (1, bitDepth – (max (QpPrimeTsMin, Qp) - 4)/6) .

Third, QP remapping is introduced, the QP for escape sample quantization process is derived as:

QPesca = ((max( QpPrimeTsMin , QPcu ) – 2) / 6) * 6 + 4

Where the QPcu is the QP of the given CU and the minimum QPcu should be 4, as first method. If the CU is coded in palette mode, the actual QP used in quantizing escape samples of the CU is QPesca.

Test 1: Only first proposed method.

Test 2: Test 1 + fix-length binarization process for escape samples.

Test 3: Test 2 + QP remapping.

# Experimental results

## Test 1 on Test sequences 4:4:4

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, BDPCM, PLT are turned on for all Classes. The results with DualITree OFF and DualITree ON are shown in Table 1 and Table 2.

Table 1: Results of proposed modification, DualITree = 0 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 106% | 107% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 99% | 105% |
| Animation | 0.00% | 0.00% | 0.00% | 100% | 96% |
| Mixed content | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 91% | 93% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 99% | 98% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 94% | 93% |
| Animation | 0.00% | 0.00% | 0.00% | 97% | 98% |
| Mixed content | 0.00% | 0.00% | 0.00% | 95% | 98% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 97% | 97% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 104% | 102% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 102% | 107% |
| Animation | 0.00% | 0.00% | 0.00% | 97% | 100% |
| Mixed content | 0.00% | 0.00% | 0.00% | 97% | 93% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |

Table 2: Results of proposed modification, DualITree = 1 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 99% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 93% | 98% |
| Animation | 0.00% | 0.00% | 0.00% | 105% | 109% |
| Mixed content | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 97% | 97% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 92% | 92% |
| Animation | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Mixed content | 0.00% | 0.00% | 0.00% | 98% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 97% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 101% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Animation | 0.00% | 0.00% | 0.00% | 99% | 94% |
| Mixed content | 0.00% | 0.00% | 0.00% | 97% | 95% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 102% | 94% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 98% |

## Test 2 on Test sequences 4:4:4

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, BDPCM, PLT are turned on for all Classes. The results with DualITree OFF and DualITree ON are shown in Table 3 and Table 4.

Table 3: Results of proposed modification, DualITree = 0 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.10% | -0.19% | -0.19% | 99% | 100% |
| TGM 720p | 0.18% | -0.21% | -0.06% | 97% | 100% |
| Animation | -0.02% | -0.02% | 0.00% | 97% | 99% |
| Mixed content | -0.06% | -0.09% | -0.16% | 99% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 96% | 97% |
| Overall | 0.01% | -0.12% | -0.09% | 98% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.24% | -0.25% | -0.18% | 100% | 100% |
| TGM 720p | -0.04% | -0.10% | -0.05% | 93% | 92% |
| Animation | -0.03% | -0.10% | -0.10% | 94% | 97% |
| Mixed content | -0.08% | -0.01% | -0.30% | 95% | 97% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | -0.09% | -0.11% | -0.13% | 96% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.03% | 0.13% | -0.03% | 102% | 100% |
| TGM 720p | 0.32% | -0.02% | 0.78% | 98% | 101% |
| Animation | -0.03% | 0.04% | 0.06% | 96% | 98% |
| Mixed content | 0.07% | 0.28% | -0.01% | 98% | 96% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Overall | 0.10% | 0.09% | 0.20% | 99% | 100% |

Table 4: Results of proposed modification, DualITree = 1 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.06% | -0.02% | -0.05% | 98% | 99% |
| TGM 720p | -0.09% | -0.05% | 0.01% | 95% | 100% |
| Animation | -0.02% | 0.00% | -0.01% | 97% | 99% |
| Mixed content | 0.01% | -0.01% | -0.03% | 101% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 97% | 98% |
| Overall | -0.04% | -0.02% | -0.02% | 98% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.13% | -0.13% | -0.14% | 95% | 96% |
| TGM 720p | 0.00% | -0.13% | -0.21% | 92% | 90% |
| Animation | 0.13% | 0.05% | 0.07% | 97% | 98% |
| Mixed content | 0.07% | -0.05% | -0.06% | 97% | 98% |
| Camera-Captured content | -0.01% | 0.04% | -0.01% | 96% | 99% |
| Overall | 0.00% | -0.06% | -0.09% | 95% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.09% | -0.01% | 104% | 103% |
| TGM 720p | 0.11% | -0.07% | 0.35% | 101% | 102% |
| Animation | -0.02% | 0.06% | -0.01% | 96% | 95% |
| Mixed content | -0.23% | -0.21% | -0.30% | 96% | 93% |
| Camera-Captured content | -0.01% | -0.01% | -0.05% | 102% | 98% |
| Overall | -0.02% | -0.03% | 0.02% | 100% | 99% |

## Test 3 on Test sequences 4:4:4

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, BDPCM, PLT are turned on for all Classes. The results with DualITree OFF and DualITree ON are shown in Table 5 and Table 6.

Table 5: Results of proposed modification, DualITree = 0 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.33% | -0.46% | -0.52% | 98% | 95% |
| TGM 720p | 0.04% | -0.08% | -0.22% | 99% | 98% |
| Animation | -0.04% | -0.04% | -0.07% | 97% | 90% |
| Mixed content | -0.24% | -0.55% | -0.56% | 95% | 93% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Overall | -0.12% | -0.25% | -0.30% | 97% | 95% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.33% | -0.31% | -0.29% | 101% | 99% |
| TGM 720p | -0.08% | -0.04% | 0.08% | 99% | 98% |
| Animation | 0.01% | -0.04% | -0.09% | 98% | 98% |
| Mixed content | -0.13% | -0.08% | -0.22% | 97% | 98% |
| Camera-Captured content | 0.00% | -0.01% | 0.00% | 105% | 101% |
| Overall | -0.12% | -0.11% | -0.11% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | -0.11% | -0.15% | 102% | 97% |
| TGM 720p | 0.00% | -0.52% | 0.20% | 95% | 96% |
| Animation | -0.05% | 0.08% | -0.02% | 94% | 94% |
| Mixed content | 0.24% | 0.08% | -0.03% | 94% | 92% |
| Camera-Captured content | 0.00% | -0.01% | 0.01% | 101% | 101% |
| Overall | 0.03% | -0.13% | 0.00% | 97% | 96% |

Table 6: Results of proposed modification, DualITree = 1 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.10% | -0.03% | -0.06% | 100% | 103% |
| TGM 720p | -0.13% | -0.09% | -0.04% | 95% | 98% |
| Animation | -0.03% | -0.06% | -0.01% | 101% | 101% |
| Mixed content | 0.02% | 0.02% | 0.01% | 101% | 99% |
| Camera-Captured content | -0.01% | 0.04% | 0.00% | 104% | 105% |
| Overall | -0.06% | -0.03% | -0.02% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.20% | -0.20% | -0.17% | 100% | 100% |
| TGM 720p | 0.01% | -0.16% | -0.26% | 101% | 99% |
| Animation | 0.16% | 0.11% | 0.10% | 100% | 100% |
| Mixed content | 0.03% | 0.01% | 0.05% | 99% | 98% |
| Camera-Captured content | -0.02% | 0.02% | -0.02% | 98% | 99% |
| Overall | -0.01% | -0.06% | -0.08% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.12% | -0.22% | -0.26% | 96% | 96% |
| TGM 720p | 0.31% | 0.09% | 0.38% | 99% | 96% |
| Animation | -0.02% | 0.03% | 0.03% | 94% | 92% |
| Mixed content | -0.11% | -0.36% | -0.22% | 90% | 94% |
| Camera-Captured content | 0.02% | 0.00% | -0.12% | 101% | 95% |
| Overall | 0.02% | -0.09% | -0.02% | 96% | 95% |

# Additional results

## Test 2 on Test sequences 4:4:4

The proposed method is evaluated for low QP (QP = 2, 7, 12, 17) according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, BDPCM, and PLT are turned on for all classes. The results with DualITree OFF and DualITree ON are shown in Table 7 and Table 8.

Table 7: Results of CE8.1-3, DualITree = 0 for low QP (2, 7, 12, 17)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT0_LQP, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2.66% | -3.19% | -3.26% | 96% | 97% |
| TGM 720p | -0.80% | -1.10% | -1.09% | 97% | 97% |
| Animation | -0.39% | -0.62% | -0.66% | 99% | 100% |
| Mixed content | -1.12% | -1.28% | -1.32% | 98% | 95% |
| Camera-Captured content | 0.00% | 0.00% | -0.01% | 103% | 106% |
| Overall | -1.15% | -1.43% | -1.46% | 98% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0_LQP, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2.57% | -2.37% | -2.42% | 99% | 96% |
| TGM 720p | -0.89% | -0.81% | -0.97% | 99% | 96% |
| Animation | -0.37% | -0.27% | -0.33% | 100% | 97% |
| Mixed content | -1.00% | -0.84% | -0.94% | 95% | 93% |
| Camera-Captured content | 0.01% | 0.00% | 0.02% | 99% | 99% |
| Overall | -1.12% | -1.00% | -1.08% | 98% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0_LQP, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.49% | -1.61% | -1.58% | 97% | 98% |
| TGM 720p | -0.11% | -0.29% | -0.29% | 95% | 95% |
| Animation | -0.52% | -0.28% | -0.37% | 94% | 95% |
| Mixed content | -0.20% | -0.43% | -0.54% | 99% | 98% |
| Camera-Captured content | 0.01% | 0.02% | 0.01% | 99% | 100% |
| Overall | -0.53% | -0.60% | -0.64% | 96% | 97% |

Table 10: Results of the CE8.1-3 on test sequences 4:4:4, DualITree = 1, Low QP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT1_LQP, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.68% | -2.25% | -2.23% | 97% | 98% |
| TGM 720p | -0.51% | -0.75% | -0.70% | 103% | 101% |
| Animation | -0.26% | -0.20% | -0.27% | 97% | 96% |
| Mixed content | -1.36% | -1.58% | -1.63% | 98% | 99% |
| Camera-Captured content | 0.00% | -0.01% | 0.00% | 103% | 97% |
| Overall | -0.85% | -1.08% | -1.09% | 99% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1_LQP, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.40% | -1.77% | -1.76% | 96% | 98% |
| TGM 720p | -0.60% | -0.48% | -0.62% | 97% | 95% |
| Animation | -0.13% | -0.35% | -0.32% | 99% | 97% |
| Mixed content | -1.24% | -0.25% | -0.30% | 95% | 95% |
| Camera-Captured content | -0.01% | 0.01% | -0.03% | 95% | 94% |
| Overall | -0.76% | -0.67% | -0.72% | 97% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1_LQP, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.16% | -1.25% | -1.27% | 98% | 99% |
| TGM 720p | -0.10% | -0.42% | -0.35% | 94% | 93% |
| Animation | -0.67% | -0.34% | -0.34% | 97% | 99% |
| Mixed content | -0.37% | -0.11% | -0.14% | 100% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.01% | 95% | 95% |
| Overall | -0.51% | -0.50% | -0.49% | 97% | 97% |

## Test 3 on Test sequences 4:4:4

The proposed method is evaluated for low QP (QP = 2, 7, 12, 17) according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, BDPCM, and PLT are turned on for all classes. The results with DualITree OFF and DualITree ON are shown in Table 9 and Table 10.

Table 9: Results of CE8.1-3, DualITree = 0 for low QP (2, 7, 12, 17)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT0_LQP, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -3.33% | -4.60% | -4.66% | 101% | 100% |
| TGM 720p | -0.99% | -1.54% | -1.56% | 98% | 96% |
| Animation | -0.53% | -0.63% | -0.58% | 96% | 97% |
| Mixed content | -1.47% | -2.00% | -2.13% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Overall | -1.46% | -2.03% | -2.06% | 99% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0_LQP, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2.78% | -2.87% | -2.89% | 96% | 98% |
| TGM 720p | -0.86% | -1.10% | -1.25% | 101% | 97% |
| Animation | -0.42% | -0.37% | -0.45% | 97% | 97% |
| Mixed content | -1.08% | -1.10% | -1.30% | 95% | 94% |
| Camera-Captured content | 0.01% | -0.01% | 0.01% | 97% | 99% |
| Overall | -1.19% | -1.27% | -1.36% | 97% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0_LQP, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.69% | -2.17% | -2.10% | 96% | 100% |
| TGM 720p | -0.29% | -0.62% | -0.81% | 96% | 97% |
| Animation | -0.44% | -0.63% | -0.67% | 98% | 98% |
| Mixed content | -0.64% | -1.61% | -1.78% | 99% | 100% |
| Camera-Captured content | 0.01% | 0.01% | 0.01% | 102% | 101% |
| Overall | -0.70% | -1.12% | -1.18% | 98% | 99% |

Table 10: Results of the CE8.1-3 on test sequences 4:4:4, DualITree = 1, Low QP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT1_LQP, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2.04% | -3.55% | -3.55% | 96% | 98% |
| TGM 720p | -0.64% | -1.14% | -1.17% | 94% | 95% |
| Animation | -0.30% | -0.33% | -0.46% | 96% | 98% |
| Mixed content | -1.72% | -2.85% | -2.88% | 96% | 97% |
| Camera-Captured content | 0.01% | -0.02% | -0.01% | 96% | 95% |
| Overall | -1.05% | -1.77% | -1.81% | 95% | 97% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1_LQP, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.57% | -2.24% | -2.21% | 94% | 95% |
| TGM 720p | -0.61% | -0.88% | -1.07% | 96% | 93% |
| Animation | -0.22% | -0.48% | -0.48% | 100% | 97% |
| Mixed content | -1.40% | -0.47% | -0.53% | 97% | 95% |
| Camera-Captured content | 0.00% | 0.01% | -0.03% | 97% | 98% |
| Overall | -0.85% | -0.96% | -1.01% | 97% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1_LQP, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.23% | -1.65% | -1.65% | 99% | 99% |
| TGM 720p | -0.05% | -0.62% | -0.51% | 93% | 91% |
| Animation | -0.75% | -0.49% | -0.51% | 97% | 97% |
| Mixed content | -0.46% | -0.93% | -0.89% | 99% | 102% |
| Camera-Captured content | 0.01% | 0.00% | 0.02% | 96% | 96% |
| Overall | -0.55% | -0.84% | -0.80% | 97% | 97% |

# Working Draft Text

## Test 1

The changes to align the scanning order of residual blocks in the VVC working draft [1] is highlight below.

8.4.5.3	Decoding process for palette mode

…

- Otherwise (bIsEscapeSample is equal to 1 and cu_transquant_bypass_flag is equal to 0), the following ordered steps apply:

- The quantization parameter qP is derived as follows:

- If cIdx is equal to 0,

qP = Max( 0 QpPrimeTsMin, Qp′Y )		(8-272)

- Otherwise, if cIdx is equal to 1,

qP = Max( 0 QpPrimeTsMin, Qp′Cb )		(8-273)

- Otherwise (cIdx is equal to 2),

qP = Max( 0 QpPrimeTsMin, Qp′Cr )		(8-274)

- The variables bitDepth is derived as follows:

bitDepth = ( cIdx  = =  0 )  ?  BitDepthY  :  BitDepthC	(8-275)

- The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

- The following applies:

tmpVal = ( PaletteEscapeVal[ cIdx ][ xCb + xL ][ yCb + yL ] *
			levelScale[ qP%6 ] )  <<  ( qP / 6 ) + 32 )  >>  6	(8-276)

recSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, tmpVal )

…

## Test 2

The changes to align the scanning order of residual blocks in the VVC working draft [1] is highlight below.

8.4.5.3	Decoding process for palette mode

…

- Otherwise (bIsEscapeSample is equal to 1 and cu_transquant_bypass_flag is equal to 0), the following ordered steps apply:

- The quantization parameter qP is derived as follows:

- If cIdx is equal to 0,

qP = Max( 0 QpPrimeTsMin, Qp′Y )		(8-272)

- Otherwise, if cIdx is equal to 1,

qP = Max( 0 QpPrimeTsMin, Qp′Cb )		(8-273)

- Otherwise (cIdx is equal to 2),

qP = Max( 0 QpPrimeTsMin, Qp′Cr )		(8-274)

- The variables bitDepth is derived as follows:

bitDepth = ( cIdx  = =  0 )  ?  BitDepthY  :  BitDepthC	(8-275)

- The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

- The following applies:

tmpVal = ( PaletteEscapeVal[ cIdx ][ xCb + xL ][ yCb + yL ] *
			levelScale[ qP%6 ] )  <<  ( qP / 6 ) + 32 )  >>  6	(8-276)

recSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, tmpVal )

…

Table 9 77 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |

…

| palette_coding( ) | palette_predictor_run | EG0 | - |
| --- | --- | --- | --- |
|  | num_signalled_palette_entries | EG0 | - |
|  | new_palette_entries | FL | cMax = cIdx = = 0 ? ( ( 1 << BitDepthY ) − 1 ) : ( (1 << BitDepthC ) − 1 ) |
|  | palette_escape_val_present_flag | FL | cMax = 1 |
|  | num_palette_indices_minus1 | 9.5.3.13 | MaxPaletteIndex |
|  | palette_idx_idc | 9.5.3.14 | MaxPaletteIndex |
|  | copy_above_indices_for_final_run_flag | FL | cMax = 1 |
|  | palette_transpose_flag | FL | cMax = 1 |
|  | copy_above_palette_indices_flag | FL | cMax = 1 |
|  | palette_run_prefix | TR | cMax = Floor( Log2( PaletteMaxRunMinus1 ) ) + 1, cRiceParam = 0 |
|  | palette_run_suffix | TB | cMax = ( PrefixOffset << 1 ) > PaletteMaxRunMinus1 ? ( PalletMaxRun − PrefixOffset ) : ( PrefixOffset − 1 ) |
|  | palette_escape_val | EG3FL | cMax = max(1, bitDepth - (Max(QpPrimeTsMin, Qp ) – 4)/6) |

## Test 3

The changes to align the scanning order of residual blocks in the VVC working draft [1] is highlight below.

8.4.5.3	Decoding process for palette mode

…

- Otherwise (bIsEscapeSample is equal to 1 and cu_transquant_bypass_flag is equal to 0), the following ordered steps apply:

- The quantization parameter qP is derived as follows:

- If cIdx is equal to 0,

qP = Max( 0 QpPrimeTsMin, Qp′Y )		(8-272)

- Otherwise, if cIdx is equal to 1,

qP = Max( 0 QpPrimeTsMin, Qp′Cb )		(8-273)

- Otherwise (cIdx is equal to 2),

qP = Max( 0 QpPrimeTsMin, Qp′Cr )		(8-274)

- The variables bitDepth is derived as follows:

bitDepth = ( cIdx  = =  0 )  ?  BitDepthY  :  BitDepthC	(8-275)

- The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

- The following applies:

qP = ((Max( 4 , qP ) – 2) / 6) * 6 + 4

tmpVal = ( PaletteEscapeVal[ cIdx ][ xCb + xL ][ yCb + yL ] *
			levelScale[ qP%6 ] )  <<  ( qP / 6 ) + 32 )  >>  6	(8-276)

recSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, tmpVal )

…

Table 9 77 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |

…

| palette_coding( ) | palette_predictor_run | EG0 | - |
| --- | --- | --- | --- |
|  | num_signalled_palette_entries | EG0 | - |
|  | new_palette_entries | FL | cMax = cIdx = = 0 ? ( ( 1 << BitDepthY ) − 1 ) : ( (1 << BitDepthC ) − 1 ) |
|  | palette_escape_val_present_flag | FL | cMax = 1 |
|  | num_palette_indices_minus1 | 9.5.3.13 | MaxPaletteIndex |
|  | palette_idx_idc | 9.5.3.14 | MaxPaletteIndex |
|  | copy_above_indices_for_final_run_flag | FL | cMax = 1 |
|  | palette_transpose_flag | FL | cMax = 1 |
|  | copy_above_palette_indices_flag | FL | cMax = 1 |
|  | palette_run_prefix | TR | cMax = Floor( Log2( PaletteMaxRunMinus1 ) ) + 1, cRiceParam = 0 |
|  | palette_run_suffix | TB | cMax = ( PrefixOffset << 1 ) > PaletteMaxRunMinus1 ? ( PalletMaxRun − PrefixOffset ) : ( PrefixOffset − 1 ) |
|  | palette_escape_val | EG3FL | cMax = max(1, bitDepth - (Max(QpPrimeTsMin, Qp ) – 4)/6) |

# References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, the 15th JVET meeting, Gothenburg, SE, 3–12 July 2019

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).