| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0531 |
| --- | --- |

| Title: | Non-CE3: Removal of 2x8 chroma Intra blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

The proposed changes on top of VVC draft 6 [1] are described as below.

## Test #1: remove 2×8 chroma intra blocks

### 6.4.2. Allowed binary split process

The variable allowBtSplit is derived as follows:

- If one or more of the following conditions are true, allowBtSplit is set equal to FALSE:

- cbSize is less than or equal to MinBtSizeY

- cbWidth is greater than maxBtSize

- cbHeight is greater than maxBtSize

- mttDepth is greater than or equal to maxMttDepth

- treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) * ( cbHeight / SubHeightC ) is less than or equal to 16

- treeType is equal to DUAL_TREE_CHROMA and btSplit is equal to SPLIT_BT_VER and ( cbWidth / SubWidthC ) is equal to 4 and ( cbHeight / SubHeightC ) is equal to 8

- treeType is equal to DUAL_TREE_CHROMA and btSplit is equal to SPLIT_BT_HOR and ( cbWidth / SubWidthC ) is equal to 2 and ( cbHeight / SubHeightC ) is equal to 16

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

…

### 6.4.3. Allowed ternary split process

The variable allowTtSplit is derived as follows:

- If one or more of the following conditions are true, allowTtSplit is set equal to FALSE:

- cbSize is less than or equal to 2 * MinTtSizeY

- cbWidth is greater than Min( MaxTbSizeY, maxTtSize )

- cbHeight is greater than Min( MaxTbSizeY, maxTtSize )

- mttDepth is greater than or equal to maxMttDepth

- x0 + cbWidth is greater than pic_width_in_luma_samples

- y0 + cbHeight is greater than pic_height_in_luma_samples

- treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) * ( cbHeight / SubHeightC ) is less than or equal to 32

- treeType is equal to DUAL_TREE_CHROMA and btSplit is equal to SPLIT_TT_VER and ( cbWidth / SubWidthC ) is  equal to 8 and ( cbHeight / SubHeightC ) is equal to 8

- treeType is equal to DUAL_TREE_CHROMA and btSplit is equal to SPLIT_TT_HOR and ( cbWidth / SubWidthC ) is equal to 2 and ( cbHeight / SubHeightC ) is equal to 32

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

–	Otherwise, allowTtSplit is set equal to TRUE.

### 7.4.9.4. Coding tree semantics

The variable modeTypeCondition is derived as follows:

- If one of the following conditions is true, modeTypeCondition is set equal to 0

- slice_type = = I  and  qtbtt_dual_tree_intra_flag is equal to 1

- modeTypeCurr is not equal to MODE_TYPE_ALL

- Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1

- cbWidth * cbHeight is equal to 64 and split_qt_flag is equal to 1

- cbWidth * cbHeight is equal to 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER

- cbWidth * cbHeight is equal to 32 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER

- Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1 + (slice_type  !=  I ? 1 : 0)

- cbWidth * cbHeight is equal to 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER

- cbWidth * cbHeight is equal to 128 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER

- cbWidth is equal to 8 and ( cbHeight / SubHeightC ) is equal to 8 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_VER

- cbWidth is equal to 16 and ( cbHeight / SubHeightC ) is equal to 8 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_VER

- cbWidth is equal to 4 and ( cbHeight / SubHeightC ) is equal to 16 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_TT_HOR

- cbWidth is equal to 4 and ( cbHeight / SubHeightC ) is equal to 32 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR

- Otherwise, modeTypeCondition is set equal to 0

### 7.4.9.7. Merge data semantics

ciip_flag[ x0 ][ y0 ] specifies whether the combined inter-picture merge and intra-picture prediction is applied for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When ciip_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- sps_ciip_enabled_flag is equal to 1.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- merge_subblock_flag[ x0 ][ y0 ] is equal to 0.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- cbWidth is less than 128.

- cbHeight is less than 128.

- cbWidth is greater than 4 and cbHeight is not equal to 16.

- cbWidth * cbHeight is greater than or equal to 64.

- Otherwise, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 0.

### 7.3.8.7.Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( cbWidth * cbHeight ) >= 64 && ( (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128) ¦ ¦ ( sps_triangle_enabled_flag && MaxNumTriangleMergeCand > 1 && slice_type = = B ) ) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( sps_mmvd_enabled_flag ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_triangle_enabled_flag && MaxNumTriangleMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && !(cbWidth = = 4 && cbWidth = = 16) && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && MaxNumTriangleMergeCand > 1 ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Test #2: remove 2×N chroma intra blocks

### 6.4.2. Allowed binary split process

The variable allowBtSplit is derived as follows:

- If one or more of the following conditions are true, allowBtSplit is set equal to FALSE:

- cbSize is less than or equal to MinBtSizeY

- cbWidth is greater than maxBtSize

- cbHeight is greater than maxBtSize

- mttDepth is greater than or equal to maxMttDepth

- treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) * ( cbHeight / SubHeightC ) is less than or equal to 16

- treeType is equal to DUAL_TREE_CHROMA and btSplit is equal to SPLIT_BT_VER and ( cbWidth / SubWidthC ) is equal to 4

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

…

### 6.4.3. Allowed ternary split process

The variable allowTtSplit is derived as follows:

- If one or more of the following conditions are true, allowTtSplit is set equal to FALSE:

- cbSize is less than or equal to 2 * MinTtSizeY

- cbWidth is greater than Min( MaxTbSizeY, maxTtSize )

- cbHeight is greater than Min( MaxTbSizeY, maxTtSize )

- mttDepth is greater than or equal to maxMttDepth

- x0 + cbWidth is greater than pic_width_in_luma_samples

- y0 + cbHeight is greater than pic_height_in_luma_samples

- treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) * ( cbHeight / SubHeightC ) is less than or equal to 32

- treeType is equal to DUAL_TREE_CHROMA and btSplit is equal to SPLIT_TT_VER and ( cbWidth / SubWidthC ) is equal to 8

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

–	Otherwise, allowTtSplit is set equal to TRUE.

### 7.4.9.4. Coding tree semantics

The variable modeTypeCondition is derived as follows:

- If one of the following conditions is true, modeTypeCondition is set equal to 0

- slice_type = = I  and  qtbtt_dual_tree_intra_flag is equal to 1

- modeTypeCurr is not equal to MODE_TYPE_ALL

- Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1

- cbWidth * cbHeight is equal to 64 and split_qt_flag is equal to 1

- cbWidth * cbHeight is equal to 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER

- cbWidth * cbHeight is equal to 32 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER

- Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1 + (slice_type  !=  I ? 1 : 0)

- cbWidth * cbHeight is equal to 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER

- cbWidth * cbHeight is equal to 128 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER

- cbWidth is equal to 8 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_VER

- cbWidth is equal to 16 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_VER

- Otherwise, modeTypeCondition is set equal to 0

### 7.4.9.7. Merge data semantics

ciip_flag[ x0 ][ y0 ] specifies whether the combined inter-picture merge and intra-picture prediction is applied for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When ciip_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- sps_ciip_enabled_flag is equal to 1.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- merge_subblock_flag[ x0 ][ y0 ] is equal to 0.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- cbWidth is less than 128.

- cbHeight is less than 128.

- cbWidth is greater than 4.

- cbWidth * cbHeight is greater than or equal to 64.

- Otherwise, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 0.

### 7.3.8.7.Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( cbWidth * cbHeight ) >= 64 && ( (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128) ¦ ¦ ( sps_triangle_enabled_flag && MaxNumTriangleMergeCand > 1 && slice_type = = B ) ) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( sps_mmvd_enabled_flag ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_triangle_enabled_flag && MaxNumTriangleMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth > 4 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && MaxNumTriangleMergeCand > 1 ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |