| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0531-v2 |
| --- | --- |

| Title: | Non-CE3: Removal of 2×N chroma intra blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

At the last JVET meeting, 1×N and 2×N intra prediction were removed from ISP, while 4×2, 2×4, 2×2 chroma intra blocks were removed from normal intra. This contribution proposes to remove the remaining 2×N chroma intra blocks in VVC to improve the worst-case hardware processing throughput. Test #1 is to just remove 2×8 chroma intra blocks, and Test #2 is to remove all 2×N chroma intra blocks. Test #3 is to remove 2x16 and 2x32 chroma intra blocks. Simulation results under the CTC conditions reportedly show that the overall BD-rate changes for {Y, Cb, Cr} are listed as follows.

| Test #1 (CTC) | AI | 0.00% | 0.16% | 0.23% | 100% | 100% |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA | 0.08% | 0.27% | 0.31% | 100% | 100% |
| Test #2 (CTC) | AI | 0.01% | 0.35% | 0.37% | 100% | 99% |
|  | RA | 0.12% | 0.50% | 0.49% | 100% | 99% |
| Test #3 (CTC) | AI | 0.00% | 0.15% | 0.12% | 100% | 100% |
|  | RA | 0.05% | 0.20% | 0.15% | 100% | 100% |

And the simulation results under AI with dual tree disabled are listed as follows.

| Test #1 | AI | -0.03% | -0.26% | -0.30% | 98% | 100% |
| --- | --- | --- | --- | --- | --- | --- |
| Test #2 | AI | -0.02% | -0.18% | -0.22% | 97% | 100% |
| Test #3 | AI | 0.00% | -0.07% | -0.07% | 98% | 99% |

# Problem statement

The local dual tree [1] was adopted at the last Gothenburg JVET meeting to improve the worst-case hardware processing throughput for single tree. With the local dual tree technique, the 4×2, 2×4, and 2×2 chroma intra blocks are disallowed in single tree by constraining the partitioning of chroma intra blocks.

Meanwhile, the removal of 1×N and 2×N subblocks intra prediction of ISP was adopted at last meeting, in order to remove the reconstruction dependency caused by the sequentially reconstruction of 1×N and 2×N subblocks in ISP.

In VVC WD6, the worst-case hardware processing throughput becomes 2×N chroma intra blocks processing, which is expressed to be worse than 4×4 intra blocks processing. The latency caused by 2×N chroma intra blocks may result in implementation issues.

# Proposed solution

In this contribution, it is proposed to remove 2×N chroma intra blocks to improve the worst-case hardware processing throughput. Three methods are proposed, with test #1 to just remove 2×8 chroma intra blocks, test #2 to remove all 2×N (with N equal to 8, 16, and 32) chroma intra blocks, and test #3 to remove 2x16 and 2x32 chroma intra blocks.

## Test #1: remove 2×8 chroma intra blocks

In this test, only 2×8 chroma intra blocks are removed. Taking 4:2:0 chroma format as an example, test #1 is applied as follows.

- In dual tree, below partitions are disallowed for a coding tree node with treeType equal to DUAL_TREE_CHROMA.

  - 4×8 parent node partitioned by vertical BT

  - 2×16 parent node partitioned by horizontal BT

  - 8×8 parent node partitioned by vertical TT

  - 2×32 parent node partitioned by horizontal TT

- In single tree, the concept of local dual tree is extended by constraining the chroma splitting at the parent node, as listed below, the chroma component of a parent node cannot split but the luma component can be further split.

  - 8×16 parent node partitioned by vertical BT

  - 16×16 parent node partitioned by vertical TT

  - 4×32 parent node partitioned by horizontal BT or horizontal TT

  - 4×64 parent node partitioned by horizontal TT

  - Moreover, the 4×16 CIIP is disallowed to avoid the 2×8 chroma intra prediction caused by CIIP

## Test #2: remove 2×N chroma intra blocks

In this test, all 2×8, 2×16, 2×32 chroma intra blocks are removed. Different from the removal of 2×8 chroma intra blocks, the restrictions are applied by constraining the coding block width. Taking 4:2:0 chroma format as an example, test #2 is applied as follows.

- In dual tree, below partitions are disallowed for a coding tree node with treeType equal to DUAL_TREE_CHROMA.

  - 4×N parent node partitioned by vertical BT

  - 8×N parent node partitioned by vertical TT

- In single tree, the concept of local dual tree is extended by constraining the chroma splitting at the parent node, as listed below, the chroma component of a parent node cannot split but the luma component can be further split.

  - 8×N parent node partitioned by vertical BT

  - 16×N parent node partitioned by vertical TT

  - Moreover, the 4×N CIIP is disallowed to avoid the 2×N chroma intra prediction caused by CIIP

## Test #3: remove 2×16 and 2x32 chroma intra blocks

In this test, 2×16 and 2×32 chroma intra blocks are removed. Taking 4:2:0 chroma format as an example, test #3 is applied as follows.

- In dual tree, below partitions are disallowed for a coding tree node with treeType equal to DUAL_TREE_CHROMA.

  - 4×16 or 4×32 parent node partitioned by vertical BT

  - 2×32 or 2×64 parent node partitioned by horizontal BT

  - 8×16 or 8×32 parent node partitioned by vertical TT

  - 2×64 parent node partitioned by horizontal TT

- In single tree, the concept of local dual tree is extended by constraining the chroma splitting at the parent node, as listed below, the chroma component of a parent node cannot split but the luma component can be further split.

  - 8×32 or 8×64 parent node partitioned by vertical BT

  - 16×32 or 16×64 parent node partitioned by vertical TT

  - 4×64 parent node partitioned by horizontal BT

  - 4×64 parent node partitioned by horizontal TT

  - Moreover, the 4×32 and 4×64 CIIP are disallowed to avoid the 2×16 and 2x32 chroma intra prediction caused by CIIP

# Simulation results

The proposed tests were implemented on top of VTM-6.0 [2] and tested under JVET CTC conditions [3]. Simulation results of Test #1, Test #2 and Test #3 reportedly are provided in Table 1, Table 2 and Table 3, respectively.

Table 1: BD-Rate results for removing 2×8 chroma intra blocks (Test #1)

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.26% | 0.04% | 100% | 100% |
| Class A2 | 0.00% | 0.31% | 0.28% | 98% | 98% |
| Class B | 0.00% | 0.17% | 0.22% | 101% | 100% |
| Class C | 0.02% | 0.19% | 0.44% | 99% | 102% |
| Class E | -0.01% | -0.11% | 0.09% | 101% | 101% |
| Overall | 0.00% | 0.16% | 0.23% | 100% | 100% |
| Class D | 0.03% | 0.62% | 0.59% | 100% | 101% |
| Class F | 0.03% | 0.33% | 0.33% | 100% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.10% | 0.15% | 100% | 100% |
| Class A2 | 0.06% | 0.22% | 0.27% | 100% | 100% |
| Class B | 0.04% | 0.22% | 0.29% | 101% | 100% |
| Class C | 0.16% | 0.49% | 0.50% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.27% | 0.31% | 100% | 100% |
| Class D | 0.18% | 0.64% | 0.69% | 101% | 102% |
| Class F | 0.08% | 0.28% | 0.33% | 101% | 99% |

Table 2: BD-Rate results for removing 2×N chroma intra blocks (Test #2)

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.37% | 0.15% | 100% | 100% |
| Class A2 | 0.02% | 0.61% | 0.55% | 100% | 99% |
| Class B | 0.01% | 0.37% | 0.45% | 100% | 99% |
| Class C | 0.04% | 0.47% | 0.59% | 100% | 101% |
| Class E | -0.02% | -0.16% | -0.02% | 101% | 100% |
| Overall | 0.01% | 0.35% | 0.37% | 100% | 99% |
| Class D | 0.05% | 0.88% | 0.82% | 101% | 101% |
| Class F | 0.03% | 0.45% | 0.50% | 99% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.10% | 0.18% | 101% | 100% |
| Class A2 | 0.15% | 0.65% | 0.66% | 99% | 99% |
| Class B | 0.09% | 0.50% | 0.44% | 100% | 99% |
| Class C | 0.22% | 0.68% | 0.66% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | 0.50% | 0.49% | 100% | 99% |
| Class D | 0.22% | 0.85% | 0.79% | 101% | 101% |
| Class F | 0.19% | 0.53% | 0.69% | 101% | 102% |

Table 3: BD-Rate results for removing 2×16 and 2×32 chroma intra blocks (Test #3)

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.15% | 0.04% | 99% | 101% |
| Class A2 | 0.00% | 0.29% | 0.25% | 99% | 100% |
| Class B | -0.01% | 0.23% | 0.16% | 101% | 101% |
| Class C | 0.00% | 0.08% | 0.18% | 100% | 100% |
| Class E | -0.01% | -0.01% | -0.09% | 100% | 101% |
| Overall | 0.00% | 0.15% | 0.12% | 100% | 100% |
| Class D | 0.01% | 0.20% | 0.13% | 99% | 100% |
| Class F | 0.00% | 0.09% | 0.13% | 100% | 100% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.07% | 0.10% | 99% | 100% |
| Class A2 | 0.09% | 0.35% | 0.30% | 99% | 99% |
| Class B | 0.03% | 0.21% | 0.11% | 100% | 100% |
| Class C | 0.06% | 0.18% | 0.13% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.20% | 0.15% | 100% | 100% |
| Class D | 0.00% | -0.07% | 0.07% | 100% | 100% |
| Class F | 0.03% | 0.10% | 0.12% | 100% | 101% |

The experimental results of Test #1, Test #2 and Test #3 under AI with dual tree disabled are presented in Table 4, Table 5 and Table 6 respectively.

Table 4: BD-Rate results for Test #1 when DualTree off

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0-DualTreeOff |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.01% | -0.05% | 96% | 99% |
| Class A2 | -0.01% | -0.13% | 0.18% | 99% | 101% |
| Class B | -0.02% | -0.08% | -0.32% | 98% | 102% |
| Class C | -0.02% | -0.23% | -0.23% | 98% | 99% |
| Class E | -0.07% | -0.97% | -1.09% | 98% | 100% |
| Overall | -0.03% | -0.26% | -0.30% | 98% | 100% |
| Class D | 0.01% | 0.09% | 0.10% | 98% | 100% |
| Class F | 0.02% | 0.12% | 0.06% | 99% | 100% |

Table 5: BD-Rate results for Test #2 when DualTree off

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0-DualTreeOff |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.13% | -0.12% | 96% | 100% |
| Class A2 | 0.00% | -0.03% | 0.45% | 96% | 100% |
| Class B | -0.03% | 0.10% | -0.08% | 97% | 100% |
| Class C | 0.00% | -0.21% | -0.28% | 97% | 100% |
| Class E | -0.07% | -1.10% | -1.13% | 97% | 99% |
| Overall | -0.02% | -0.18% | -0.22% | 97% | 100% |
| Class D | 0.02% | 0.22% | 0.30% | 96% | 98% |
| Class F | 0.02% | 0.12% | 0.07% | 97% | 101% |

Table 6: BD-Rate results for Test #3 when DualTree off

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0-DualTreeOff |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.09% | -0.03% | 96% | 98% |
| Class A2 | 0.01% | 0.10% | 0.24% | 99% | 101% |
| Class B | -0.01% | 0.04% | -0.14% | 99% | 99% |
| Class C | 0.00% | -0.15% | -0.18% | 98% | 100% |
| Class E | -0.02% | -0.45% | -0.15% | 99% | 99% |
| Overall | 0.00% | -0.07% | -0.07% | 98% | 99% |
| Class D | 0.00% | 0.00% | 0.11% | 98% | 98% |
| Class F | 0.00% | 0.04% | -0.01% | 100% | 101% |

# Conclusions

This contribution proposes to remove 2×N chroma intra blocks for both dual tree and single tree to improve the worst-case hardware processing throughput. Test #1 is to just remove 2×8 chroma intra blocks. Test #2 is to remove 2×N chroma intra blocks. Test #3 is to remove 2×16 and 2×32 chroma intra blocks. The average luma BD-rate of Test #1 is 0.00%/0.08% for AI/RA compared with VTM-6.0. Test #2 has slightly worse coding efficiency than Test #1. Test #3 has slightly better coding efficiency than Test #1. It is recommended adopting Test #2 to the next version of VVC.

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, SE.

VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, CH.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).