| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0536 |
| --- | --- |

| Title: | Non-CE3: MRL with non-MPM intra modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to extend multi-reference line (MRL) intra prediction with non-MPM intra modes. In VVC, only the intra modes in MPM are used for MRL. This contribution proposes to use both MPM and non-MPM intra modes for MRL so that the MRL can generally be applied as other intra coding tools and MPM flag parsing does not need to depend on MRL flag.

Experimental results for proposed method show -0.01% luma BD-rate change with 100% encoding time in RA configuration.

Proposed method

In the current VVC spec and VTM6, MRL can be applied using MPM intra modes only, and this restriction makes the additional parsing condition that the MRL index is always checked in the parsing of MPM flag, as shown in Fig 1.

| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| --- | --- |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |

Figure 1. Parsing condition of MPM flag in the current VVC

In this proposal, the non-MPM intra modes are also used in the MRL to improve the freedom of algorithm choice and the parsing condition of flag, and thus the parsing condition of MPM is simplified as shown in Fig 2.

| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| --- | --- |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |

Figure 2. Parsing condition of MPM flag in the proposed method

With this removal, MRL can be applied in general intra modes with the reduced flag parsing complexity.

Experimental result

The proposed method has been implemented using VTM6.0 as software base according to the common test conditions defined in JVET-N1010 [1]. Table 1 and Table 2 show the test results of proposed method in AI and RA configuration.

Table 1 Experimental results of test1 for all-intra (AI) test condition; anchor is VTM6

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.06% | 0.11% | 101% | 100% |
| Class A2 | 0.00% | 0.03% | -0.01% | 101% | 100% |
| Class B | 0.01% | -0.03% | -0.01% | 101% | 99% |
| Class C | 0.02% | -0.07% | 0.04% | 100% | 100% |
| Class E | -0.02% | 0.07% | 0.06% | 101% | 99% |
| Overall | 0.01% | 0.00% | 0.03% | 101% | 99% |
| Class D | 0.01% | 0.06% | 0.01% | 100% | 100% |
| Class F | -0.04% | -0.02% | -0.08% | 100% | 100% |

Table 2 Experimental results of test1 for random-access (RA) test condition; anchor is VTM6

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.15% | 0.06% | 100% | 100% |
| Class A2 | -0.02% | 0.16% | 0.11% | 100% | 100% |
| Class B | -0.02% | 0.04% | 0.02% | 100% | 101% |
| Class C | -0.01% | 0.07% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.09% | 0.03% | 100% | 100% |
| Class D | -0.05% | -0.42% | -0.34% | 100% | 100% |
| Class F | -0.07% | -0.03% | 0.02% | 100% | 101% |

Conclusion

This contribution proposes to extend the MRL using general intra modes. The proposed method provides the extended freedom of MRL algorithm and the simplified MPM flag parsing condition with negligible encoding time change.

Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th meeting, Geneva, CH, 19-27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).