| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0537 |
| --- | --- |

| Title: | Non-CE3: Cleanups on local dual tree for non-4:2:0 chroma formats |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Jizheng Xu, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

A local dual tree design was adopted at the 15th JVET meeting for the purpose of disabling small chroma intra blocks, i.e. 4×2, 2×4 and 2×2. However, this purpose does not hold for chroma formats other than 4:2:0. It is proposed in this contribution to 1) disable local dual tree in 4:4:4 as already in VTM-6.0; 2) disable local dual tree in 4:0:0 in alignment with 4:4:4; 3) fix local dual tree in 4:2:2 to reflect the purpose of disabling 4×2, 2×4 and 2×2 chroma intra blocks. The proposed cleanups do not influence the CTC case. The case of 4:2:2 was tested with JCT-VC RExt YUV4:2:2 sequences, and the overall BD-rate impact for {Y, Cb, Cr} is {-0.13%, -0.45%, -0.28%} for RA; and { -0.30%, -0.47%, -0.38%} for LDB with similar encoding and decoding time.

# Local dual tree in VVC draft 6

The local dual tree design was adopted into VVC WD6 [1] to solve the hardware latency and memory access concern on small chroma intra blocks such as 4×2, 2×4 and 2×2. However, its support for chroma formats other than 4:2:0 may not be well considered. It is obvious that the intention of removing 4×2, 2×4 and 2×2 chroma intra blocks does not hold for non-4:2:0 chroma formats. A VTM ticket (#456) was submitted to describe the software problem when enabling local dual tree in 4:4:4. Later on, it was asserted that local dual tree has conflicts with some coding tools, e.g. palette and a software fix to disable local dual tree was integrated into VTM-6.0. It shows that local dual tree design was not mature for non-4:2:0 chroma formats and the related issues need to be thoroughly considered.

With the local dual tree design, the size and partition type of each coding tree node are checked to determine a mode type (e.g., MODE_TYPE_INTER, MODE_TYPE_INTRA, or MODE_TYPE_ALL), and there would be no chance to split a MODE_TYPE_INTRA parent node into multiple child nodes with small chroma intra blocks. If local dual tree is applied to a block (e.g., modeTypeCondition is not equal to 0), the prediction of luma and chroma components are affected. For example, MODE_TYPE_INTER allows further split the luma and chroma of a parent node to multiple child nodes but only INTER modes are allowed for child nodes. On the other hand, MODE_TYPE_INTRA allows further split for luma but no split for chroma, meanwhile, only INTRA and IBC modes are allowed for luma, and only INTRA mode is allowed for chroma.

# Discussion and proposed solution

The local dual tree modifies the prediction modes allocation for luma blocks in 4:4:4 and 4:0:0 chroma format (e.g., INTRA and IBC prediction modes are not allowed per MODE_TYPE_INTER), which should be not desirable as the problem of small blocks (2×4, 4×2, 2×2) stated in the local dual tree do not exist in 4:0:0 format.

Moreover, for 4:2:2 chroma format, the current rules introduced by local dual tree unnecessarily disable some large chroma intra blocks, which lowers the coding efficiency. For example, as shown in Figure 1(b), in case of MODE_TYPE_INTRA and 4:2:2 chroma format, given a 16×4 parent node being partitioned by vertical BT, the current WD6 applies non-split 8×4 intra prediction to the associated 8×4 chroma intra block, instead of vertically BT splitting the associated 8x4 chroma intra block into two 4×4 chroma intra blocks. This behavior should be not intended according to the spirit of the local dual tree (i.e., local dual tree is to disable 4×2, 2×4, and 2×2 chroma intra blocks).

- (b)                               (c)

Figure 1. Illustration of chroma intra block partitions in case of MODE_TYPE_INTRA under 4:2:2 chroma format, where (a) the parent node of luma block, (b) associate chroma intra block partition according to VVC WD6, and (c) associate chroma intra block partition by the proposed method.

To solve the above problems, we propose the following cleanups.

## Aspect #1: align spec to the software on local dual tree for 4:4:4

As menitoned above, local dual tree on 4:4:4 has two issues. One is that it does not serve the purpose as it was proposed. The other is that it has conflicts with some 4:4:4 coding tools. In VTM-6.0, local dual tree is disabled as indicated by the VTM ticekt 456 and the MR#852. We suggest to align the spec to the software.

## Aspect #2: bugfix on local dual tree for 4:0:0

Furthermore, for 4:0:0 chroma format, the case is similar to 4:4:4. Thus, we suggest to also disable local dual tree for 4:0:0 chroma format.

## Aspect #3: a cleanup on local dual tree for 4:2:2

For 4:2:2 chroma format, it is proposed that the local dual tree is only applied to coding tree nodes that would be partitioned into 4×2, 2×2, and 2×4 child nodes as what has been done for 4:2:0 case. And for other cases, the local dual tree should never be activated. For example, for blocks with block size and partition type shown in Figure 1(c), the local dual tree is never applied and the chroma block are always split together with the associated luma block in single tree.

# Simulation results

The proposed method for 4:2:2 was implemented on top of VTM-6.0 [2] and tested for JCT-VC RExt CTC test set [3] under JVET CTC conditions [4]. Simulation results for aspect #2 reportedly are provided in Table 1.

Table 1: BD-Rate results for 4:2:2 in RExt test set

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | -0.10% | -0.10% | -0.10% |
| Kimono1_1920x1080_24_10bit_422.yuv | -0.05% | -0.10% | -0.14% |
| EBUHorse_1920x1080_50_10bit_422.yuv | -0.04% | -0.08% | -0.07% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.72% | -1.02% | -0.91% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.09% | -0.78% | -0.03% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | -0.04% | -0.19% | -0.12% |
| Seeking_1920x1080_50_10bit_422.yuv | -0.08% | -0.88% | -0.57% |
| Overall | -0.13% | -0.45% | -0.28% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | -0.01% | 0.06% | -0.15% |
| Kimono1_1920x1080_24_10bit_422.yuv | -0.09% | -0.23% | -0.26% |
| EBUHorse_1920x1080_50_10bit_422.yuv | -0.07% | 0.03% | -0.17% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.94% | -1.19% | -1.18% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | -0.72% | -0.92% | -0.60% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | -0.10% | -0.26% | 0.09% |
| Seeking_1920x1080_50_10bit_422.yuv | -0.18% | -0.76% | -0.37% |
| Overall | -0.30% | -0.47% | -0.38% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |

# Conclusions

In this contribution, three aspects are proposed to cleanup the local dual tree in VVC WD6. The aspects #1 and #2 are to disable the undesirable local dual tree for 4:4:4 and 4:0:0 chroma formats. The aspect #3 is to remove the unnecessary local dual tree constraints for 4:2:2 . The experimental results demonstrate that the proposed method improves the coding efficiency for 4:2:2 (e.g., -0.13% BD-rate changes for RA and -0.30% BD-rate changes for LDB) without complexity increase. It is recommended adopting the proposed cleanups.

# Proposed spec changes

The proposed changes on top of VVC draft 6 [1] are described as below.

#### 7.4.9.4 Coding tree semantics

The variable modeTypeCondition is derived as follows:

- If one of the following conditions is true, modeTypeCondition is set equal to 0

- slice_type = = I  and  qtbtt_dual_tree_intra_flag is equal to 1

- modeTypeCurr is not equal to MODE_TYPE_ALL

- chroma_format_idc is equal to 0

- chroma_format_idc is equal to 3

- Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1

- cbWidth * cbHeight is equal to 64 and split_qt_flag is equal to 1

- cbWidth * cbHeight is equal to 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER

- cbWidth * cbHeight is equal to 32 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER

- Otherwise, if chroma_format_idc is equal to 1 and one of the following conditions is true, modeTypeCondition is set equal to 1 + (slice_type  !=  I ? 1 : 0)

- cbWidth * cbHeight is equal to 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER

- cbWidth * cbHeight is equal to 128 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER

- Otherwise, modeTypeCondition is set equal to 0

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, SE.

VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.

C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006-N1028, January 2014

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, CH.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).