| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0538 |
| --- | --- |

| Title: | Non-CE6: Transform skip with a fixed transform shift |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Weijia Zhu, Kai Zhang, Hongbin Liu, Zhiping Deng 8910 University Center Lane, San Diego, CA 92122, USA. | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the current VVC design, a transform shift is applied to simulate transform process for all blocks even for a transform skip coded block. In this contribution, two methods are proposed. Method 1 is to have a fixed shifting to fit the 16-bit range. Method 2 is to remove such a transform simulation for transform skip blocks. It is asserted that computational complexity reduction is achieved. No BD-rate changes under the CTC are observed. For low QPs (2, 7, 12, 17), some minor BD-rate changes are reported.

# Introduction

In the current design, after dequantization, a left shift with [5+(( Log2(nTbW)+Log2(nTbH))/2)] bits is applied to transform skip blocks, where nTbW and nTbH are transform block width and height respectively. It is to align the energy of transform skip block to that of transformed block with the same size. To compensate such a left bit shifting, during dequantization process, a corresponding right shift is also applied. The block size depedent per-sample operations of right shift during dequantization and left shift during inverse transform introduce unneccessary complexity.

# Proposed method

In this contribution, two methods are tested. Method 1 is to fix the shifting to let dequantization coefficients within the 16-bit range for transform skip blocks. Method 2 is to remove the left shifting during the inverse transform stage and corresponding right shifting in the dequantization process for transform skip blocks. Both methods remove size dependent shifting during dequantization and inverse transform.

Method 2 makes transform skip blocks be true “transform skip”, instead of applying a transform identical to a constant scaling in the current design. With the proposed change of method 2, for a transform skip block, after dequantization, prediction process can be directly applied. It is noted that this also aligns with the potential design of a transquant bypass mode. It is like a transform bypass mode.

The proposed methods simplify the transform process, reduce the complexity in the inverse transform stage. They also simplify the transform shift derivation for transformed blocks, which currently needs to know if a block is of transform skip or not.

# Experimental results

VTM-6.0 and corresponding CTC are used to evaluate the proposed method. It is noted that Method 1 and 2 generate exactly the same results. Table I summarizes the results under CTC and table II reports the results for low QPs, i.e., (2, 7, 12, 17). It is asserted that removing bit shifting as proposed does not bring much BD-rate changes. Detailed results can be found in the accompanying Excel sheets.

Table I. Performance of the proposed method vs. VTM-6.0 under CTC.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |
| 4:2:0 TGM | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 106% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 102% |
| 4:2:0 TGM | 0.00% | 0.00% | 0.00% | 101% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 106% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 98% |
| 4:2:0 TGM | 0.00% | 0.00% | 0.00% | 97% | 96% |

Table II. Performance of the proposed method vs. VTM-6.0 for low QPs.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 101% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 104% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 105% |
| Class E | 0.05% | -0.04% | -0.06% | 105% | 110% |
| Overall | 0.01% | -0.01% | -0.01% | 103% | 104% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 109% |
| Class F | 0.00% | -0.01% | -0.04% | 104% | 103% |
| 4:2:0 TGM | -0.07% | 0.00% | 0.00% | 104% | 103% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.01% | 101% | 102% |
| Class A2 | 0.00% | 0.00% | 0.01% | 98% | 99% |
| Class B | 0.00% | 0.00% | 0.01% | 102% | 100% |
| Class C | -0.01% | -0.03% | 0.00% | 104% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.00% | 102% | 101% |
| Class D | 0.00% | -0.02% | -0.01% | 103% | 100% |
| Class F | -0.05% | -0.19% | -0.23% | 103% | 104% |
| 4:2:0 TGM | -0.04% | -0.03% | -0.02% | 103% | 102% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | -0.02% | 97% | 97% |
| Class C | 0.00% | -0.01% | -0.01% | 103% | 103% |
| Class E | 0.00% | 0.03% | 0.06% | 100% | 97% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.00% | 0.03% | -0.03% | 102% | 109% |
| Class F | -0.04% | -0.01% | -0.06% | 102% | 102% |
| 4:2:0 TGM | -0.06% | -0.07% | -0.01% | 101% | 105% |

# Conclusions

Two methods are proposed to remove size dependent shifting in dequantization and inverse transform for transform skip blocks. Both methods can reduce complexity. Because method 2 is simpler than method 1 without coding efficiency differences, it is recommended adopting method 2 into the next version of VVC draft.

# Changes to the draft text

The changes, marked in grey, are based on JVET-O2001-vE.

## Suggested changes for Method 1

8.7.2 Scaling and transformation process

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable cIdx specifying the colour component of the current block,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height.

Output of this process is the (nTbW)x(nTbH) array of residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables bitDepth, bdShift and tsShift are derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC		(8-942)

bdShift = Max( 20 − bitDepth, 0 )		(8-943)

tsShift = 5			(8-944)

…

8.7.3 Scaling process for transform coefficients

…

The variable rectNonTsFlag is derived as follows:

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 	(8-955)
		transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

The variables  bdShift, rectNorm and bdOffset are derived as follows:

-	If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the following applies:

bdShift = bitDepth - 5

-	Otherwise, the following applies:

bdShift = bitDepth + ( ( rectNonTsFlag ?  1  :  0 ) + 	(8-956)
	( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + dep_quant_enabled_flag

bdOffset = ( 1 << bdShift ) >> 1					(8-957)

The list levelScale[ ][ ] is specified as levelScale[ j ][ k ] = { { 40, 45, 51, 57, 64, 72 }, { 57, 64, 72, 80, 90, 102 } } with j = 0..1, k = 0..5.

The (nTbW)x(nTbH) array dz is set equal to the (nTbW)x(nTbH) array TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is  derived as follows:

–	If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

- sps_scaling_list_enabled_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ] is equal to 1.

–	Otherwise, the following applies:

m[ x ][ y ] = ScalingFactor[ Log2( nTbW ) ][ Log2( nTbH ) ][ matrixId ][ x ][ y ], 
			with matrixId as specified in Table 7-5		(8-958)

- The scaling factor ls[ x ][ y ] is derived as follows:

–	If dep_quant_enabled_flag is equal to 1, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ (qP + 1) % 6 ] ) << ( (qP + 1) / 6 )	(8-959)

–	Otherwise (dep_quant_enabled_flag is equal to 0), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ qP % 6 ] ) << ( qP / 6 )	(8-960)

- When BdpcmFlag[ xTbY ][ yYbY ] is equal to 1, dz[ x ][ y ] is modified as follows:

–	If BdpcmDir[ xTbY ][ yYbY ] is equal to 0 and x is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x − 1 ][ y ] + dz[ x ][ y ])	(8-961)

–	Otherwise, if BdpcmDir[ xTbY ][ yYbY ] is equal to 1 and y is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x ][ y − 1 ] + dz[ x ][ y ])	(8-962)

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = ( dz[ x ][ y ] * ls[ x ][ y ] +bdOffset )  >>  bdShift	(8-963)

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )	(8-964)

## Suggested changes for Method 2

8.7.2 Scaling and transformation process

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable cIdx specifying the colour component of the current block,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height.

Output of this process is the (nTbW)x(nTbH) array of residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables bitDepth, bdShift and tsShift are derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC		(8-942)

bdShift = Max( 20 − bitDepth, 0 )		(8-943)

tsShift = 5 + ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 )		(8-944)

The variable codedCIdx is derived as follows:

- If cIdx is equal to 0 or TuCResMode[ xTbY ][ yTbY ] is equal to 0, codedCIdx is set equal to cIdx.

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 1 or 2, codedCIdx is set equal to 1.

- Otherwise, codedCIdx is set equal to 2.

The variable cSign is set equal to ( 1 − 2 * slice_joint_cbcr_sign_flag ).

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

  - The scaling process for transform coefficients as specified in clause 8.7.3 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx being set equal to codedCIdx and the bit depth of the current colour component bitDepth as inputs, and the output is an (nTbW)x(nTbH) array of scaled transform coefficients d.

  - The (nTbW)x(nTbH) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the residual sample array values r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

r[ x ][ y ] = d[ x ][ y ]  <<  tsShift	(8-945)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ] is equal to 0 or and cIdx is not equal to 0), the transformation process for scaled transform coefficients as specified in clause 8.7.4.1 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the (nTbW)x(nTbH) array of scaled transform coefficients d as inputs, and the output is an (nTbW)x(nTbH) array of residual samples r.

  - The intermediate residual samples res [ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the following applies:

res [ x ][ y ] = d[ x ][ y ]

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ] is equal to 0 or and cIdx is not equal to 0), the following applies:

res [ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-946)

  - The residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If cIdx is equal to codedCIdx, the following applies:

resSamples[ x ][ y ] = res[ x ][ y ]	(8-947)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

resSamples[ x ][ y ] = cSign * res[ x ][ y ]	(8-948)

- Otherwise, the following applies:

resSamples[ x ][ y ] = ( cSign * res[ x ][ y ] ) >> 1	(8-949)

8.7.3 Scaling process for transform coefficients

…

The variable rectNonTsFlag is derived as follows:

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 	(8-955)
		transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

The variables  bdShift, rectNorm and bdOffset are derived as follows:

-	If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the following applies:

bdShift = 10

-	Otherwise, the following applies:

bdShift = bitDepth + ( ( rectNonTsFlag ?  1  :  0 ) + 	(8-956)
	( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + dep_quant_enabled_flag

bdOffset = ( 1 << bdShift ) >> 1					(8-957)

The list levelScale[ ][ ] is specified as levelScale[ j ][ k ] = { { 40, 45, 51, 57, 64, 72 }, { 57, 64, 72, 80, 90, 102 } } with j = 0..1, k = 0..5.

The (nTbW)x(nTbH) array dz is set equal to the (nTbW)x(nTbH) array TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is  derived as follows:

–	If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

- sps_scaling_list_enabled_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ] is equal to 1.

–	Otherwise, the following applies:

m[ x ][ y ] = ScalingFactor[ Log2( nTbW ) ][ Log2( nTbH ) ][ matrixId ][ x ][ y ], 
			with matrixId as specified in Table 7-5		(8-958)

- The scaling factor ls[ x ][ y ] is derived as follows:

–	If dep_quant_enabled_flag is equal to 1, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ (qP + 1) % 6 ] ) << ( (qP + 1) / 6 )	(8-959)

–	Otherwise (dep_quant_enabled_flag is equal to 0), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ qP % 6 ] ) << ( qP / 6 )	(8-960)

- When BdpcmFlag[ xTbY ][ yYbY ] is equal to 1, dz[ x ][ y ] is modified as follows:

–	If BdpcmDir[ xTbY ][ yYbY ] is equal to 0 and x is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x − 1 ][ y ] + dz[ x ][ y ])	(8-961)

–	Otherwise, if BdpcmDir[ xTbY ][ yYbY ] is equal to 1 and y is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x ][ y − 1 ] + dz[ x ][ y ])	(8-962)

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = ( dz[ x ][ y ] * ls[ x ][ y ] +bdOffset )  >>  bdShift	(8-963)

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )	(8-964)

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).