| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 Oct. 2019 | Document: JVET-P0545-v1 |
| --- | --- |

| Title: | CE6-2.3-related: Reduced 8×8 matrices for LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kui Fan, Li Zhang, Kai Zhang, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-265-9942 kui.fan@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

The current LFNST design defines eight 16×16 matrices and eight 16×48 matrices. In this contribution, 16×48 matrices are proposed to be further reduced to 16×36 matrices and only one coefficient pattern is utilized. With the proposed method, it is asserted that the required on-chip memory size of transform matrices is reduced by 1.5 KB and the computational complexity for 8×8 LFNST are reduced by 25%. Experimental results reportedly show that the BD-rate changes are negligible, i.e., 0.03%, 0.02% for AI and RA configurations, respectively. For the low QP settings according to CE6 test conditions, the BD-rate change is reported to be 0.00% for AI configuration.

# Introduction

In the current design, eight 16×16 matrices and eight 16×48 matrices need to be stored for LFNST. The memory costs of matrices in LFNST are summarized in Table 1.

Table 1. Memory costs of matrices in LFNST

| Transform size | Number of matrices | Memory cost (Byte) |
| --- | --- | --- |
| 16×16 | 8 | 2048 |
| 16×48 | 8 | 6144 |
| Total: 8192 |  |  |

In CE6-2.3 [2] [3], the 16×48 LFNST matrices are replaced by 16×36 matrices. The matrices are designed based on the specific coefficient patterns, which is selected according to lfnstIdx.

8 6 6 6 4 4 1 1 6 6 6 6 4 4 4 0 ( lfnstIdx == 0) ( lfnstIdx == 1)

Figure 1. Two coefficient patterns used in CE6.2-3

# Proposed methods

In this contribution, only the second coefficient pattern in Figure 1 is used. The corresponding coefficients in the existing LFNST matrices are kept, and coefficients in other positions are removed from the LFNST matrices directly. No retraining is applied.

The proposed modification reduces the memory cost by 1.5 KB. Meanwhile, computational complexity in terms of multiplication operations is reduced by 25%. When compared to CE6.2-3, this solution is simpler since only one pattern is used. There is no need to allocate on-chip memory to store the two patterns.

# Experimental results

The experiments were carried out under “All Intra”, “Random Access”, and “Low Delay B” configurations as defined in the JVET Common Test Condition (CTC) [1] on top of VTM-6.0. In addition, the experimental results of low QP for AI configuration defined in Description of CE6 [2] are also reported.

Table 4 shows the CTC results of the proposed method. Table II shows the results of Low QP.

Table 2. CTC test results of the proposed method on top of VTM-6.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | 0.12% | 0.12% | 100% | 97% |
| Class A2 | 0.03% | 0.11% | 0.00% | 100% | 114% |
| Class B | 0.03% | 0.06% | -0.02% | 100% | 99% |
| Class C | -0.03% | -0.20% | -0.06% | 99% | 97% |
| Class E | 0.02% | 0.08% | 0.06% | 99% | 97% |
| Overall | 0.03% | 0.02% | 0.01% | 99% | 100% |
| Class D | 0.02% | 0.01% | -0.05% | 99% | 97% |
| Class F |  |  |  |  |  |
|  | Random access Main10 |  |  |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.08% | 0.01% | 101% | 100% |
| Class A2 | 0.03% | 0.13% | 0.07% | 100% | 100% |
| Class B | -0.01% | 0.03% | -0.01% | 99% | 99% |
| Class C | 0.04% | 0.04% | -0.01% | 100% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | 0.01% | 100% | 98% |
| Class D | -0.03% | -0.13% | 0.04% | 99% | 96% |
| Class F |  |  |  |  |  |

Table 3. Low QP test results of the proposed method on top of VTM-6.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over TM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.01% | 102% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.01% | 0.00% | 0.01% | 100% | 104% |
| Class E | 0.01% | -0.03% | -0.03% | 101% | 99% |
| Overall | 0.00% | -0.01% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.02% | 101% | 107% |
| Class F |  |  |  |  |  |

# Conclusion

In this contribution, we propose to reduce the size of 16×48 LFNST matrices to 16×36 with one fixed pattern. The proposed method reduces the memory cost of matrices for LFNST by 1.5KB with marginal performance loss in the AI, RA, and LDB configurations. It is recommended to adopt this method in the next VVC WD.

# Reference

[1]	F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

[2] H. Egilmez, M. Salehifar, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling,” JVET-O2026, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 15th Meeting: Gothenburg, SE, 3–12 July 2019.

[3] T. Zhou, T. Hashimoto, T. Ikai, “CE6-2.3a: Simplification with new LFNST transform basis,” JVET-P0065, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th Meeting: Geneva, CH, 2–12 Oct. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).