| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0546 |
| --- | --- |

| Title: | Non-CE4: An applying condition of BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

In this contribution, it is pointed out that there is an inconsistency between an applying condition and processes of the current Bi-Directional Optical Flow (BDOF). To solve this inconsistency, it is proposed that the applying condition is changed to both POC distances for L0 and L1 must be equal. Experimental results show that BD-rate for luma is 0.01% (RA) in comparison with VTM-6.0.

Introduction

In the VVC D6 [1], whether Bi-Directional Optical Flow (BDOF) is applied or not is determined by checking some applying conditions at decoder-side for each CU. One of the conditions is “DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0”. This condition means that BDOF may be applied when one of reference pictures is in past and the other reference picture is in future regardless of POC distance are different between L0 and L1. On the other hand, predicted sample values from L0 and L1 are calculated without any weighting coefficients considering POC distance in the current BDOF process as follows.

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[ x ][ y ]  =  ( predSamplesL0[ hx + 1 ][vy] >> shift1 ) − 	(8-803)
	( predSampleL0[ hx − 1 ][ vy ] ) >> shift1 )

gradientVL0[ x ][ y ]  =  ( predSampleL0[ hx ][ vy + 1 ] >> shift1 ) − 	(8-804)
	( predSampleL0[ hx ][vy − 1 ] ) >> shift1 )

gradientHL1[ x ][ y ]  =  ( predSamplesL1[ hx + 1 ][vy] >> shift1 ) − 	(8-805)
	( predSampleL1[ hx − 1 ][ vy ] ) >> shift1 )

gradientVL1[ x ][ y ]  =   ( predSampleL1[ hx ][ vy + 1 ] >> shift1 ) − 	(8-806)
	( predSampleL1[ hx ][vy − 1 ] ) >> shift1 )

- The variables diff[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows:

diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> shift2 ) − ( predSamplesL1[ hx ][ vy ] >> shift2 )	(8-807)

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> shift3	(8-808)

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> shift3	(8-809)

Therefore, it can be considered that the current BDOF process assumes POC distances for L0 and L1 are equal. Decoder-side Motion Vector Refinement (DMVR) also has processes that predicted sample values from L0 and L1 are calculated without any weighting coefficients. And one of the DMVR applying conditions considers POC distance as follows: “DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )”.

Proposed Method

To solve an inconsistency between the applying condition and the processes of BDOF, it is proposed that changing the applying condition as follows.

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - sps_bdof_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flag is equal to 0.

      - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

      - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

      - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

      - sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

      - ciip_flag[ xCb ][ yCb ] is equal to 0.

      - BcwIdx[ xCb ][ yCb ] is equal to 0.

      - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

      - cbWidth is greater than or equal to 8.

      - cbHeight is greater than or equal to 8.

      - cbHeight * cbWidth is greater than or equal to 128.

      - For X being each of 0 and 1, the pic_width_in_luma_samples and pic_height_in_luma_samples of the reference picture refPicLX associated with the refIdxLX are equal to the pic_width_in_luma_samples and pic_height_in_luma_samples of the current picture, respectively.

      - cIdx is equal to 0.

    - Otherwise, bdofFlag is set equal to FALSE.

Experimental Results

The proposed method is implemented on top of VTM-6.0. In order to verify the performance of the proposed method, we have run simulations according to the common test conditions of JVET-N1010 [2].

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Test results

Table 1 show the experimental results. BD-rate for luma is 0.01% (RA) in comparison with VTM-6.0. The BD-rate loss is negligible.

Table 1 Experimental results for RA condition.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | 0.04% | 100% | 100% |
| Class A2 | 0.02% | 0.00% | 0.01% | 100% | 100% |
| Class B | 0.01% | 0.02% | -0.01% | 100% | 100% |
| Class C | 0.00% | 0.05% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.02% | 100% | 100% |
| Class D | 0.05% | 0.00% | 0.04% | 100% | 100% |
| Class F | 0.00% | 0.01% | 0.01% | 100% | 100% |

Conclusion

In this contribution, it was pointed out that there is an inconsistency between an applying condition and processes of the current Bi-Directional Optical Flow (BDOF). To solve this inconsistency, it was proposed that the applying condition was changed to both POC distances for L0 and L1 had to be equal. Experimental results showed that BD-rate for luma is 0.01% (RA) in comparison with VTM-6.0. The BD-rate loss was negligible.

References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Gothenburg, SE, July 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).