| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0548-v2 |
| --- | --- |

| Title: | Non-CE3 : Removal of MRL restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Hyeongmun Jang Jangwon Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {jin78.heo, hm.jang, jangwon84.choi, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution removes the MRL restriction at the CTU boundary, and neighboring block availability is simply set to false. The proposed method provides the coding performance -0.01% and 0.00% BD-rate in AI and RA configuration, respectively without increasing the complexity of encoding and decoding. The removal of the MRL restriction simplifies the MRL syntax signaling and harmonizes it with intra coding process.

Introduction

In VTM-6.0, the multiple reference line (MRL) [1] coding mode is restricted at the CTU boundary. This contribution proposes the following changes to unify the intra coding process and to simplify the MRL syntax signaling.

- Test 1

  - Remove the MRL restriction at the CTU boundary so that MRL is always enabled. The MRL index is signaled without confirmation of CTU boundary.

  - When MRL coding mode is applied at the CTU boundary, the availability for all samples within above-left except for the sample with x = −1 – refIdx and y = −1, above, and above-right block is always set to false. Where, the variable refIdx is the intra prediction reference line index.

- Test 2

  - Remove the MRL restriction at the CTU boundary so that MRL is always enabled. The MRL index is signaled without confirmation of CTU boundary.

  - When MRL coding mode is applied at the CTU boundary, the availability for all samples within above-left, above, and above-right block is always set to false.

Experimental result

The proposed method has been implemented using VTM-6.0 and under the common test conditions [2].

- Test 1:

  - Signaling MRL index without CTU boundary checking.

  - Availability for all samples within above-left except for (−1 – refIdx, −1), above, and above-right block is set to false for MRL at the CTU boundary.

Table 1. Coding performance of the proposed method (AI)

Table 2. Coding performance of the proposed method (RA)

- Test 2:

  - Signaling MRL index without CTU boundary checking.

  - Availability for all samples within above-left, above, and above-right block is set to false for MRL at the CTU boundary.

Table 3. Coding performance of the proposed method (AI)

Table 4. Coding performance of the proposed method (RA)

Conclusion

In conclusion, the proposed method provides the coding performance -0.01% and 0.00% BD-rate in AI and RA configuration, respectively without increasing encoding and decoding time. It is recommended for the proposed change to be adopted as a part of VVC.

Reference

- B. Bross, et al., “CE3: Multiple reference line intra prediction,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0283, 12th Meeting, Macao, CN, 3–12 Oct. 2018.

- F. Bossen, el al., “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).