| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0548 |
| --- | --- |

| Title: | Non-CE3 : Removal of MRL restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Hyeongmun Jang Jangwon Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {jin78.heo, hm.jang, jangwon84.choi, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Working Draft Text

The changes in the VVC working draft is highlight below.

      - Coding unit syntax

| else { |  |
| --- | --- |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |

    - Derivation process for neighbouring block availability

Inputs to this process are:

- the luma location ( xCurr, yCurr ) of the top-left sample of the current block relative to the top-left luma sample of the current picture,

- the luma location ( xNbY, yNbY ) covered by a neighbouring block relative to the top-left luma sample of the current picture,

- the variable checkPredModeY specifying whether availability depends on the prediction mode.

- the variable cIdx specifying the colour component of the current block.

- the variable refIdx specifying the intra prediction reference line index.

Output of this process is the availability of the neighbouring block covering the location ( xNbY, yNbY ), denoted as availableN.

The variable bCTUforMRL is derived as follows:

bCTUforMRL = ( ( yCurr & ( 1 << ( CtbLog2SizeY − 1 ) − 1 ) = = 0 ) && refIdx ) ? TRUE : FALSE.

The neighbouring block availability availableN is derived as follows:

–	If one or more of the following conditions are true, availableN is set equal to FALSE:

–	xNbY is less than 0.

–	yNbY is less than 0.

–	xNbY is greater than or equal to pic_width_in_luma_samples.

–	yNbY is greater than or equal to pic_height_in_luma_samples.

–	IsAvailable[ cIdx ][ yNbY ][ xNbY ] is equal to FALSE.

–	The neighbouring block is contained in a different brick than the current block, availableN is set equal to FALSE. [Ed. (YK): The case outside of a slice boundary is covered by this sentence as each slice always contains an integer number of complete bricks.]

–	bCTUforMRL is equal to TRUE and yNbY is less than yCurr.

–	Otherwise, availableN is set equal to TRUE.

When all of the following conditions are true, availableN is set equal to FALSE.

–	checkPredModeY is equal to TRUE.

–	availableN is set equal to TRUE.

–	CuPredMode[ 0][ xNbY ][ yNbY ] is not equal to CuPredMode[ 0][ xCurr ][ yCurr ].