| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0554 |
| --- | --- |

| Title: | Non-CE5: Clean up of coefficient coding of adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VVC draft 6, the absolute value of a chroma adaptive loop filter coefficient is constrained to be in the range of 0 to 27 − 1, inclusive. While a chroma filter coefficient is constrained to be in the range of −27 − 1 to 27 − 1, inclusive. Following these two constraints, the chroma coefficients cannot be equal to −27, however this value is whithin 8 bits, and secondly −27 – 1 is outside of 8 bits range. This contribution proposes some options to clean up the conflict.

# Introduction

In VVC draft 6, the absolute value of a chroma adaptive loop filter coefficient is constrained to be in the range of 0 to 27 − 1, inclusive. While a chroma filter coefficient is constrained to be in the range of −27 − 1 to 27 − 1, inclusive. There is a conflict in the above constrains.

However, for luma coefficients there is no constrain for absolute coeffcient value and only a contstriant expressed on a final value to be within 8 bits as follows:

It is a requirement of bitstream conformance that the values of AlfCoeffL[ adaptation_parameter_set_id ][ filtIdx ][ j ] with filtIdx = 0..NumAlfFilters − 1, j = 0..11 shall be in the range of −27 to 27 − 1, inclusive.

# Proposed method

## Method 1

Constrain luma absolute coefficient value to be in the same range as chroma absolute coefficient value and remove constraints on the final (after adding the sign) coefficient values.

alf_luma_coeff_abs[ sfIdx ][ j ] specifies the absolute value of the j-th coefficient of the signalled luma filter indicated by sfIdx. When alf_luma_coeff_abs[ sfIdx ][ j ] is not present, it is inferred to be equal 0. It is a requirement of bitstream conformance that the values of alf_luma_coeff_abs[ sfIdx ][ j ] shall be in the range of 0 to 27 − 1, inclusive.

It is a requirement of bitstream conformance that the values of AlfCoeffL[ adaptation_parameter_set_id ][ filtIdx ][ j ] with filtIdx = 0..NumAlfFilters − 1, j = 0..11 shall be in the range of −27 to 27 − 1, inclusive.

It is a requirement of bitstream conformance that the values of AlfCoeffC[ adaptation_parameter_set_id ][ altIdx ][ j ] with altIdx = 0..alf_chroma_num_alt_filters_minus1, j = 0..5 shall be in the range of −27 − 1 to 27 − 1, inclusive.

## Method 2

Constrain chroma coefficient values to be in the same range as luma coefficient values and remove constraint on the chroma absolute coefficient values.

alf_chroma_coeff_abs[ altIdx ][ j ] specifies the absolute value of the j-th chroma filter coefficient for the alternative chroma filter with index altIdx. When alf_chroma_coeff_abs[ altIdx ][ j ] is not present, it is inferred to be equal 0. It is a requirement of bitstream conformance that the values of alf_chroma_coeff_abs[ altIdx ][ j ] shall be in the range of 0 to 27 − 1, inclusive.

It is a requirement of bitstream conformance that the values of AlfCoeffC[ adaptation_parameter_set_id ][ altIdx ][ j ] with altIdx = 0..alf_chroma_num_alt_filters_minus1, j = 0..5 shall be in the range of −27  to 27 − 1, inclusive.

## Method 3

Constrain chroma coefficient values to be in the same range as luma coefficient values and remove constraint on the chroma absolute coefficient values.

If the absolute coefficient value is equal to 27, the only possibility is to be negative, so when the alf_luma_coeff_abs[ sfIdx ][ j ] or alf_chroma_coeff_abs[ altIdx ][ j ] is 128, there is no need to signal the sign and the coefficient is inferred to be negative.

| if( alf_luma_coeff_abs[ sfIdx ][ j ] && alf_luma_coeff_abs[ sfIdx ][ j ] != 128 ) |  |
| --- | --- |
| alf_luma_coeff_sign[ sfIdx ][ j ] | u(1) |

| if( alf_chroma_coeff_abs[ altIdx ][ j ] && alf_chroma_coeff_abs[ altIdx ][ j ] != 128 ) |  |
| --- | --- |
| alf_chroma_coeff_sign[ altIdx ][ j ] | u(1) |

When alf_luma_coeff_sign[ sfIdx ][ j ] is not present, it is inferred to be equal to 01.

alf_chroma_coeff_abs[ altIdx ][ j ] specifies the absolute value of the j-th chroma filter coefficient for the alternative chroma filter with index altIdx. When alf_chroma_coeff_abs[ altIdx ][ j ] is not present, it is inferred to be equal 0. It is a requirement of bitstream conformance that the values of alf_chroma_coeff_abs[ altIdx ][ j ] shall be in the range of 0 to 27 − 1, inclusive.

When alf_chroma_coeff_sign[ altIdx ][ j ] is not present, it is inferred to be equal to 01.

It is a requirement of bitstream conformance that the values of AlfCoeffC[ adaptation_parameter_set_id ][ altIdx ][ j ] with altIdx = 0..alf_chroma_num_alt_filters_minus1, j = 0..5 shall be in the range of −27  to 27 − 1, inclusive.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).