| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0556 |
| --- | --- |

| Title: | CE5-related: Temporal buffer removal for cross component adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Jie Dong, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In JVET-P0080, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma samples. Adaptation parameter sets are used to carry signalled filter coefficients. Additionally, CC-ALF coefficients used for each chroma component of a slice are also stored in a buffer corresponding to a temporal sublayer for future re-use. In this contribution, the performance of the reusing coefficients from this extra temporal buffer is tested. When not using the temporal buffer, compared to CE5-2.1 under common test conditions, no noticeable performance difference is observed.

# Introduction

In JVET-P0080, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma sample values. Filter coefficients are signalled and stored in adaptation parameter sets (APS). In addition, CC-ALF coefficients used for each chroma component of a slice are also stored in a buffer corresponding to a temporal sublayer for future re-use. For each chroma component of a slice, when CC-ALF is enabled, a re-use flag is signalled to indicate whether coefficients from the corresponding temporal buffer are used. If coefficients in temporal buffers are not re-used, an APS id will be signalled. The temporal buffer increases the storage overhead in both encoder and decoder.

# Proposed method

In this proposal, the temporal buffer is not used in CC-ALF. In another word, a slice can only use CC-ALF coefficients in APSs, which is consistent with the design of adaptive loop filter (ALF).

# Experimental results

The proposed method is implemented on top of the in-loop version of CC-ALF in CE5-2.1. The simulations were conducted under JVET common test conditions [1].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.00% | 99% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.01% | 99% | 99% |
| Class E | 0.00% | 0.00% | 0.01% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.05% | 0.02% | 101% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | 0.02% | 100% | 100% |
| Class A2 | -0.01% | 0.12% | 0.04% | 99% | 100% |
| Class B | 0.00% | 0.06% | 0.12% | 99% | 101% |
| Class C | 0.01% | 0.00% | 0.05% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.05% | 0.07% | 99% | 100% |
| Class D | 0.03% | 0.18% | 0.26% | 100% | 99% |
| Class F | 0.01% | 0.02% | 0.02% | 98% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.06% | 0.04% | 101% | 100% |
| Class C | -0.01% | -0.09% | -0.23% | 102% | 100% |
| Class E | -0.11% | -0.54% | 0.29% | 101% | 102% |
| Overall | -0.03% | -0.14% | 0.01% | 101% | 100% |
| Class D | -0.01% | 0.68% | 0.13% | 99% | 100% |
| Class F | 0.06% | 0.19% | 0.86% | 101% | 101% |

# Reference

[1] JVET-N1010, “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).