| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0565 |
| --- | --- |

| Title: | CE8-related: Cross-component residual prediction for 4:4:4 format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 80 5066 9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

Residual modification process for transform blocks using cross-component prediction was adopted in the HEVC range extensions. This tool modifies the chrome residual signals by adding the scaled luma residual signals. In this contribution, a similar approach is proposed to improve the coding efficiency for 4:4:4 sequences. In VVC, luma and chroma coding trees is different in intra slice when chroma separate tree is enabled. An enabled condition of the modification is proposed to conform the VVC coding tree structure. Simulation results for 4:4:4 sequences show that -0.88% / -1.03% / -0.93% for AI/RA/LDB in single tree case and -0.51% / -0.74% for RA/LDB in chroma separate tree case.

Introduction

Cross-component prediction refines the chroma residual signals by addition the scaled luma residual signals, which was adopted in HEVC development [1]. Chroma residual signal is modified as following equation;

r[ x ][ y ]  +=  ( ResScaleVal[ cIdx ][ xTbY ][ yTbY ] *
			( ( rY[ x ][ y ]  <<  BitDepthC )  >>  BitDepthY ) )  >>  3,

where r, rY, and ResScaleVal are chroma residual signals, luma residual signals, and scaling value, respectively, in a transform block. This method requires that the luma and chroma transform sizes are identical due to the simple hardware implementation.

In VVC, chroma separate tree introduce the difference between luma and chroma transform size. Intra sub-partition also introduce the similar situation. This situation affects the CCRP enabled conditions while CCRP approach with syntax can be reused for VVC.

In the remaining part, we focus only for the CCRP enabled condition for VVC while we reuse the existing cross-component prediction syntax, semantics, and decoding process in HEVC.

Proposed method

Cross-component residual prediction (CCRP) is allowed only for the identical transform size between luma and chroma. It means CCRP is enabled when single tree case and no-intra sub partition, which are newly introduced into VVC compared with HEVC. CCRP can apply the case when luma and chroma transform size is the same even though the dual tree is enabled. Since the enabled condition check becomes complicated, such relaxation is not introduced in this contribution.

The CCRP method is located at after the joint chroma coding (JCC) and LMCS. When JCC and/or LMCS is utilized, CCRP is applied after chroma residual signal of each chroma component is decoded with LMCS. When JCC and/or LMCS is not utilized, CCRP is applied after inverse transform of chroma coefficients. The several possible locations of CCRP were preliminary examined like before JCC and LMCS. While similar gains were observed, the number of context coded bins or some complexity issues are identified.

For the fast encoding technique, a scaling value is derived from luma and chroma residual signals before transform and quantization. To maintain the coding performance, rate-distortion costs for both CCRP enabled and disabled cases are compared.

Simulation results

The proposed method is implemented on the VTM-6.0. A test condition follows the JVET CE8 test condition [2]. The summary of BD-rate is shown in the following table. The first and second test set are single tree case in Table 1 and chroma separate tree case in Table 2, respectively.

Encoding run-time for all intra with single tree case is slightly increased to 101% while that for other cases remains 100% because of encoder optimization.

Table 1 Simulation results of the all intra, random access, and low-delay B conditions 
by the proposed CCRP method for single tree case.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.69% | -2.36% | -2.09% | 101% | 101% |
| TGM 720p | -0.67% | -1.41% | -1.13% | 101% | 100% |
| Animation | -0.24% | -2.20% | -1.23% | 101% | 99% |
| Mixed content | -1.34% | -3.33% | -2.86% | 101% | 98% |
| Camera-Captured content | 0.09% | -1.49% | -1.39% | 101% | 99% |
| Overall | -0.88% | -2.17% | -1.74% | 101% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2.02% | -3.19% | -2.74% | 100% | 100% |
| TGM 720p | -0.98% | -1.97% | -1.74% | 100% | 100% |
| Animation | -0.65% | -4.49% | -2.48% | 100% | 99% |
| Mixed content | -1.22% | -3.66% | -3.31% | 100% | 100% |
| Camera-Captured content | 0.59% | -5.29% | -4.56% | 99% | 99% |
| Overall | -1.03% | -3.48% | -2.78% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.88% | -3.00% | -2.69% | 100% | 100% |
| TGM 720p | -0.88% | -2.60% | -1.77% | 100% | 99% |
| Animation | -0.74% | -5.25% | -2.17% | 101% | 98% |
| Mixed content | -0.71% | -3.80% | -3.11% | 100% | 100% |
| Camera-Captured content | 0.29% | -4.38% | -10.44% | 100% | 99% |
| Overall | -0.93% | -3.64% | -3.41% | 100% | 99% |

Table 1 Simulation results of the random access and low-delay B conditions 
by the proposed CCRP method for dual tree case.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.25% | -2.23% | -2.06% | 100% | 100% |
| TGM 720p | -0.55% | -1.01% | -0.99% | 100% | 100% |
| Animation | -0.53% | -3.33% | -1.83% | 100% | 99% |
| Mixed content | -0.17% | -0.83% | -0.76% | 100% | 100% |
| Camera-Captured content | 0.57% | -4.31% | -3.60% | 100% | 100% |
| Overall | -0.51% | -2.13% | -1.70% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.89% | -2.88% | -2.70% | 100% | 99% |
| TGM 720p | -0.50% | -2.36% | -1.65% | 100% | 100% |
| Animation | -0.50% | -4.71% | -2.44% | 100% | 99% |
| Mixed content | -0.43% | -2.54% | -1.57% | 100% | 100% |
| Camera-Captured content | 0.31% | -4.35% | -10.31% | 100% | 99% |
| Overall | -0.74% | -3.21% | -3.13% | 100% | 100% |

Conclusion

In this contribution, we proposed cross-component residual prediction (CCRP) for 4:4:4 format. The CCRP is enabled only the single tree case with no-intra sub partition. We also propose the fast computation method to derive the scaling value as an encoder optimization. The simulation results show the improvement of both luma and chroma coding performance without sacrificing the encoding/decoding run-time. We recommend to adopt the proposal to the next working draft.

References

- W. Pu, W.-S. Kim, J. Chen, J. Sole, M. Karczewicz, “JCTVC-O0202 RCE1: Descriptions and Results for Experiments 1, 2, 3, and 4”, JCTVC-O0202, Geneva, CH, 23 Oct. – 1 Nov. 2013.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools,” JVET-O2028, Gothenburg, SE, 3–12 July 2019.

Patent rights declaration

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).