| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0566-v2 |
| --- | --- |

| Title: | CE6-related: On LFNST Support Patterns |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Amir Said, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes two alternative designs for LFNST. The first design aims to simplify the existing 8x8 LFNST design by replacing the L-shaped, 48-sample support pattern with the 6x6-square pattern. It is experimentally shown that the proposed simplification with optimized transform matrices provides -0.01%AI and 0.00% RA over VTM-6.0. The second design aims to provide additional coding gains by introducing a circular 48-sample support pattern for 8x8 LFNST. The experimental results show that -0.09% AI and -0.04% RA BD-rates are achieved over VTM-6.0.

# Introduction

In VTM-6.0, 8x8 LFNST is defined by a collection of 16x48 transform matrices, and it produces at most 16 transform coefficients by transforming 48 DCT-2 coefficients. Specifically, the L-shaped pattern with 48 samples in Fig. 1 is used.

Fig 1: L-shaped pattern with 48 samples used in current VTM for 8x8 LFNST

# Proposed Method

In this contribution, two alternative transform support patterns are proposed for 8x8 LFNST.

- Method 1: 6x6-block pattern illustrated in Fig. 2 is proposed.

- Method 2: Circular pattern with 48-samples depicted in Fig. 3 is proposed.

Fig 2: 6x6 block pattern for 8x8 LFNST corresponding to the shaded area

Fig 3: Circular pattern with 48 samples for 8x8 LFNST corresponding to the shaded area

The main benefit of Method 1 over existing LFNST is that it reduces the number of multiplications as well as memory used to store transform matrices for 8x8 LFNST. The memory requirements and number of multiplications of Method 1 are identical to the method tested in CE6-2.3a (JVET-P0065), where patterns with 36 samples are proposed. Method 1 further simplifies CE6-2.3a by using a single 6x6-square block, while CE6-2.3a uses two irregular (non-square) separate patterns in Fig.4 chosen depending on the LFNST index.

Fig. 4: Two patterns with 36 samples for 8x8 LFNST proposed in JVET-P0065 (CE6-2.3a).

Method 2 provides additional coding gains over VTM-6.0 without increasing the memory requirements and number of multiplications.

As part of the proposed methods, the transform matrices for both 4x4 and 8x8 LFNST are optimized. The set of optimized matrices are attached to this submission:

- “8x8_lfnst_6x6_square.h” includes matrices for the 6x6-block pattern,

- “8x8_lfnst_circular.h” includes matrices for the circular block pattern with 48 samples, and

- “4x4_lfnst.h” includes matrices for 4x4 LFNST.

In the following section, the experimental results of (i) proposed methods and (ii) proposed methods without optimized 4x4 LFNSTs are presented.

The draft text changes for Method 1 and 2 are reflected in documents “JVET-P0566_WD_Method1.docx” and “JVET-P0566_WD_Method2.docx”, respectively.

# Experimental Results

The proposed methods are implemented on top of VTM-6.0 and evaluated under common test conditions (CTC). Note that low-delay B results are omitted, since LFNST is disabled in CTC for low-delay B.

Table 1.  Results of Method 1 over VTM-6.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.22% | 0.37% | 100% | 100% |
| Class A2 | 0.00% | 0.14% | 0.08% | 101% | 101% |
| Class B | -0.01% | 0.12% | 0.11% | 100% | 100% |
| Class C | -0.07% | -0.11% | 0.01% | 101% | 102% |
| Class E | -0.05% | 0.10% | 0.08% | 101% | 101% |
| Overall | -0.01% | 0.09% | 0.12% | 101% | 101% |
| Class D | -0.04% | 0.10% | 0.03% | 100% | 100% |
| Class F | -0.09% | 0.04% | 0.02% | 100% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.11% | 0.18% | 100% | 100% |
| Class A2 | 0.04% | 0.13% | 0.03% | 101% | 102% |
| Class B | -0.02% | 0.07% | -0.02% | 100% | 100% |
| Class C | -0.04% | -0.01% | 0.13% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.07% | 0.07% | 101% | 101% |
| Class D | -0.07% | -0.22% | -0.05% | 100% | 100% |
| Class F | 0.01% | 0.00% | -0.02% | 100% | 100% |

Table 2.  Results of Method 2 over VTM-6.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.13% | 0.18% | 100% | 100% |
| Class A2 | -0.06% | 0.10% | 0.00% | 101% | 101% |
| Class B | -0.07% | 0.12% | 0.10% | 100% | 99% |
| Class C | -0.12% | -0.09% | 0.04% | 101% | 105% |
| Class E | -0.12% | 0.02% | 0.01% | 101% | 101% |
| Overall | -0.09% | 0.06% | 0.07% | 101% | 101% |
| Class D | -0.09% | -0.01% | -0.08% | 101% | 100% |
| Class F | -0.09% | -0.06% | -0.14% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.08% | 0.13% | 100% | 100% |
| Class A2 | 0.00% | 0.09% | 0.06% | 101% | 102% |
| Class B | -0.05% | 0.06% | 0.12% | 100% | 101% |
| Class C | -0.06% | 0.02% | 0.12% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.06% | 0.11% | 101% | 101% |
| Class D | -0.06% | 0.00% | -0.14% | 101% | 101% |
| Class F | -0.09% | -0.16% | 0.04% | 100% | 100% |

Table 3.  Results of Method 1 without optimized 4x4 LFNST matrices over VTM-6.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.28% | 0.31% | 100% | 103% |
| Class A2 | 0.02% | 0.02% | 0.02% | 101% | 100% |
| Class B | 0.02% | 0.13% | 0.12% | 100% | 99% |
| Class C | 0.01% | -0.04% | 0.10% | 100% | 101% |
| Class E | 0.02% | 0.15% | 0.08% | 101% | 100% |
| Overall | 0.03% | 0.10% | 0.12% | 100% | 100% |
| Class D | 0.00% | 0.10% | -0.02% | 100% | 99% |
| Class F | 0.01% | -0.04% | -0.10% | 100% | 101% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.22% | 0.27% | 101% | 100% |
| Class A2 | 0.03% | 0.17% | 0.17% | 101% | 101% |
| Class B | -0.01% | 0.15% | -0.02% | 100% | 100% |
| Class C | 0.02% | 0.06% | -0.05% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.14% | 0.07% | 101% | 101% |
| Class D | -0.03% | -0.23% | 0.09% | 100% | 99% |
| Class F | 0.00% | -0.08% | 0.07% | 100% | 100% |

Table 4.  Results of Method 2 without optimized 4x4 LFNST matrices over VTM-6.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.05% | 0.21% | 100% | 100% |
| Class A2 | -0.03% | 0.07% | -0.05% | 100% | 100% |
| Class B | -0.05% | 0.08% | 0.07% | 100% | 100% |
| Class C | -0.03% | -0.09% | 0.01% | 101% | 103% |
| Class E | -0.05% | 0.10% | 0.09% | 101% | 98% |
| Overall | -0.05% | 0.04% | 0.06% | 100% | 100% |
| Class D | -0.02% | 0.09% | -0.10% | 100% | 100% |
| Class F | -0.04% | -0.10% | -0.11% | 100% | 98% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.07% | 0.12% | 100% | 100% |
| Class A2 | 0.02% | 0.07% | -0.01% | 100% | 100% |
| Class B | -0.03% | 0.12% | 0.05% | 100% | 101% |
| Class C | -0.01% | 0.08% | 0.09% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.09% | 0.06% | 100% | 100% |
| Class D | -0.04% | -0.27% | 0.03% | 100% | 100% |
| Class F | -0.01% | -0.06% | 0.06% | 100% | 100% |

# Conclusions

This document proposes two alternatives to LFNST. Method 1 simplifies the existing LFNST design with negligible BD-rate differences, and Method 2 provides about 0.1% AI and 0.05% RA gains over VTM-6.0 without introducing any additional memory requirements and multiplications. It is recommended to adopt Method 1 or Method 2 in VVC.

# References

- F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

- T. Zhou, T. Hashimoto, T. Ikai, “CE6-2.3a: Simplification with new LFNST transform basis”, JVET-P0065, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th Meeting, Geneva, CH, 1–11 Oct. 2019.

- A. Said and X. Zhao, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-L1026, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th meeting, Macao, CN, Oct. 2018.

- A. Said, X. Zhao, “CE6: Summary Report on Transforms and Transform Signaling”, JVET-M0026, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th meeting, Marrakech, MA, 19-27 March 2019.

- X. Zhao, H. E. Egilmez, “CE6: Summary Report on Transforms and Transform Signaling”, JVET-N0026, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th meeting, Geneva, CH, 19-27 March 2019.

- X. Zhao, H. E. Egilmez, “CE6: Summary Report on Transforms and Transform Signaling”, JVET-O0026, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 15th meeting, Gothenburg, SE, 3-12 July 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).