| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0570 |
| --- | --- |

| Title: | AHG10/Non-CE5: Performance of encoder-side deblocking optimization in VTM-6.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The encoder-only deblocking optimization is tested on top of VTM-6.0. The average Y/U/V BD-rates and encoding time for the common test conditions for the filter on top of VTM-6.0 are reported to be −0.77%/−1.53%/−1.56%/105% for all intra, −0.92%/−1.94%/−1.93%/103% for random access and −0.78%/−0.90%/−0.88%/102%  for low delay B.

# Introduction

In VTM-6.0, deblocking filter can be applied in the encoder-side rate distortion optimization (RDO) as an option, such that the coding efficiency affected by deblocking filter is considered in RDO. In this contribution, the performance of this encoder-only optimization is presented.

# Experimental results

With common test conditions in JVET-N1010, EncDbOpt=1 is an addition setting for the test.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.99% | -1.43% | -1.16% | 104% | 100% |
| Class A2 | -0.72% | -1.82% | -1.67% | 104% | 100% |
| Class B | -0.81% | -1.52% | -1.76% | 105% | 99% |
| Class C | -0.58% | -1.41% | -1.45% | 106% | 102% |
| Class E | -0.81% | -1.53% | -1.71% | 106% | 101% |
| Overall | -0.77% | -1.53% | -1.56% | 105% | 100% |
| Class D | -0.51% | -1.24% | -1.28% | 105% | 101% |
| Class F | -0.30% | -0.94% | -0.92% | 104% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.08% | -2.22% | -1.93% | 103% | 101% |
| Class A2 | -1.14% | -2.15% | -1.97% | 103% | 102% |
| Class B | -0.91% | -2.06% | -2.21% | 103% | 100% |
| Class C | -0.64% | -1.44% | -1.55% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.92% | -1.94% | -1.93% | 103% | 101% |
| Class D | -0.50% | -1.69% | -1.64% | 101% | 99% |
| Class F | -0.52% | -1.17% | -1.09% | 102% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.83% | -0.96% | -1.19% | 102% | 103% |
| Class C | -0.63% | -0.68% | -0.72% | 102% | 98% |
| Class E | -0.90% | -1.12% | -0.57% | 102% | 100% |
| Overall | -0.78% | -0.90% | -0.88% | 102% | 101% |
| Class D | -0.47% | -0.92% | -0.38% | 104% | 99% |
| Class F | -0.53% | -0.48% | -0.02% | 103% | 102% |