| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0587-v2 |
| --- | --- |

| Title: | CE7-related: Simplifying the derivation process of ctxInc for residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a modified derivation process of ctxInc for residual coding by re-organizing the indexing of the context variables associated the syntax elements par_level_flag and abs_level_gtx_flag. In the proposed method, the three context variables for entropy coding the values of abs_level_gtx_flag[n][0], par_level_flag[n], and abs_level_gtx_flag[n][1] for a current coefficient at the scanning position n can be grouped into a subset and accessed jointly. The proposed method reportedly does not have any impact on BD bitrate performance.

# Introduction

In VVC Draft 6 [1], when the syntax element transform_skip_flag is equal to 0, the same derivation process of ctxInc is employed for the syntax elements abs_level_gtx_flag[n][0], par_level_flag[n], and abs_level_gtx_flag[n][1]. The variable ctxInc for the syntax elements par_level_flag and abs_level_gtx_flag is derived as follows:

- The variable locNumSig and locSumAbsPass1 is derived by invoking the derivation process for the variables locNumSig and locSumAbsPass1 specifies in clause 9.3.4.2.7 with colour component index cIdx, the luma location ( x0, y0), the current coefficient scan location (xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as input.

- The variable ctxOffset is set equal to Min( locSumAbsPass1 − locNumSig, 4 ).

- The variable d is set equal to xC + yC.

- If xC is equal to LastSignificantCoeffX and yC is equal to LastSignificantCoeffY, ctxInc is derived as follows:

ctxInc = ( cIdx  = =  0  ?  0  :  21 )	(9-61)

- Otherwise, if cIdx is equal to 0, ctxInc is derived as follows:

ctxInc = 1 + ctxOffset + ( d  = =  0  ?  15  :  ( d < 3  ?  10  :  ( d < 10  ?  5  :  0 ) ) )	(9-62)

- Otherwise (cIdx is greater than 0), ctxInc is derived as follows:

ctxInc = 22 + ctxOffset + ( d  = =  0  ?  5  :  0 )	(9-63)

The syntax elements abs_level_gtx_flag[n][0], par_level_flag[n], and abs_level_gtx_flag[n][1] are also present for codingl a residual block with transform_skip_flag equal to 1. The index increments for the context vairiables associated with transform_skip_flag equal to 1 are mixed with the indices for the context vairiables associated with transform_skip_flag equal to 0.

# Proposed method

This contribution proposes to reorganize the indices of the context variables associated with the syntax elements par_level_flag and abs_level_gtx_flag. In the proposed method, the context variables employed for coding the same transform block are grouped together. That is, the context variables associated with the syntax elements par_level_flag and abs_level_gtx_flag for coding a residual block with transform_skip_flag equal to 1 are grouped separately from the context variables for coding a residual block with transform_skip_flag equal to 0. In a transform block, the context variables associated with entropy coding the transform coefficients from the same block zone (determined by d) are further grouped. In a block zone, the context variables for par_level_flag and abs_level_gtx_flag associated with the same context information are further grouped together. In this way, the three context vairables for entropy coding the values of abs_level_gtx_flag[n][0], par_level_flag[n], and abs_level_gtx_flag[n][1] for a current coefficient at the scanning position n can be gouped into a subset and accessed jointly.

When transform_skip_flag is equal to 0, the context variables associated with the syntax elements par_level_flag and abs_level_gtx_flag are grouped into one set. The variable ctxInc for the syntax elements par_level_flag and abs_level_gtx_flag is derived as follows:

- The variable locNumSig and locSumAbsPass1 is derived by invoking the derivation process for the variables locNumSig and locSumAbsPass1 specifies in clause 9.3.4.2.7 with colour component index cIdx, the luma location ( x0, y0), the current coefficient scan location (xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as input.

- The variable ctxSyntaxId is set equal to 0 if the syntax element is abs_level_gtx_flag[ n ][ 0 ], equal to 1 if the syntax element is par_level_flag, and equal to 2, otherwise.

- The variable ctxOffset is set equal to Min( locSumAbsPass1 − locNumSig, 4 ).

- The variable d is set equal to xC + yC.

- If xC is equal to LastSignificantCoeffX and yC is equal to LastSignificantCoeffY, ctxInc is derived as follows:

ctxInc = ( cIdx  = =  0  ?  0  :  21 )* 3 + ctxSyntaxId	-

- Otherwise, if cIdx is equal to 0, ctxInc is derived as follows:

ctxInc = ( 1 + ctxOffset + ( d  = =  0  ?  15  :  ( d < 3  ?  10  :  ( d < 10  ?  5  :  0 ) ) ) ) * 3 +  		  ctxSyntaxId			-

- Otherwise (cIdx is greater than 0), ctxInc is derived as follows:

ctxInc = ( 22 + ctxOffset + ( d  = =  0  ?  5  :  0 )) + ctxSyntaxId	-

The context variables associated with the syntax elements par_level_flag and abs_level_gtx_flag corresponding to transform_skip_flag equal to 1 are separately grouped into two sets. The syntax element par_level_flag is entropy coded using a single context variable. The variable ctxInc for the syntax element abs_level_gtx_flag is derived as follows:

- If the syntax element is abs_level_gtx_flag[ n ][ 0 ], the following applies:

- If intra_bdpcm_flag is equal to 1, ctxInc is derived as follows:

ctxInc = 3

- Otherwise, if xC is greater than 0 and yC is greater than 0, ctxInc is derived as follows:

ctxInc = 0 + sig_coeff_flag[ xC − 1 ][ yC ] + sig_coeff_flag[ xC ][ yC − 1 ]

- Otherwise, if xC is greater than 0, ctxInc is derived as follows:

ctxInc = 0 + sig_coeff_flag[ xC − 1 ][ yC ]

- Otherwise, if yC is greater than 0, ctxInc is derived as follows:

ctxInc = 0 + sig_coeff_flag[ xC ][ yC − 1 ]

- Otherwise, ctxInc is derived as follows:

ctxInc = 0

- Otherwise, if the syntax element is abs_level_gtx_flag[ n ][ j ] with j > 0, the following applies:

ctxInc = 3 + j

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).