| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0589 |
| --- | --- |

| Title: | AHG8: On inter-layer reference picture index range |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VVC draft 6, a picture from other layer can be used as a reference picture. The index ilrp_idc of the inter-layer reference picture is signaled in a reference picture structure, while the value of the index is constrained to be in a range of 0 to GeneralLayerIdx[ nuh_layer_id ] − 1, inclusive. However, not all layers having a smaller layer index may be a dependent layer, in such case the value of ilrp_idc may be smaller than GeneralLayerIdx[ nuh_layer_id ] – 1.

In this contribution, it is proposed to count the number of direct dependent layers and constrain the range of ilrp_idc to be no greater than the number of the direct dependent layers.

# Introduction and proposal

An inter-layer reference picture is derived from the ilrp_idc signaled in the reference picture list structure with a value range of 0 to GeneralLayerIdx[ nuh_layer_id ] − 1, inclusive. However, not all layers with a smaller layer index may be used for inter-layer prediction.

The variable GeneralLayerIdx[ i ], specifying the layer index of the layer with nuh_layer_id equal to vps_layer_id[ i ], is derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ )		(7-3)
	GeneralLayerIdx[ vps_layer_id[ i ] ] = i

In this contribution, it is proposed to count the number of direct dependent layers and constrain the range of ilrp_idc to be no greater than the number of the direct dependent layers.

ilrp_idc[ listIdx ][ rplsIdx ][ i ] specifies the index, to the list of directly dependent layers, of the ILRP of i-th entry in ref_pic_list_struct( listIdx, rplsIdx ) syntax structure to the list of directly dependent layers. The value of ilrp_idc[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to the NumDirectDependentLayers[GeneralLayerIdx[ nuh_layer_id ]] − 1, inclusive.

The variable DirectDependentLayerIdx[ i ][ j ], specifying the j-th direct dependent layer of the i-th layer, and NumDirectDependentLayers[ i ] are derived as follows:

for( i = 1; i <= vps_max_layers_minus1; i− − )

NumDirectDependentLayers[ i ] = 0
	if( !vps_independent_layer_flag[ i ] ) {			for( j = i, k = 0; j  >=  0; j− − )		(7-2)
			if( vps_direct_dependency_flag[ i ][ j ] ) {
				NumDirectDependentLayers[ i ] ++				DirectDependentLayerIdx[ i ][ k++ ] = j			}

}

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).