| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0592-v3 |
| --- | --- |

| Title: | AHG8/Non-CE1: Phase shifts for resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VVC draft 6, reference picture resampling process assumes zero phase shift between luma and chroma components, while sequences may have other alignment between luma and chroma grids, such as in CTC half phase shift in the vertical direction is used.

In this contribution, it is proposed to signal luma and chroma phase shifts in PPS that are used for reference picture resampling and scalability by applying the phase shifts during motion compensation when scaling ratio is different from 1x.

In the third revision of the contribution, specification changes for the method agreed during the review of this proposal is provided in the attached document. In the agreed method, two flags are introduced to specify chroma phase offsets for horizontal and vertical directions, and those phase offsets are used in the resampling process.

# Introduction and proposal

In VVC draft 6, zero phase shift is assumed between luma and chroma components (type 2), while other alignment between luma and chroma grids may be used too as shown on Fig. 1. Video sequences in CTC have half phase shift in the vertical direction (type 0).

Figure 1. Chroma sampling types.

In HEVC scalable extension, luma and chroma phases are signaled to account the phase difference between different layers.

In this contribution, it is proposed to signal luma and chroma phase shifts in PPS that are used to compensate the phase difference between a current and a reference pictures when resampling (scaling ratio is different from 1x) is applied.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| resample_phase_set_present_flag | u(1) |
| if( resample_phase_set_present_flag ) { |  |
| phase_hor_luma | ue(v) |
| phase_ver_luma | ue(v) |
| phase_hor_chroma | ue(v) |
| phase_ver_chroma | ue(v) |
| } |  |
| … |  |

resample_phase_set_present_flag equal to 1 specifies that the resampling phase set is present in the PPS. resample_phase_set_present_flag equal to 0 specifies that the resampling phase set is not present in the PPS.

The resampling phase set specifies the phase offsets used in the resampling process of a picture referring to the PPS.

phase_hor_luma specifies the luma phase shift in the horizontal direction used in the resampling process. The value of phase_hor_luma shall be in the range of 0 to 31, inclusive. When not present, the value of phase_hor_luma is inferred to be equal to 0.

phase_ver_luma specifies the luma phase shift in the vertical direction used in the resampling process. The value of phase_ver_luma shall be in the range of 0 to 31, inclusive. When not present, the value of phase_ver_luma is inferred to be equal to 0.

phase_hor_chroma specifies the chroma phase shift in the horizontal direction used in the resampling process. The value of phase_hor_chroma shall be in the range of 0 to 63, inclusive. When not present, the value of phase_hor_chroma is inferred to be equal to 0.

phase_ver_chroma specifies the chroma phase shift in the vertical direction used in the resampling process. The value of phase_ver_chroma shall be in the range of 0 to 63, inclusive. When not present, the value of phase_ver_chroma is inferred equal to 0 if chroma_format_idc is equal to 3 (4:4:4 chroma format), or equal to 8, otherwise.

The phase shifts are taken into the account in the motion compensation process for luma and choma blocks if scaling ratio is different from 1x.

      - 8.5.6.3 Fractional sample interpolation process

        - 8.5.6.3.1 General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture,

- a variable sbWidth specifying the width of the current coding subblock,

- a variable sbHeight specifying the height of the current coding subblock,

- a motion vector offset mvOffset,

- a refined motion vector refMvLX,

- the selected reference picture sample array refPicLX,

- the half sample interpolation filter index hpelIfIdx,

- the bi-directional optical flow flag bdofFlag,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an (sbWidth + brdExtSize)x(sbHeight + brdExtSize) array predSamplesLX of prediction sample values.

The prediction block border extension size brdExtSize is derived as follows:

brdExtSize = ( bdofFlag | | ( inter_affine_flag[ xSb ][ ySb ] && sps_affine_prof_enabled_flag ) ) ? 2 : 0	(8-752)

The variable fRefWidth is set equal to the PicOutputWidthL of the reference picture in luma samples.

The variable fRefHeight is set equal to PicOutputHeightL of the reference picture in luma samples.

The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

- If cIdx is equal to 0, the following applies:

  - The scaling factors and their fixed-point representations are defined as

hori_scale_fp = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL >>  1 ) ) / PicOutputWidthL	(8-753)

vert_scale_fp = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL >>  1 ) ) / PicOutputHeightL	(8-754)

  - Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in 1/16-sample units. These variables are used only in this clause for specifying fractional-sample locations inside the reference sample arrays refPicLX.

  - The top-left coordinate of the bounding block for reference sample padding ( xSbIntL, ySbIntL ) is set equal to ( xSb + ( mvLX[ 0 ]  >>  4 ), ySb + ( mvLX[ 1 ]  >>  4 ) ).

  - For each luma sample location ( xL = 0..sbWidth − 1 + brdExtSize, yL = 0..sbHeight − 1 + brdExtSize ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows:

- Let ( refxSbL, refySbL ) and ( refxL, refyL ) be luma locations pointed to by a motion vector ( refMvLX[0], refMvLX[1] ) given in 1/16-sample units. The variables refxSbL, refxL, refySbL, and refyL are derived as follows:

ref_ phase_hor_luma is set equal to phase_hor_luma of refPicLX, ref_ phase_ver_luma is set equal to phase_ver_luma of refPicLX.

addX = hori_scale_fp == (1 <<14) ? 0 : phase_hor_luma * hori_scale_fp – (1 <<14 ) * ref_ phase_hor_luma

addY = vert_scale_fp == (1 <<14) ? 0 : phase_ver_luma * vert_scale_fp – (1 <<14 ) * ref_ phase_ver_luma

refxSbL = ( ( xSb  <<  4 ) + refMvLX[ 0 ] ) * hori_scale_fp + addX	(8-755)

refxL = ( ( Sign( refxSb ) * ( ( Abs( refxSb ) + 128 )  >>  8 )
		+ xL * ( ( hori_scale_fp + 8 )  >>  4 ) ) + 32 )  >>  6	(8-756)

refySbL = ( ( ySb  <<  4 ) + refMvLX[ 1 ] ) * vert_scale_fp + addY	(8-757)

refyL = ( ( Sign( refySb ) * ( ( Abs( refySb ) + 128 )  >>  8 ) + yL *
		( ( vert_scale_fp + 8 )  >>  4 ) ) + 32 )  >>  6	(8-758)

- The variables xIntL, yIntL, xFracL and yFracL are derived as follows:

xIntL = refxL  >>  4		(8-759)

yIntL = refyL  >>  4		(8-760)

xFracL = refxL & 15		(8-761)

yFracL = refyL & 15		(8-762)

  - If bdofFlag is equal to TRUE or ( sps_affine_prof_enabled_flag is equal to TRUE and inter_affine_flag[ xSb ][ ySb ] is equal to TRUE ), and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntL + ( xFracL >> 3) − 1), yIntL + ( yFracL >> 3 ) − 1 ) and refPicLX as inputs.

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

  - Otherwise, the  prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 8-tap interpolation filtering process as specified in clause 8.5.6.3.2 with ( xIntL − ( brdExtSize > 0 ? 1 : 0 ), yIntL − ( brdExtSize > 0 ? 1 : 0 ) ), ( xFracL, yFracL ), ( xSbIntL, ySbIntL ), refPicLX, hpelIfIdx, sbWidth, sbHeight and ( xSb, ySb ) as inputs.

- Otherwise (cIdx is not equal to 0), the following applies:

  - Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in 1/32 sample units. These variables are used only in this clause for specifying general fractional-sample locations inside the reference sample arrays refPicLX.

  - The top-left coordinate of the bounding block for reference sample padding ( xSbIntC, ySbIntC ) is set equal to ( (xSb / SubWidthC ) + ( mvLX[ 0 ]  >>  5), ( ySb / SubHeightC ) + ( mvLX[ 1 ]  >>  5 ) ).

  - For each chroma sample location ( xC = 0..sbWidth − 1, yC = 0.. sbHeight − 1 ) inside the prediction chroma sample arrays predSamplesLX, the corresponding prediction chroma sample value predSamplesLX[ xC ][ yC ] is derived as follows:

- Let (refxSbC, refySbC) and ( refxC, refyC ) be chroma locations pointed to by a motion vector (mvLX[ 0 ], mvLX[ 1 ]) given in 1/32-sample units. The variables refxSbC, refySbC, refxC and refyC are derived as follows:

ref_phase_hor_chroma is set equal to phase_hor_chroma of refPicLX, ref_ phase_ver_chroma is set equal to phase_ver_chroma of refPicLX.

addX = hori_scale_fp == (1 <<14) ? 0 : ( phase_hor_chroma * hori_scale_fp – (1 <<14 ) * ref_phase_ver_chroma) * 2 / SubWidthC

addY = vert_scale_fp == (1 <<14) ? 0 : ( phase_ver_chroma * vert_scale_fp – (1 <<14 ) * ref_phase_ver_chroma) * 2 / SubHeightC

refxSbC =  ( ( xSb / SubWidthC  <<  5 ) + mvLX[ 0 ] ) * hori_scale_fp + addX	 (8-763)

refxC = ( ( Sign( refxSbC ) * ( ( Abs( refxSbC ) + 256 )  >>  9 )
		+ xC * ( ( hori_scale_fp + 8 )  >>  4 ) ) + 16 )  >>  5	(8-764)

refySbC =  ( ( ySb / SubHeightC  <<  5 )  + mvLX[ 1 ] ) * vert_scale_fp + addY	 (8-765)

refyC = ( ( Sign( refySbC ) * ( ( Abs( refySbC ) + 256 )  >>  9 )
		+ yC* ( ( vert_scale_fp + 8 )  >>  4 ) ) + 16 )  >>  5	(8-766)

- The variables xIntC, yIntC, xFracC and yFracC are derived as follows:

xIntC = refxC  >>  5		(8-767)

yIntC = refyC  >>  5		(8-768)

xFracC = refyC & 31		(8-769)

yFracC = refyC & 31		(8-770)

  - The prediction sample value predSamplesLX[ xC ][ yC ] is derived by invoking the process specified in clause 8.5.6.3.4 with ( xIntC, yIntC ), ( xFracC, yFracC ), ( xSbIntC, ySbIntC ), sbWidth, sbHeight and refPicLX as inputs.

# Test results

The proposed method was implemented on top VTM-6.1 with CE1 affine patch, and two tests are performed:

Test 1: Phase shift for chroma is set corresponding to sample type 0, i.e. vertical chroma phase shift is set to 0.5.

Test 2: Phase shift for source downsampling is kept unchanged, i.e. zero for both luma and chroma, but in normative resampling it is set to (0.5, 0.5) for luma and (0, 0.5) for chroma.

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.11% | -0.26% | 98% | 96% |
| Class C | -0.04% | -0.30% | -0.08% | 98% | 90% |
| Class E | -0.24% | -0.16% | 0.69% | 98% | 98% |
| Overall | -0.09% | -0.18% | 0.04% | 98% | 95% |
| Class D | -0.05% | -0.12% | -0.01% | 96% | 89% |
| Class F | 0.20% | 0.24% | -0.34% | 98% | 98% |

Test 1. 1.5x PSNR 1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.46% | -0.56% | -0.88% | 98% | 96% |
| Class C | -1.06% | 0.30% | 0.78% | 98% | 90% |
| Class E | -3.50% | -2.39% | -1.31% | 98% | 98% |
| Overall | -1.42% | -0.73% | -0.44% | 98% | 95% |
| Class D | -1.04% | -0.40% | -0.71% | 96% | 89% |
| Class F | -1.09% | -0.65% | -1.16% | 98% | 98% |

Test 1. 1.5x PSNR2

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 1.06% | 0.61% | 98% | 98% |
| Class C | -0.08% | 0.37% | 0.15% | 99% | 97% |
| Class E | -0.07% | 1.15% | 0.73% | 99% | 98% |
| Overall | -0.04% | 0.85% | 0.49% | 99% | 98% |
| Class D | -0.03% | 0.14% | 0.30% | 98% | 92% |
| Class F | 0.13% | 0.00% | 0.15% | 96% | 92% |

Test 1. 2.0x PSNR 1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.18% | -0.54% | 98% | 98% |
| Class C | -0.04% | 0.68% | -0.31% | 99% | 97% |
| Class E | 0.05% | -2.56% | 0.17% | 99% | 98% |
| Overall | -0.02% | -0.49% | -0.29% | 99% | 98% |
| Class D | -0.04% | -0.35% | 0.14% | 98% | 92% |
| Class F | 0.10% | -0.30% | 0.52% | 96% | 92% |

Test 1. 2.0x PSNR 2

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.47% | 0.34% | 99% | 99% |
| Class C | 0.03% | 0.42% | 0.36% | 99% | 94% |
| Class E | -0.08% | 0.62% | 2.06% | 98% | 96% |
| Overall | 0.00% | 0.49% | 0.78% | 99% | 96% |
| Class D | 0.08% | 0.84% | 0.15% | 98% | 95% |
| Class F | -0.15% | 0.45% | -0.05% | 99% | 97% |

Test 2. 1.5x PSNR 1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 27.64% | -0.09% | -0.12% | 99% | 99% |
| Class C | 11.87% | 0.24% | 0.32% | 99% | 94% |
| Class E | 43.26% | 1.38% | 2.52% | 98% | 96% |
| Overall | 26.29% | 0.39% | 0.69% | 99% | 96% |
| Class D | 12.22% | 0.94% | 0.17% | 98% | 95% |
| Class F | 16.17% | -1.37% | -1.49% | 99% | 97% |

Test 2. 1.5x PSNR 2

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 1.67% | 1.61% | 99% | 98% |
| Class C | -0.07% | 0.96% | 0.83% | 99% | 97% |
| Class E | 0.06% | 3.17% | 4.33% | 99% | 98% |
| Overall | 0.00% | 1.81% | 2.03% | 99% | 97% |
| Class D | 0.01% | 0.54% | 1.15% | 99% | 94% |
| Class F | -1.86% | -0.94% | -1.49% | 99% | 97% |

Test 2. 2.0x PSNR 1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 79.93% | 2.39% | 3.16% | 99% | 98% |
| Class C | 22.19% | 2.39% | 1.81% | 99% | 97% |
| Class E | 79.48% | 0.04% | 3.65% | 99% | 98% |
| Overall | 60.57% | 1.80% | 2.83% | 99% | 97% |
| Class D | 21.77% | 0.97% | 3.01% | 99% | 94% |
| Class F | 28.21% | 1.52% | 1.43% | 99% | 97% |

Test 2. 2.0x PSNR 2

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).