| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0595 |
| --- | --- |

| Title: | Non-CE4: Neighboring locations for CIIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geonjung Ko, Dongcheol Kim, Jason Jung, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 greg.ko@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to change the derivation of neighboring locations used for CIIP weight determination. CIIP (Combined inter merge and intra prediction) refers two neighboring locations to determine the weight for the combination of the inter prediction signal and the intra prediction signal. The two neighboring locations are left and above of the current block, but for the color format other than 4:2:0, the derived location in the specification indicates a non-adjacent location for chroma components. The proposed change fixes the specification text on neighboring locations to represent left and above locations for all color formats.

# Introduction

CIIP (Combined inter merge and intra prediction) refers two neighboring locations to determine the weight used for combining the inter prediction signal and the intra prediction signal [1]. The two neighboring locations are left (NbA) and above (NbB) of the current block as shown in Figure 1.

Figure 1. Neighboring locations for CIIP

Also two neighboring locations are represented in luma samples in the specification as below.

- ( xNbA, yNbA ) = ( xCb − 1, yCb − 1 + ( cbHeight << scallFact ) )

- ( xNbB, yNbB ) = ( xCb − 1  + (cbWidth << scallFact ), yCb − 1 )

where scallFact = ( cIdx = = 0 ) ? 0 : 1. Neighboring luma locations are derived using bit shift by scallFact, which is equal to 1 for chroma components. Since both SubWidthC and SubHeightC are equal to 2 for 4:2:0 format, the derived locations indicate NbA and NbB in Figure 1. However, if SubWidthC or SubHeightC is equal to 1, a non-adjacent location is indicated for chroma components.

# Proposed specification change

It is proposed to change the derivation of neighboring locations for CIIP to represent left and above locations for all color formats. Table 1 shows the proposed change.

Table 1. Proposed specification change

#### 8.5.6.7 Weighted sample prediction process for combined merge and intra prediction Inputs to this process are: - a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture, - the width of the current coding block cbWidth, - the height of the current coding block cbHeight, - two (cbWidth)x(cbHeight) arrays predSamplesInter and predSamplesIntra, - a variable cIdx specifying the colour component index. Output of this process is the (cbWidth)x(cbHeight) array predSamplesComb of prediction sample values. The variable bitDepth is derived as follows: - If cIdx is equal to 0, bitDepth is set equal to BitDepthY. - Otherwise, bitDepth is set equal to BitDepthC. The variable scallFact is derived as follows: scallFactWidth = ( cIdx = = 0 ¦ ¦ SubWidthC = = 1 ) ? 0 : 1. (8-838) scallFactHeight = ( cIdx = = 0 ¦ ¦ SubHeightC = = 1 ) ? 0 : 1. (8-839) The neighbouring luma locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb − 1 + ( cbHeight << scallFactHeight ) ) and ( xCb − 1 + (cbWidth << scallFactWidth ), yCb − 1 ), respectively. …

# Conclusion

This contribution proposes to change the derivation of neighboring locations for CIIP to indicate adjacent left and above locations for all color formats. The proposed change has no impact on VTM. It is suggested to adopt the proposed specification change in the next version of VVC draft text.

# Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th meeting, Gothenburg, SE, 3–12 July 2019.