| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0599-v2 |
| --- | --- |

| Title: | Non-CE3: Cleanup of interpolation filtering for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Filippov, V. Rufitskiy | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This document presents a cleanup of interpolation filtering process by replacing 4-tap “Gaussian” filter with a 4-tap smoothing filter obtained by convolving a linear filter with 3-tap FIR filter that has coefficients of [1, 2, 1]/4. Over VTM-6.0, this modification reportedly shows the overall results of 0.00% (Y), -0.02% (U), -0.01% (V) for AI configuration and -0.02% (Y), 0.07% (U), -0.01% (V) for RA configuration. Both encoding and decoding time values are kept appr. 100% for AI and RA configurations.

# Introduction

In the current design of VTM-6.0 [1], a set of 4-tap filter coefficients are defined to perform interpolation for intra prediction. Coefficients of an interpolation filter is defined in a form of a lookup table that contain 32 entries of 4 coefficients for each filter.

It could be noticed that frequency responses of “fG” filters given in Table 8-9 of [1] change irregularly with respect to the value of a fractional sample position.

This cleanup was initially proposed as contribution JVET-M0158 at the JVET meeting in Marrakech. As mentioned in the meeting notes [2], this proposal "may be logical, but we don’t need to consider such a small change before the design is more mature" (p. 186).

Original set of coefficient for a DCTIF (“fC”) and “Gaussian” (”fG”) interpolation filters is given in Table 1. As shown in Fig. 1, the frequency response of “Gaussian” interpolation filter has some inconsistency that may cause subjective artifacts.

Table1. Original set of coefficient for a DCTIF (“fC”) and Gaussian (”fG”) interpolation filters.

| Fractional sample position p | fC interpolation filter coefficients |  |  |  | fG interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] | fG[ p ][ 0 ] | fG[ p ][ 1 ] | fG[ p ][ 2 ] | fG[ p ][ 3 ] |
| 0 | 0 | 64 | 0 | 0 | 16 | 32 | 16 | 0 |
| 1 | −1 | 63 | 2 | 0 | 15 | 29 | 17 | 3 |
| 2 | −2 | 62 | 4 | 0 | 15 | 29 | 17 | 3 |
| 3 | −2 | 60 | 7 | −1 | 14 | 29 | 18 | 3 |
| 4 | −2 | 58 | 10 | −2 | 13 | 29 | 18 | 4 |
| 5 | −3 | 57 | 12 | −2 | 13 | 28 | 19 | 4 |
| 6 | −4 | 56 | 14 | −2 | 13 | 28 | 19 | 4 |
| 7 | −4 | 55 | 15 | −2 | 12 | 28 | 20 | 4 |
| 8 | −4 | 54 | 16 | −2 | 11 | 28 | 20 | 5 |
| 9 | −5 | 53 | 18 | −2 | 11 | 27 | 21 | 5 |
| 10 | −6 | 52 | 20 | −2 | 10 | 27 | 22 | 5 |
| 11 | −6 | 49 | 24 | −3 | 9 | 27 | 22 | 6 |
| 12 | −6 | 46 | 28 | −4 | 9 | 26 | 23 | 6 |
| 13 | −5 | 44 | 29 | −4 | 9 | 26 | 23 | 6 |
| 14 | −4 | 42 | 30 | −4 | 8 | 25 | 24 | 7 |
| 15 | −4 | 39 | 33 | −4 | 8 | 25 | 24 | 7 |
| 16 | −4 | 36 | 36 | −4 | 8 | 24 | 24 | 8 |
| 17 | −4 | 33 | 39 | −4 | 7 | 24 | 25 | 8 |
| 18 | −4 | 30 | 42 | −4 | 7 | 24 | 25 | 8 |
| 19 | −4 | 29 | 44 | −5 | 6 | 23 | 26 | 9 |
| 20 | −4 | 28 | 46 | −6 | 6 | 23 | 26 | 9 |
| 21 | −3 | 24 | 49 | −6 | 6 | 22 | 27 | 9 |
| 22 | −2 | 20 | 52 | −6 | 5 | 22 | 27 | 10 |
| 23 | −2 | 18 | 53 | −5 | 5 | 21 | 27 | 11 |
| 24 | −2 | 16 | 54 | −4 | 5 | 20 | 28 | 11 |
| 25 | −2 | 15 | 55 | −4 | 4 | 20 | 28 | 12 |
| 26 | −2 | 14 | 56 | −4 | 4 | 19 | 28 | 13 |
| 27 | −2 | 12 | 57 | −3 | 4 | 19 | 28 | 13 |
| 28 | −2 | 10 | 58 | −2 | 4 | 18 | 29 | 13 |
| 29 | −1 | 7 | 60 | −2 | 3 | 18 | 29 | 14 |
| 30 | 0 | 4 | 62 | −2 | 3 | 17 | 29 | 15 |
| 31 | 0 | 2 | 63 | −1 | 3 | 17 | 29 | 15 |

Fig. 1. Frequency responses of the “Gaussian” FIR filters used in [1]

# Proposed method

In this document, it is proposed to use a combination of a [1, 2, 1]/4 3-tap FIR reference sample smoothing filter and a linear interpolation filter for the modes that have a fractional slope. No changes to MDIS checking or any other conditions are proposed.

Alternatively, this combination could be represented as a modified set of 4-tap filter coefficient (Table 2).

Table 2. Modified set of 4-tap filter coefficient according to the proposed modification

| Fractional sample position p | fC interpolation filter coefficients |  |  |  | fG interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] | fG[ p ][ 0 ] | fG[ p ][ 1 ] | fG[ p ][ 2 ] | fG[ p ][ 3 ] |
| 0 | 0 | 64 | 0 | 0 | 16 | 32 | 16 | 0 |
| 1 | −1 | 63 | 2 | 0 | 16 | 32 | 16 | 0 |
| 2 | −2 | 62 | 4 | 0 | 15 | 31 | 17 | 1 |
| 3 | −2 | 60 | 7 | −1 | 15 | 31 | 17 | 1 |
| 4 | −2 | 58 | 10 | −2 | 14 | 30 | 18 | 2 |
| 5 | −3 | 57 | 12 | −2 | 14 | 30 | 18 | 2 |
| 6 | −4 | 56 | 14 | −2 | 13 | 29 | 19 | 3 |
| 7 | −4 | 55 | 15 | −2 | 13 | 29 | 19 | 3 |
| 8 | −4 | 54 | 16 | −2 | 12 | 28 | 20 | 4 |
| 9 | −5 | 53 | 18 | −2 | 12 | 28 | 20 | 4 |
| 10 | −6 | 52 | 20 | −2 | 11 | 27 | 21 | 5 |
| 11 | −6 | 49 | 24 | −3 | 11 | 27 | 21 | 5 |
| 12 | −6 | 46 | 28 | −4 | 10 | 26 | 22 | 6 |
| 13 | −5 | 44 | 29 | −4 | 10 | 26 | 22 | 6 |
| 14 | −4 | 42 | 30 | −4 | 9 | 25 | 23 | 7 |
| 15 | −4 | 39 | 33 | −4 | 9 | 25 | 23 | 7 |
| 16 | −4 | 36 | 36 | −4 | 8 | 24 | 24 | 8 |
| 17 | −4 | 33 | 39 | −4 | 8 | 24 | 24 | 8 |
| 18 | −4 | 30 | 42 | −4 | 7 | 23 | 25 | 9 |
| 19 | −4 | 29 | 44 | −5 | 7 | 23 | 25 | 9 |
| 20 | −4 | 28 | 46 | −6 | 6 | 22 | 26 | 10 |
| 21 | −3 | 24 | 49 | −6 | 6 | 22 | 26 | 10 |
| 22 | −2 | 20 | 52 | −6 | 5 | 21 | 27 | 11 |
| 23 | −2 | 18 | 53 | −5 | 5 | 21 | 27 | 11 |
| 24 | −2 | 16 | 54 | −4 | 4 | 20 | 28 | 12 |
| 25 | −2 | 15 | 55 | −4 | 4 | 20 | 28 | 12 |
| 26 | −2 | 14 | 56 | −4 | 3 | 19 | 29 | 13 |
| 27 | −2 | 12 | 57 | −3 | 3 | 19 | 29 | 13 |
| 28 | −2 | 10 | 58 | −2 | 2 | 18 | 30 | 14 |
| 29 | −1 | 7 | 60 | −2 | 2 | 18 | 30 | 14 |
| 30 | 0 | 4 | 62 | −2 | 1 | 17 | 31 | 15 |
| 31 | 0 | 2 | 63 | −1 | 1 | 17 | 31 | 15 |

Frequency responses for the proposed “fG” filter are shown in Fig. 2.

Fig. 2. Frequency responses for the proposed “fG” filters.

To evaluate the difference in subjective quality, intra predictors for mode 14 (HOR_IDX - 4) are shown for the VTM-6.0 “Gaussian” filter and the proposed one in Fig. 3 and Fig. 5, respectively.

| Fig. 3. Intra predictors for mode 14 (HOR_IDX - 4) obtained using the VTM-6.0 “Gaussian” filter | Fig. 4. Intra predictors for mode 14 (HOR_IDX - 4) obtained using the proposed filter |
| --- | --- |

# Experimental results

## The simulation results using the JVET common test conditions (CTC) [2] over VTM-6.0 are reportedly presented in Table 3 for AI and RA configurations.

Table 3. Simulation results for the proposed replacements (test 1)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.01% | -0.04% | 100% | 100% |
| Class B | 0.00% | 0.00% | -0.03% | 100% | 100% |
| Class C | 0.01% | -0.07% | 0.06% | 100% | 100% |
| Class E | -0.01% | -0.02% | -0.05% | 100% | 100% |
| Overall | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class D | 0.00% | 0.01% | -0.10% | 100% | 100% |
| Class F | -0.02% | 0.00% | -0.08% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.07% | -0.04% | 100% | 100% |
| Class A2 | 0.02% | 0.02% | -0.05% | 100% | 100% |
| Class B | -0.04% | 0.06% | 0.03% | 100% | 100% |
| Class C | -0.04% | 0.13% | -0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.07% | -0.01% | 100% | 100% |
| Class D | -0.01% | -0.29% | -0.02% | 100% | 100% |
| Class F | 0.00% | -0.17% | 0.02% | 100% | 100% |

The second test compares two smoothing filters: the Gaussian one of the VTM-6.0 with the one being proposed. DCTIF was disabled for both anchor and test simulations so that interpolation could be performed with just one filter type per simulation. Results of this test are reportedly presented in Table 4 for AI and RA configurations.

Table 4. Simulation results of test 2: (DCTIF is disabled for anchor and test)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 without DCTIF intra interpolation |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.10% | -0.06% | 100% | 100% |
| Class A2 | -0.06% | 0.01% | -0.05% | 100% | 100% |
| Class B | -0.07% | -0.12% | -0.05% | 100% | 100% |
| Class C | -0.05% | 0.01% | 0.06% | 100% | 100% |
| Class E | -0.09% | -0.10% | -0.05% | 100% | 100% |
| Overall | -0.06% | -0.06% | -0.03% | 100% | 100% |
| Class D | -0.04% | -0.01% | -0.07% | 100% | 100% |
| Class F | -0.02% | 0.02% | 0.09% | 100% | 101% |
|  | Random Access |  |  |  |  |
|  | Over VTM6.0 without DCTIF intra interpolation |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.06% | 0.02% | 100% | 100% |
| Class A2 | -0.06% | -0.20% | -0.07% | 100% | 100% |
| Class B | -0.04% | 0.05% | -0.13% | 100% | 100% |
| Class C | -0.01% | -0.08% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.03% | -0.07% | 100% | 100% |
| Class D | -0.02% | 0.20% | 0.20% | 100% | 100% |
| Class F | -0.03% | 0.10% | -0.07% | 100% | 100% |

# Conclusion

In this contribution, we propose to clean up the VTM-6.0 4-tap smoothing “Gaussian” interpolation filter having inconsistency in its frequency response by the 4-tap smoothing interpolation filter obtained by convolving a linear filter with 3-tap FIR filter that has coefficients of [1, 2, 1]/4. Minor gain is observed for RA configuration under the JVET CTC conditions. We recommend to adopt the proposed filter into the VTM reference software and the VVC spec draft.

# References

- B. Bross, J. Chen, S. Liu., Versatile Video Coding (Draft 6), “Versatile Video Coding (Draft 6)”, JVET-O2001, 15th JVET Meeting, Gothenburg, Sweden, July 2019.

- G. J. Sullivan, J.-R. Ohm, “Meeting Report of the 13th JVET Meeting”, JVET-M1000, 13th JVET Meeting, Marrakech, January 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET Meeting, Geneva, Switzerland, March 2019.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).