| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0602 |
| --- | --- |

| Title: | [AHG18][non-CE5] Boundary strength derivation for CUs with TPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution proposes modification of the boundary strength derivation for CUs with triangle partition mode which was originally discussed on 3rd of September on the JVET reflector as a part of AHG18 activities.

Introduction

In the 15th JVET meeting, modification of boundary strength derivation for CUs with transform skip mode was proposed in JVET-O0656. The meeting note of JVET-O0656 indicated that “It is definitely necessary to take some action”, and also suggested the establishment of AHG18 to study issues in a large context of TS, lossless, BDPCM, QP alignment and deblocking alignment.

During the meeting cycle, as a part of AHG18 activities, a thorough study of the interaction between deblocking filter and other coding tools were conducted. This contribution proposes BS derivation for CUs with TPM.

Proposed modification

The following two issues were found in the study of interaction between deblocking and motion vector storage of TPM. For both cases, the potential discontinuity of the block boundary caused by the discrepancy of the weights was overlooked in the current VTM design.

Case 1)

Let the block P be coded by TPM (L0: MVP1, L1:MVP2) and the adjacent block Q be coded by full-block bi-prediction (L0: MVQ1, L1:MVQ2) as shown Figure 5.

When MVP1 and MVP2 are identical or mostly equal to MVQ1 and MVQ2, respectively, bS is set equal to 0 if both blocks are skipped.

However, the prediction of block P is generated by spatially varying weighted average and that of block Q is generated by equal weighted average. This may cause discontinuity of the block boundary, but the current VTM design does not consider the case.

Figure 5.

Case 2)

In the last meeting, simplification of motion vector storage is adopted where motion vector of MVP2 is stored when the same reference picture list is used for both MVP1 and MVP2. Due to this change, discontinuity of the block boundary may appear in the following situation.

Let be the block P be coded by TPM (both MVP1 and MVP2 are from L0) and the block Q be coded uni-prediction with MVQ (identical or mostly equal to MVP2) as shown in Figure 6. Due to the adoption of the abovementioned simplification, the MVP2 is stored in the diagonal position.

Since MV difference is smaller than the threshold, bS of the boundary in the green square of Fig. 6 is set equal to 0 when both block P and Q are skipped. However, the prediction of block P is generated by spatially varying weighted average and that of block Q is uni-predicted by MVQ which is almost equal to MVP2. This may cause potential discontinuity on the block boundary, but not considered in the current VTM.

Figure 6.

To solve these issues, it is suggested that BS is set equal to 1 for subblock boundaries in the diagonal (anti-diagonal) position of the TPM if the boundary is not inside the TPM block.

The proposed modification of the specification is to add one condition to the BS derivation as follows (texts yellow highlighted are changed):

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - …

    - Otherwise, if the sample p0 is in the coding block of a coding unit coded with triangle merge mode, and the coding subblocks containing p0 and q0 are not in the same coding block, and the sample p0 is not generated by weighted sample prediction.

    - Otherwise, if the sample q0 is in the coding block of a coding unit coded with triangle merge mode, and the coding subblocks containing p0 and q0 are not in the same coding block and , the sample q0 is not generated by weighted sample prediction.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).