| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0603 |
| --- | --- |

| Title: | CABAC zero word stuffing signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Adarsh Ramasubramonian Vadim Seregin Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to signal the number of stuffing bytes needed to satisfy picture level maximum bin to bit ratio constraint after coding a picture instead of appending cabac zero bytes.

# Proposal

VVC (Draft 6) imposes picture level bin to bit ratio limit by requiring BinCountsInNalUnits, the number of bins parsed when decoding the contents of all VCL NAL units of a coded picture,  to be less than  or equal to ( 32 ÷ 3 ) * NumBytesInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  32 where NumBytesInVclNalUnits is the sum of the values of NumBytesInNalUnit for all VCL NAL units of a coded picture and RawMinCuBits is given by, RawMinCuBits = MinCbSizeY * MinCbSizeY *( BitDepthY + 2 * BitDepthC / ( SubWidthC * SubHeightC ) ). This constraint can be met by increasing the number of coded bits of a picture by stuffing it with cabac zero words (0x0000) when needed.

It is proposed to signal the number stuffing bytes needed by a fixed length code at the end of a picture instead of encoding cabac zero words to increase the number of bits of a coded picture, avoiding coding of stuffing bytes. The number of coded bits of a picture is calculated as NumBytesInVclNalUnits + signalled number of stuffing bytes. This total value is used for as the bit count of a coded picture.

The proposed signalling as follows:

#### 7.3.2.11 RBSP slice trailing bits syntax

| rbsp_slice_trailing_bits( ) { | Descriptor |
| --- | --- |
| rbsp_trailing_bits( ) |  |
| while( more_rbsp_trailing_data( ) ) |  |
| cabac_zero_word /* equal to 0x0000 */ | f(16) |
| if( num_stuffing_bytes_flag ) |  |
| num_stuffing_bytes | f(32) |
| } |  |

num_stuffing_bytes: number of bytes stuffed.

#### 7.3.2.12 RBSP trailing bits syntax

| rbsp_trailing_bits( ) { | Descriptor |
| --- | --- |
| rbsp_stop_one_bit /* equal to 1 */ | f(1) |
| num_stuffing_bytes_flag | f(1) |
| while( !byte_aligned( ) ) |  |
| rbsp_alignment_zero_bit /* equal to 0 */ | f(1) |
| } |  |

num_stuffing_bytes_flag : When set to 1, num_stuffing_bytes field is present in rbsp_slice_training_bits() process.

#### 7.4.3.11 RBSP slice trailing bits semantics

cabac_zero_word is a byte-aligned sequence of two bytes equal to 0x0000.

Let NumBytesInVclNalUnits be the sum of the values of NumBytesInNalUnit for all VCL NAL units of a coded picture and the num_stuffing_bytes field value.

Let BinCountsInNalUnits be the number of times that the parsing process function DecodeBin( ), specified in clause TBD, is invoked to decode the contents of all VCL NAL units of a coded picture.

Let the variable RawMinCuBits be derived as follows:

RawMinCuBits = MinCbSizeY * MinCbSizeY *
				( BitDepthY + 2 * BitDepthC / ( SubWidthC * SubHeightC ) )	(7-46)

The value of BinCountsInNalUnits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  32.

NOTE – The constraint on the maximum number of bins resulting from decoding the contents of the coded slice NAL units can be met by inserting a number of cabac_zero_word syntax elements to increase the value of NumBytesInVclNalUnits. Each cabac_zero_word is represented in a NAL unit by the three-byte sequence 0x000003 (as a result of the constraints on NAL unit contents that result in requiring inclusion of an emulation_prevention_three_byte for each cabac_zero_word).

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).