| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 Oct. 2019 | Document: JVET-P0606-v2 |
| --- | --- |

| Title: | AHG18: Enabling lossless coding with minimal impact on VVC design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Benjamin Bross, Tung Nguyen Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany Marta Karczewicz, Yung-Hsuan Chao, Hongtao Wang, Muhammed Coban 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | firstname.lastname@hhi.fraunhofer.de martak@qti.qualcomm.com |
| Source: | Fraunhofer HHI, Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution analyses the required changes to enable lossless coding in VVC. To facilitate lossless coding with minimal impact on current VVC syntax and decoding process, it is proposed to enable transform skip for chroma as tested in CE8-2.1 (JVET-P0058). In addition to that, in order to improve the coding efficiency with regard to an HM lossless anchor, the modified transform skip residual coding as tested in CE7-1.2d is proposed (JVET-P0079). The proposed design enables lossless coding with minimal impact on the syntax, i.e. without introducing an additional cu_transquant_bypass_flag for lossless coding and condition other normative parts on that new syntax element. Compared to the HM lossless anchor, results show a bit-rate reduction of 5.27% for AI and 1.23% for RA and compared to the AHG18 VTM lossless anchor, results show a bit-rate reduction of 0.01% for AI and a bit-rate increase of 2.48% for RA.

In v2, full frame results have been added.

# AHG18 VTM lossless anchor

The AHG18 lossless VTM anchor is based on the HEVC transform quantization bypass and extended to new VVC tools with normative and non-normative changes as follows.

## Normative changes

The CU-level flag cu_transquant_bypass_flag is introduced that controls the following:

- skips transform but compared to VTM transform skip:

  - skips transform_skip_flag syntax

  - skips transform also for chroma

  - skips transform for ISP

  - uses regular residual coding instead of transform skip residual coding

- disables scaling for luma and chroma (implicitly signalled)

- disables sign hiding (implicitly signalled)

- skips SBT syntax (implicitly signalled, syntax inferred as disabled)

- skips LFNST syntax (implicitly signalled, syntax inferred as disabled)

- disables deblocking (implicitly signalled)

- disables SAO  (implicitly signalled)

## Non-normative changes

In addition to that, the encoder needs to be configured as follows:

- Set maximum transform size to 32x32 (explicitly signalled)

- Disables Joint Cb Cr Residual (JCCR) coding on block level (explicitly signalled as 0)

- Disables BDPCM (not required and reported as a bug, signalled but never executed)

- Disables LMCS (explicitly signalled)

- Disables ALF (explicitly signalled)

# Proposed changes

While HEVC requires to disable scaling for lossless coding, which can be controlled via the CU-level flag cu_transquant_bypass_flag. VVC allows lossless coding of the prediction residual by skip transform and signalling an adequate QP value, e.g. 4. The other changes implicitly disable tools that can prevent lossless coding like in-loop filter depending on cu_transquant_bypass_flag. However, disabling these tools can also be done in a non-normative way, e.g. by disabling the deblocking filter.

Consequently, allowing transform skip for chroma as tested in CE8-2.1 already enables lossless coding in VVC and explicitly disabling the tools critical to lossless. The following table summarizes the changes to the current VVC draft introduced by the AHG18 lossless anchor and the proposed minimal lossless design.

| TOOL | AHG18 lossless anchor | Change | Proposed | Change |
| --- | --- | --- | --- | --- |
| SBT off | implicitly (transquant bypass) | yes | explicitly | no |
| LFNST off | implicitly (transquant bypass) | yes | explicitly | no |
| skip trafo Y | implicitly (transquant bypass) | yes | explicitly | no |
| skip trafo Cb | implicitly (transquant bypass) | yes | explicitly | yes |
| skip trafo Cr | implicitly (transquant bypass) | yes | explicitly | yes |
| skip trafo for ISP | implicitly (transquant bypass) | yes | no | no |
| BDPCM off | explicitly | no | explicitly | no |
| residual coding | regular | yes | transform skip (TSRC) | no |
| scaling off | implicitly (transquant bypass) | yes | implicitly (QP < 5) | no |
| JCCR off | explicitly | no | explicitly | no |
| SDH off | implicitly (transquant bypass) | yes | implicit (TSRC) | no |
| MaxTrafoSize=32x32 | explicitly | no | explicitly | no |
| LMCS off | explicitly | no | explicitly | no |
| DF off | implicitly (transquant bypass) | yes | explicitly | no |
| SAO off | implicitly (transquant bypass) | yes | explicitly | no |
| ALF off | explicitly | no | explicitly | no |

In top of that, a modified transform skip residual coding is proposed to increase the coding efficiency in lossless coding.

Both the necessary change and the change to improve coding efficiency are further described in the following.

## Necessary change to enable lossless (CE8-2.1)

In order to be able to skip the transform for chroma as well, two additional transform_skip_flag syntax elements are introduced as proposed and tested in CE8-2.1 for 4:4:4 (JVET-P0058) In summary this includes:

- Signal transform_skip_flag per component:

  - Additional single context model for chroma TS

  - No changes of both context model and binarization for MTS signaling

- Interaction with TS residual coding

  - TS residual coding is straightforwardly used for chroma TS as well as luma TS case.

  - No additional context models are added for the chroma TS residual coding stage

The following tools are implicitly disabled by the transform skip flag being one:

- MTS

- LFNST

- DepQuant

The following encoder only changes are required and triggered by the parameter CostMode set to lossless:

- Set maximum transform size to 32x32 (explicitly signalled)

- Disables Joint Cb Cr Residual (JCCR) coding on block level (explicitly signalled as 0)

- Disables Quantization/Scaling (explicitly signalled via QP=4 (8bit), QP=-12 (10bit)

- Disables LMCS (explicitly signalled)

- Disables DF (explicitly signalled)

- Disables SAO (explicitly signalled)

- Disables ALF (explicitly signalled)

- Disables RDOQ

- InternalBitDepth=0 (Input=Internal Bit Depth)

## Change to improve coding efficiency

### Modified transform skip residual coding (alternative CE7-1.2d)

In order to further increase the efficiency for lossless residual coding, the modifications proposed in JVET-P0079 for CE7-1.2d and its alternative implementation have been added to the lossless transform skip from CE8-2.1. The binarization and encoding of the coefficients is modified so that after the limit of context coded bins is reached, the coefficients are coded without been split into 8 bypass bins + remainder as done in current VVC transform skip residual coding.

# Results

## Test1: Transform skip for chroma with regular residual coding

Table I: Bit-rate savings over HM lossless

|  | Over HM lossless |  | Over VTM lossless* |  |
| --- | --- | --- | --- | --- |
|  | AI | RA | AI | RA |
| Class A1 | -6.02% | -4.93% | -0.05% | -0.07% |
| Class A2 | -3.42% | -3.16% | 0.22% | 0.02% |
| Class B | -4.15% | -3.37% | 0.06% | -0.17% |
| Class C | -5.33% | -3.79% | 0.15% | -0.28% |
| Class E | -6.29% |  | 1.45% |  |
| Overall | -4.96% | -3.75% | 0.32% | -0.14% |
| Enc. Time [%] | 3302% | 1435% | 58% | 89% |
| Dec. Time [%] | 191% | 155% | 99% | 98% |
| Class D | -5.45% | -4.11% | 0.26% | -0.25% |
| Class F | -18.62% | -15.68% | -2.95% | -2.15% |
| Class TGM | -46.85% | -29.90% | -2.01% | -1.29% |

*It should be noted that the AHG18 VTM lossless anchor enables transform skip for ISP which is not the case in tests 1-4. Disabling ISP for lossless results in a 0.67% loss in AI and 0.13% in RA.

## Test2: Transform skip for chroma with transform skip residual coding

Table I: Bit-rate savings over HM lossless

|  | Over HM lossless |  | Over VTM lossless* |  |
| --- | --- | --- | --- | --- |
|  | AI | RA | AI | RA |
| Class A1 | 6.56% | 8.36% | 13.41% | 13.97% |
| Class A2 | 14.91% | 14.22% | 19.24% | 17.97% |
| Class B | 4.80% | 5.76% | 9.40% | 9.26% |
| Class C | 2.21% | 4.21% | 8.12% | 8.01% |
| Class E | -3.50% |  | 4.47% |  |
| Overall | 4.82% | 7.56% | 10.60% | 11.61% |
| Enc. Time [%] | 3689% | 1546% | 65% | 96% |
| Dec. Time [%] | 212% | 176% | 110% | 111% |
| Class D | 2.80% | 3.43% | 8.96% | 7.57% |
| Class F | -15.90% | -13.71% | 0.25% | -0.14% |
| Class TGM | -46.42% | -33.75% | -1.53% | -6.17% |

## Test3: Transform skip for chroma (CE8-2.1) with modified transform skip residual coding (CE7-1.2d)

Table I: Bit-rate savings over HM lossless

|  | Over HM lossless |  | Over VTM lossless* |  |
| --- | --- | --- | --- | --- |
|  | AI | RA | AI | RA |
| Class A1 | -5.62% | -2.63% | 0.41% | 2.37% |
| Class A2 | -2.65% | 0.14% | 1.01% | 3.43% |
| Class B | -3.87% | 0.66% | 0.34% | 3.99% |
| Class C | -5.19% | 2.03% | 0.30% | 5.75% |
| Class E | -6.47% |  | 1.25% |  |
| Overall | -4.69% | 0.26% | 0.61% | 4.02% |
| Enc. Time [%] | 3349% | 1344% | 59% | 83% |
| Dec. Time [%] | 224% | 180% | 116% | 114% |
| Class D | -6.63% | 0.97% | -1.02% | 5.01% |
| Class F | -19.62% | -15.12% | -4.22% | -1.88% |
| Class TGM | -49.33% | -35.34% | -6.78% | -8.63% |

*It should be noted that the AHG18 VTM lossless anchor enables transform skip for ISP which is not the case in tests 1-4. Disabling ISP for lossless results in a 0.67% loss in AI and 0.13% in RA.

## Test4: Transform skip for chroma (CE8-2.1) with modified transform skip residual coding (alternative CE7-1.2d)

Table I: Bit-rate savings over HM lossless

|  | Over HM lossless |  | Over VTM lossless* |  |
| --- | --- | --- | --- | --- |
|  | AI | RA | AI | RA |
| Class A1 | -6.02% | -3.47% | -0.02% | 1.48% |
| Class A2 | -3.22% | -1.49% | 0.43% | 1.74% |
| Class B | -4.40% | -0.74% | -0.21% | 2.54% |
| Class C | -5.73% | 0.05% | -0.27% | 3.70% |
| Class E | -7.39% |  |  |  |
| Overall | -5.27% | -1.23% | -0.01% | 2.48% |
| Enc. Time [%] | 3404% | 1404% | 60% | 87% |
| Dec. Time [%] | 211% | 167% | 109% | 106% |
| Class D | -6.94% | -0.88% | -1.34% | 3.08% |
| Class F | -20.14% | -16.15% | -4.83% | -3.02% |
| Class TGM | -49.38% | -36.37% | -6.86% | -9.97% |

# Draft changes

Draft text for the necessary change as tested in CE8-2.1 can be found in JVET-P0058.

Draft text for the modified transform skip residual coding as proposed in the modification of CE7-1.2d can be found in JVET-P0079.

# Patent rights declaration(s)

Fraunhofer HHI, Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).