| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0607-v2 |
| --- | --- |

| Title: | AHG18: Two Stage Residual Coding for Lossless |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Benjamin Bross, Tung Nguyen Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes two stage residual coding to improve the coding efficiency for the lossless coding use case. The proposed technique invokes the residual coding/decoding a second time to encode/decode the quantization error signal to allow for lossless coding. The proposed method provides 4.94% bit-rate reduction for AI compared to the AHG18 VTM lossless anchor and 9.95% bit-rate reduction compared to the HM lossless anchor.

# Two stage residual coding for lossless

The basic principle is shown in Figure 1. It can be seen that the residual samples (spatial for transform skip or frequency after transform) are quantized as in regular lossy coding using the QP set at the encoder. In order to enable lossless coding, the resulting quantization error is fed again into the same residual coding as the quantized residual. The two-stage approach is performed for each color component, i.e. first stage and second stage for Y, first and second for Cb and first and second for Cr. This means that, e.g. if transform skip mode is selected, the quantized spatial luma residual is coded using transform skip residual coding and after that, transform skip residual coding is invoked again for the luma quantization error. The coding of the quantization error is immediately after the coding of the regular transform block.

The encoder can disable this mode at a higher level, e.g. using a SPS / slice header flag or on a block level by setting the CBF to 0 in the second residual coding.

Figure 1 - Two stage residual coding and decoding for lossless

The results presented in the next section where generated using a base QP of 15 for 8-bit sequences and a base QP of 4 for 10-bit sequences which was empirically determined to work best over all classes of a respective bit depth. However, more advance QP selection algorithm at the encoder are expected to provide more coding efficiency improvement.

# Results

Table I: Bit-rate savings over AHG18 VTM and HM lossless anchors

|  | Over AHG18 VTM lossless |  |  |  | Over HM lossless |  |  |  | Over HM RExt lossless |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI |  | RA |  | AI |  | RA |  | AI |  | RA |
| Class A1 | -3.89% | 0.30% |  | -9.67% |  | -4.59% |  | -4.68% |  | -0.54% |  |
| Class A2 | -7.53% | -1.83% |  | -10.86% |  | -4.96% |  | -5.45% |  | -0.49% |  |
| Class B | -5.83% | -0.49% |  | -9.78% |  | -3.68% |  | -5.21% |  | -1.71% |  |
| Class C | -2.84% | -0.05% | -8.16% | -3.57% | -2.98% | -1.67% |
| Class E | -3.89% |  |  | -11.99% |  |  |  | -4.14% |  | - |  |
| Overall | -4.94% | -0.48% |  | -9.95% |  | -4.09% |  | -4.49% |  | -1.22% |  |
| Enc. Time [%] | 195% | 125% |  | 11024% |  | 2008% |  | 11715% |  | 1903% |  |
| Dec. Time [%] | 92% | 90% | 178% | 142% | 189% | 151% |
| Class D | -4.74% | -0.33% |  | -10.12% |  | -4.18% |  | -2.91% |  | -1.58% |  |
| Class F | -1.95% | 0.25% |  | -17.99% |  | -13.72% |  | -7.10% |  | -4.61% |  |
| Class TGM | 4.93% | 1.71% |  | -43.19% |  | -27.77% |  | -26.39% |  | -8.59% |  |

# Conclusion

The results show that a two-stage residual coding approach can provide coding efficiency gains for lossless. Aspects for further study include an encoder algorithm to select the QP that is optimal in terms of bit-rate reduction and modifications of the second residual coding stage.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).