| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0614-v1 |
| --- | --- |

| Title: | Non-CE5: On the gridsize for virtual boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | bray@qti.qualcomm.com geertv@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to modify the gridsizes for virtual boundaries from 8x8 to 4x4, in order to be aligned with the 4x4 deblocking process.

# Introduction

In the last Geneva meeting, signalling of virtual boundaries in PPS level was adopted (JVET-N0438) [2]. The main motivation is to disable application of loop filtering across discontinuous edges for 360 videos (for example, for cubemap projection the boundary between two cubemap faces). The grid size for virtual boundaries is set to 8x8 in luma samples [1].

Fig 1. An example of cubemap projected 360 image.

However, in the last Gothenburg meeting, the 4x4 luma deblocking filter was adopted. With this adoption, now if a discontinuous edge occurs at 4x4 grid (but not at 8x8 grid), there is no way to disable deblocking across that edge, as the virtual boundaries are defined in 8x8 grid.

# Proposal

We propose to modify the virtual boundary gridsize from 8x8 to 4x4, to align with the deblocking gridsize.

# References

[1] Bross et al., “Versatile Video Coding (Draft 6),” Joint Video Experts Team, doc. JVET-O2001, Gothenburg, SE, Jul. 2019.

[2] Lin et al., “AHG12: Loop filter disabled across virtual boundaries”, Document of Joint Video Experts Team, JVET-N1010, Geneva, CH, Mar. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).