| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0617-v1 |
| --- | --- |

| Title: | Non-CE4: On TPM merge mode in the presence of weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Filippov, V. Rufitskiy | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

In this proposal, a mechanism to provide slice-level weighting for triangle prediction is proposed. Parameters of weighted prediction and TPM are combined prior the weighting process in order to minimize the number of per-sample operations and reduce the number of steps required to calculate predicted samples.

# Introduction

From [1], it is known that TPM is not available for a PU that uses slice-level weighted prediction (WP). PU is predicted either using WP weighting or using triangle merge mode (TPM).

For slice-level weighted prediction, each element of the reference picture list that is signalled on the slice level has a pair of weight and offset assigned for the referenced picture. In this case, a block determines weights and offsets for the linear combination with respect to the pair of reference indices that indicate a pair of reference picture list elements, and hence, a set of parameters for linear combination (including weights and offsets). Hence, predicted sample f is determined as follows:

,

wherein pi is a sample of a reference block i, {wi, Oi} are weight and offset specified for an element of ith reference picture list, parameter WD is the bit depth of weights wi.

In TPM, weights applied to the two samples being of the reference blocks depend on the position (x,y) of a predicted sample within a predicted block. In particular, predicted sample p is determined as follows:

,

wherein the sum of weights{,} is constant, i.e. , and the weights’ values are normalized to the weight bitdepth s.

# Proposed method

It is proposed to combine WP and triangle merge mode PU prediction (see Fig. 1). Specifically, it is proposed to use single weighting operation in this harmonized TPM/WP prediction.

Figure 1. Combination of WP and triangle merge mode PU prediction

Thus, obtaining a predicted sample is done as follows:

In this equation pi is a sample of a reference block i, {wi, Oi} are weight and offset specified for an element of ith reference picture list, parameter WD is the bit depth of weights wi.

It could be noticed, that calculation of  and  could be performed on the slice level and so that the result of multiplication could be fetched from a LUT.

Just as in a regular WP, clipping operation is performed for the result of the prediction, but not for the intermediate result of motion compensation for each of the reference block. Hence, the proposed method do not extend the number of operations to perform TPM with WP. As compared to the VVC specification draft [2], the proposed method introduces the following modifications highlighted by yellow:

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesLA and predSamplesLB,

- a variable triangleDir specifying the partition direction,

- a variable cIdx specifying colour component index.

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

The variable nCbR is derived as follows:

nCbR = ( nCbW > nCbH )  ?  ( nCbW / nCbH )  :  ( nCbH / nCbW )

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

Variables shift1 and offset1 are derived as follows:

–	The variable shift1 is set equal to Max( 5, 17 − bitDepth).

–	The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

Depending on the values of triangleDir, wS and cIdx, the prediction samples pbSamples[ x ][ y ] with x = 0..nCbW − 1 and y = 0..nCbH − 1 are derived as follows:

–	The variable wIdx is derived as follows:

- If cIdx is equal to 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( x / nCbR − y ) + 4 ) )	
	: ( Clip3( 0, 8, ( x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is equal to 0 and triangleDir is equal to 1, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( nCbH − 1 − x / nCbR − y ) + 4 ) ) 
		( Clip3( 0, 8, ( nCbW − 1 − x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is greater than 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( x / nCbR − y ) + 2 ) ) 
	: ( Clip3( 0, 4, ( x − y / nCbR ) + 2 ) )

- Otherwise (if cIdx is greater than 0 and triangleDir is equal to 1), the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( nCbH − 1 − x / nCbR − y ) + 2 ) ) 
		( Clip3( 0, 4, ( nCbW − 1 − x − y / nCbR ) + 2 ) )

–	The variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows:

wValue = ( cIdx = = 0 )  ?  Clip3( 0, 8, wIdx )  :  Clip3( 0, 8, wIdx * 2 )

–	The values of w0, w1, o0 and o1 are defined as follows

- If cIdx is equal to 0 for luma samples, the following applies:

log2Wd = luma_log2_weight_denom + shift1

w0 = (triangleDir ==0) ? LumaWeightL0[ refIdxL0 ]	 : LumaWeightL1[ refIdxL1 ]

w1 = (triangleDir ==0) ? LumaWeightL1[ refIdxL1 ] : LumaWeightL0[ refIdxL0 ]

o0 = ((triangleDir ==0) ? luma_offset_l0[ refIdxL0 ] : luma_offset_l1[ refIdxL1 ] ) <<  ( BitDepthY − 8 )

o1 = ((triangleDir ==0) ? luma_offset_l1[ refIdxL1 ] : luma_offset_l0[ refIdxL0 ] ) <<  ( BitDepthY − 8 )

- Otherwise (cIdx is not equal to 0 for chroma samples), the following applies:

log2Wd = ChromaLog2WeightDenom + shift1

cIdxM1= cIdx-1

w0 = (triangleDir ==0) ? ChromaWeightL0[ refIdxL0 ][cIdxM1]: ChromaWeightL1[ refIdxL1 ] [ cIdxM1]

w1 = (triangleDir ==0) ? ChromaWeightL1[ refIdxL1 ][cIdxM1]: ChromaWeightL0[ refIdxL0 ] [ cIdxM1]

o0 = ((triangleDir ==0) ? ChromaOffsetL0 [ refIdxL0 ] : ChromaOffsetL1 [ refIdxL1 ] )

o1 = ((triangleDir ==0) ? ChromaOffsetL1 [ refIdxL1 ] : ChromaOffsetL0 [ refIdxL0 ] )

o0 = o0 << ( BitDepthC − 8 )

o0 = o1 << ( BitDepthC − 8 )

–	The offset value is derived as follows:

offsetBi = (((triangleDir ==0) ? o1 : o0)*wValue + ((triangleDir ==0) ? o0 : o1)* (8- wValue))

offsetBi = offsetBi >> (3 - 1)

–	The joint weight wBi0 is derived as follows:

wBi0 = (triangleDir ==0) ? w1 : w0)*wValue

wBi1 = (triangleDir ==0) ? w0 : w1)*(8-wValue)

–	The prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1,
	( predSamplesL0[ x ][ y ] * wBi0 + predSamplesL1[ x ][ y ] * wBi1 +
	offsetBi )  >>  (3+ log2Wd + 1 ) )

# Experimental result

All the tests are based on the VTM6.0 software. The further part of this section describes test conditions specified for this test and reports test results for the tests.

## Test Conditions

In our experiments, we use same conditions that was used for fade-in fade-out simulations in AVC [4], HEVC [5] and VVC [3]: a linear fade is applied to the first 2 seconds of the regular sequence set and the experiments are done with the 2-second sequences. A linear fade-out (sequence to black or white) is applied in the 1st half and a linear fade-in (respectively black or white to sequence) in the 2nd half (Figure 2). The fade strength is limited to the range [0.25, 0.75] instead of [0, 1] to avoid extreme PSNR numbers at the frame level that cause abnormal rate-distortion graphs.

In order to simplify simulations and avoid redefining configuration parameters of input CTC sequences, weighting of original and setting up then number of coded frames is implemented within proposed modifications to the VTM software.

Figure 2: Linear Fade-to-black or Fade-to-white weights applied on the sequence and the black/white pictures respectively [3].

In all the tests, macro JVET_O0280_SIMD_TRIANGLE_WEIGHTING is set to zero, since the proposal has only a non-SIMD implementation of the proposed method.

## Test description

Test descriptions are summarized in Table 1. Test 1 confirms that refactoring does not affect the results obtained. Bit streams of Test 1 are reported to exactly those produced by the VTM-6.0 software. Results for Tests 2-5 are reportedly show in Tables 2-8 for the two fading scenarios.

In both scenarios a frame of original picture is linearly blended with the “all-black” (scenario 1, Tables 2-4) or “all-white” (scenario 2, Tables 5-8) frame.

Table 1. Tests description

| Test number | Test | Anchor | Test conditions |
| --- | --- | --- | --- |
| Test 1 | Verification of refactoring part. Results are the same as for VTM6.0 | VTM6.0 | CTC [2], full-length |
| Test 2 | TPM tool-off test Weighted prediction is on: --WeightedPredB=1 --WeightedPredP=1 --Triangle=0 | Proposed method is disabled. Weighted prediction is on: --WeightedPredB=1 --WeightedPredP=1 | About of 2-seconds length, fadein/fadeout sequences |
| Test 3 | Test 2 + proposed method |  |  |
| Test 4 | TPM-off test without WP for fade-in/fade-out sequences --Triangle=0. Weighted prediction is off. | Proposed method is disabled. Weighted prediction is off. |  |

## The simulation results for the tests of Table 1 carried out for fade-to-black sequences are reportedly presented in Tables 2-4.

Table 2. Simulation results for Test 2 on fade-to-black sequences

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.44% | 0.64% | 1.05% | 95% | 96% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.12% | 0.70% | -0.11% | 96% | 97% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay B |  |  |  |  |
|  | Over VTM6.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.22% | 0.81% | 1.04% | 95% | 98% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 3. Simulation results for Test 3 on fade-to-black sequences.

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -1.01% | -1.00% | -0.95% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.81% | -0.68% | -1.20% | 98% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay B |  |  |  |  |
|  | Over VTM6.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.34% | -0.63% | -0.76% | 100% | 98% |
| Class E | -0.11% | 0.20% | -0.50% | 99% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.28% | 0.55% | 0.06% | 100% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 4. Simulation results for Test 4 on fade-to-black sequences

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.62% | 0.53% | 0.69% | 96% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.39% | -0.13% | 0.25% | 96% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay B |  |  |  |  |
|  | Over VTM6.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.83% | 1.07% | 0.64% | 93% | 99% |
| Class E | 0.55% | 0.14% | 0.53% | 91% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.94% | 0.14% | 1.08% | 92% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

## The simulation results for the tests of Table 1 carried out for fade-to-white sequences are reportedly presented in Tables 5-7.

Table 5. Simulation results for Test 2 on fade-to-white sequences.

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.39% | 0.41% | 0.42% | 96% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.06% | 0.27% | -0.65% | 96% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay B |  |  |  |  |
|  | Over VTM6.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.42% | 0.57% | 0.26% | 96% | 100% |
| Class E | 0.05% | 1.60% | -0.62% | 96% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.20% | 0.41% | -0.36% | 96% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 6. Simulation results for Test 3 on fade-to-white sequences.

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -1.07% | -1.74% | -1.34% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.69% | -0.89% | -1.24% | 99% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay B |  |  |  |  |
|  | Over VTM6.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.42% | -1.37% | -1.34% | 100% | 98% |
| Class E | -0.25% | -0.16% | -0.50% | 100% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.30% | 0.03% | -1.14% | 100% | 98% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 6. Simulation results for Test 4 on fade-to-white sequences.

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.66% | 0.67% | 0.54% | 95% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.16% | 0.23% | -0.39% | 95% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay B |  |  |  |  |
|  | Over VTM6.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.81% | 1.08% | 0.59% | 92% | 100% |
| Class E | 0.41% | -0.02% | -0.38% | 91% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.79% | 1.71% | 0.63% | 92% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusion

It is proposed to adopt the suggested modification to the VVC specification draft and to the VTM Software.

# References

- B. Bross, J. Chen, S. Liu., Versatile Video Coding (Draft 6), “Versatile Video Coding (Draft 6)”, JVET-O2001, 15th JVET Meeting, Gothenburg, Sweden, July 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET Meeting, Geneva, Switzerland, March 2019.

- P. Bordes, E. François, F. Léannec “AHG13 - Weighted Prediction vs Generalized Bi-prediction with Fade sequences”, JVET-L0201, 12th JVET Meeting, Macao, China, October 2018.

- Yoshihiro Kikuchi and Takeshi Chujoh,” Simplification of the Weighted Prediction and Verification result,” JVT Document, JVT-F077, Awaji, December, 2002.

- A.Fujibayashi, S.Kanumuri and F.Bossen,” TE12: Performance of Partition Based Illumination Compensation (PBIC),” JVT Document, JCTVC-C041, Guangzhou, October, 2010.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).