| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0624 |
| --- | --- |

| Title: | AHG16: Bugfix for local dual tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xiang Li, Xin Zhao, Shan Liu | Tel: Email: | {leolzhao, xlxiangli, xinzzhao, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In VVC Draft 6, a coding tree node in an inter coding region of local dual tree can split into 4x4 coding units (CU). However, in this case, neither Intra (including IBC) nor Inter coding mode is allowed for these coding tree, because 4x4 blocks are not allowed to use Inter mode and CUs in Inter coding region cannot use Intra mode. In this contribution, two alternative methods are proposed to fix this issue. In the first method, the tree depth within local dual tree region is restricted to be no larger than 1. In the second method, the tree depth for inter coding region in local dual tree is restricted to be no larger than 1. It is reported that the first method achieves 0.09%, 0.01%, and -0.02% luma BD-rate changes for AI, RA and LDB configurations, respectively, and the second method achieves 0.00% and -0.02% luma BD-rate changes for RA and LDB configurations, respectively.

Introduction

In VTM6.0, Small Chroma Intra Prediction Unit (SCIPU) was adopted [1]. SCIPU aims to disallow chroma intra CBs smaller than 16 chroma samples by creating a local dual tree in single tree. Before entering local dual tree, chroma partitions has the same split with the associated luma partitions. Every CU has the flexibility to decide whether to be intra coded, inter coded or IBC coded. After entering local dual tree, all the CUs should be inter coded or non-inter coded (intra or IBC). When current CU is inter coded, chroma component also follows the same split with luma CU. Otherwise, when current CU is non-inter coded, luma CU can be further split whereas the associated chroma intra CU cannot be further split. The condition for determining whether current CU is the parent node of local dual tree or not is listed below:

- CU size is 64 (8x8, 4x16, 16x4 CUs) and current CU is further QT split

- CU size is 64 (8x8, 4x16, 16x4 CUs) and current CU is further TT split

- CU size is 32 (4x8, 8x4 CUs) and current CU is further BT split

- CU size is 64 (8x8, 4x16, 16x4 CUs) and current CU is further BT split

- CU size is 128 (4x32, 32x4, 16x8, 8x16) and current CU is further TT split

When one of the above conditions is true, current CU is marked as the parent node of local dual tree. In addition, when condition a), b) or c) is satisfied, the CUs within this area is constrained to be all intra coded or all IBC coded. When condition d) or e) is satisfied, one more flag needs to be signaled to indicate whether all the CUs within this area is inter-coding region or non-inter coding region. In other words, only when condition d) or e) is satisfied, this region in local dual tree can be inter coding region.

One issue related to local dual tree is that a coding tree node in an inter coding region can be split into 4x4 coding units (CU), wherein neither Intra (including IBC) nor Inter coding mode is allowed for these coding tree, this is because 4x4 blocks cannot use Inter mode and CUs in inter coding region cannot use Intra mode.

Proposed method

Method #1:

It is observed that it will not generate 4x4 inter CUs if the coding tree nodes within local dual tree can only split once. Therefore, it is proposed that the tree depth in local dual tree is restricted to be no larger than 1.

Method #2:

It is proposed that the tree depth for inter coding region in local dual tree is restricted to be no larger than 1.

Experimental results

The proposed methods in this contribution were implemented on top of VTM6.0 software. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-N1010 [2]. Two tests are conducted to evaluate the performance of the proposed method. Since local dual tree only affect single tree case, dual tree is turned off for I-slice under all intra configuration. RA and LDB follow the CTC conditions. For method #2, it does not affect results under all intra configuration both for single tree and dual tree case, so only RA and LDB results are provided for method #2.

Table 3-1: Results of the method #1 over VTM-6.0.

|  | All Intra Main 10 ( Dual tree off ) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.03% | 99% | 100% |
| Class A2 | 0.01% | 0.06% | 0.03% | 98% | 99% |
| Class B | 0.05% | 0.10% | -0.06% | 97% | 100% |
| Class C | 0.27% | 0.23% | 0.18% | 97% | 99% |
| Class E | 0.07% | 0.02% | 0.13% | 96% | 99% |
| Overall | 0.09% | 0.10% | 0.06% | 97% | 100% |
| Class D | 0.40% | 0.16% | 0.31% | 98% | 98% |
| Class F | 0.44% | 0.41% | 0.28% | 98% | 99% |

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.02% | -0.07% | 101% | 101% |
| Class A2 | 0.04% | 0.06% | 0.03% | 100% | 100% |
| Class B | -0.01% | 0.01% | -0.07% | 100% | 100% |
| Class C | 0.02% | -0.03% | -0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | -0.06% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.11% | 99% | 101% |
| Class F | -0.01% | 0.01% | -0.01% | 100% | 101% |

|  | Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.02% | -0.15% | 100% | 99% |
| Class C | -0.02% | 0.05% | 0.11% | 100% | 100% |
| Class E | -0.02% | 0.43% | 0.11% | 100% | 98% |
| Overall | -0.02% | 0.12% | 0.00% | 100% | 99% |
| Class D | -0.07% | 0.10% | 0.51% | 99% | 101% |
| Class F | 0.10% | -0.03% | 0.41% | 100% | 102% |

Table 3-2: Results of the method #2 over VTM-6.0.

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.09% | -0.04% | 101% | 101% |
| Class A2 | 0.02% | 0.03% | 0.08% | 101% | 100% |
| Class B | -0.01% | 0.12% | 0.08% | 100% | 100% |
| Class C | -0.03% | -0.12% | -0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.01% | 100% | 100% |
| Class D | -0.05% | -0.03% | 0.02% | 100% | 101% |
| Class F | -0.01% | -0.01% | -0.04% | 100% | 100% |

|  | Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.03% | -0.09% | 100% | 99% |
| Class C | -0.05% | -0.17% | -0.15% | 100% | 99% |
| Class E | -0.01% | 0.36% | 0.19% | 100% | 99% |
| Overall | -0.02% | 0.02% | -0.04% | 100% | 99% |
| Class D | -0.06% | -0.21% | -0.21% | 99% | 100% |
| Class F | 0.02% | 0.37% | 0.47% | 100% | 100% |

Spec text

The following proposed text modifications of the proposed method are based on JVET-O2001-vE. The added texts are highlighted in green, and deleted texts are marked by red strikethrough.

Spec text for method #1

| coding_tree( x0, y0, cbWidth, cbHeight, qgOnY, qgOnC, cbSubdiv, cqtDepth, mttDepth, depthOffset, partIdx, treeTypeCurr, modeTypeCurr ) { | Descriptor |
| --- | --- |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ¦ ¦ allowSplitQT ) &&( x0 + cbWidth <= pic_width_in_luma_samples ) && (y0 + cbHeight <= pic_height_in_luma_samples ) && modeTypeCurr = = MODE_TYPE_ALL ) |  |
| split_cu_flag | ae(v) |
| if( cu_qp_delta_enabled_flag && qgOnY && cbSubdiv <= cu_qp_delta_subdiv ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| if( cu_chroma_qp_offset_enabled_flag && qgOnC && cbSubdiv <= cu_chroma_qp_offset_subdiv ) |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| if( split_cu_flag ) { |  |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ) && allowSplitQT ) |  |
| split_qt_flag | ae(v) |
| if( !split_qt_flag ) { |  |
| if( ( allowSplitBtHor ¦ ¦ allowSplitTtHor ) && ( allowSplitBtVer ¦ ¦ allowSplitTtVer ) ) |  |
| mtt_split_cu_vertical_flag | ae(v) |
| if( ( allowSplitBtVer && allowSplitTtVer && mtt_split_cu_vertical_flag ) ¦ ¦ ( allowSplitBtHor && allowSplitTtHor && !mtt_split_cu_vertical_flag ) ) |  |
| mtt_split_cu_binary_flag | ae(v) |
| } |  |
| if( modeTypeCondition = = 1 ) |  |
| modeType = MODE_TYPE_INTRA |  |
| else if( modeTypeCondition = = 2 ) { |  |
| mode_constraint_flag | ae(v) |
| modeType = mode_constraint_flag ? MODE_TYPE_INTRA : MODE_TYPE_INTER |  |
| } else { |  |
| modeType = modeTypeCurr |  |
| } |  |
| treeType = ( modeType = = MODE_TYPE_INTRA ) ? DUAL_TREE_LUMA : treeTypeCurr |  |

Spec text for method #2

| coding_tree( x0, y0, cbWidth, cbHeight, qgOnY, qgOnC, cbSubdiv, cqtDepth, mttDepth, depthOffset, partIdx, treeTypeCurr, modeTypeCurr ) { | Descriptor |
| --- | --- |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ¦ ¦ allowSplitQT ) &&( x0 + cbWidth <= pic_width_in_luma_samples ) && (y0 + cbHeight <= pic_height_in_luma_samples ) && modeTypeCurr ! = MODE_TYPE_INTER ) |  |
| split_cu_flag | ae(v) |
| if( cu_qp_delta_enabled_flag && qgOnY && cbSubdiv <= cu_qp_delta_subdiv ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| if( cu_chroma_qp_offset_enabled_flag && qgOnC && cbSubdiv <= cu_chroma_qp_offset_subdiv ) |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| if( split_cu_flag ) { |  |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ) && allowSplitQT ) |  |
| split_qt_flag | ae(v) |
| if( !split_qt_flag ) { |  |
| if( ( allowSplitBtHor ¦ ¦ allowSplitTtHor ) && ( allowSplitBtVer ¦ ¦ allowSplitTtVer ) ) |  |
| mtt_split_cu_vertical_flag | ae(v) |
| if( ( allowSplitBtVer && allowSplitTtVer && mtt_split_cu_vertical_flag ) ¦ ¦ ( allowSplitBtHor && allowSplitTtHor && !mtt_split_cu_vertical_flag ) ) |  |
| mtt_split_cu_binary_flag | ae(v) |
| } |  |
| if( modeTypeCondition = = 1 ) |  |
| modeType = MODE_TYPE_INTRA |  |
| else if( modeTypeCondition = = 2 ) { |  |
| mode_constraint_flag | ae(v) |
| modeType = mode_constraint_flag ? MODE_TYPE_INTRA : MODE_TYPE_INTER |  |
| } else { |  |
| modeType = modeTypeCurr |  |
| } |  |
| treeType = ( modeType = = MODE_TYPE_INTRA ) ? DUAL_TREE_LUMA : treeTypeCurr |  |

Conclusion

This contribution provides two methods to fix the bug for local dual tree. It is suggested to adopt the proposed method.

References

- Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei, Yin Zhao, Haitao Yang, “CE3-2.1.1 and CE3-2.1.2: Removing 2x2, 2x4, and 4x2 chroma CBs,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-O0050, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).