| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0628 |
| --- | --- |

| Title: | CE-1-2.1 Motion compensation interpolation filters for ARC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala Madhu P Krishnan Wei Dai | Email: | pankajtva@gmail.com |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

ARC has received increased attention in the past three meeting cycles, and numerous documents consider ARC for VVC. This contribution reports the results of core experiment 1.2-1 where existing motion compensation interpolation filter are replaced by FastVDO filters when the reference picture resolution is larger than the current picture. Filters are proposed for 1.5x, 2x and 4x scaling ratios. BDRATE gains of 2.52% and 0.03% ( for PSNR2); 4.72% and 1.03% (for PSNR1) are achieved for luma channels of Class C and B sequences respectively for 2x scaling ratio. BDRATE gains of 1.33% and 1.50% ( for PSNR1); 0.72% and 4.65% (for PSNR2) are achieved for luma channels of Class C and F sequences respectively for 1.5x scaling ratio using the new filters

# Introduction

The concept of ARC was originally part of H.263 Annex P [1] , and was introduced in concept for HEVC by JCTVC-F158 [2]; the acronym “ARC” itself, which at the time stood for Adaptive Resolution Coding (where “Coding” is now “Change”), was actually first used in [3]. JCTVC-F158 discussed the utility of adapting resolution within a single stream without introducing an IDR frame.

Our vision for ARC (or reference picture resampling) functionality is to define resampling of both reference pictures and source pictures using a single set of co-designed resampling filters, to allow for high quality prediction. This was initially intended to be outside the MEC process. The ARC functionality can also be embedded within the interpolation process, as it is currently set up in the CE1 branch software. But these two approaches to ARC are incompatible, and the current approach requires an interpolation filter instead of a resampling filter.

In this contribution FastVDO details: a)  phased versions of our interpolation filters (from JVT-V030 [4], cited in our proposal JVET-O0319 [5]).

## Description of filters

In Tables 1,2 & 3 filters with 16 phases for 4x, 2x and 1.5x scaling ratios are listed respectively.

Table 1: Coefficients for 4:1 reference downsampling

| Fractional sample position p | coefficients for 4:1 downsampling |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 | p4 | p5 | p6 | p7 |
| 0 | 1 | 8 | 14 | 16 | 14 | 9 | 1 | 1 |
| 1 | 1 | 8 | 14 | 15 | 15 | 9 | 1 | 1 |
| 2 | 1 | 7 | 14 | 15 | 15 | 9 | 2 | 1 |
| 3 | 1 | 7 | 13 | 15 | 15 | 10 | 2 | 1 |
| 4 | 1 | 6 | 13 | 15 | 15 | 10 | 2 | 2 |
| 5 | 0 | 5 | 13 | 15 | 15 | 11 | 3 | 2 |
| 6 | 1 | 5 | 12 | 15 | 14 | 11 | 3 | 3 |
| 7 | 1 | 4 | 12 | 15 | 15 | 11 | 3 | 3 |
| 8 | 1 | 4 | 12 | 15 | 15 | 12 | 4 | 1 |
| 9 | 3 | 3 | 11 | 15 | 15 | 12 | 4 | 1 |
| 10 | 3 | 3 | 11 | 14 | 15 | 12 | 5 | 1 |
| 11 | 2 | 3 | 11 | 15 | 15 | 13 | 5 | 0 |
| 12 | 2 | 2 | 10 | 15 | 15 | 13 | 6 | 1 |
| 13 | 1 | 2 | 10 | 15 | 15 | 13 | 7 | 1 |
| 14 | 1 | 2 | 9 | 15 | 15 | 14 | 7 | 1 |
| 15 | 1 | 1 | 9 | 15 | 15 | 14 | 8 | 1 |

Table 2: Coefficients for 2:1 downsampling

| Fractional sample position p | coefficients for 2:1 downsampling |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 | p4 | p5 | p6 | p7 |
| 0 | -2 | 3 | 18 | 29 | 18 | 3 | -2 | -2 |
| 1 | -2 | 2 | 17 | 29 | 19 | 3 | -2 | -2 |
| 2 | -2 | 2 | 16 | 28 | 20 | 4 | -2 | -2 |
| 3 | -2 | 1 | 15 | 28 | 21 | 5 | -2 | -2 |
| 4 | -2 | 1 | 14 | 28 | 22 | 6 | -2 | -2 |
| 5 | -2 | 0 | 12 | 27 | 23 | 6 | -2 | -2 |
| 6 | -2 | 0 | 11 | 27 | 24 | 7 | -2 | -2 |
| 7 | -1 | -1 | 10 | 26 | 25 | 8 | -1 | -2 |
| 8 | -1 | -1 | 9 | 25 | 25 | 9 | -1 | -1 |
| 9 | -1 | -1 | 8 | 24 | 26 | 10 | -1 | -1 |
| 10 | -1 | -2 | 7 | 24 | 26 | 11 | 0 | -1 |
| 11 | -2 | -2 | 6 | 23 | 27 | 12 | 0 | -2 |
| 12 | -2 | -2 | 6 | 22 | 28 | 14 | 1 | -2 |
| 13 | -2 | -2 | 5 | 21 | 28 | 15 | 1 | -2 |
| 14 | -2 | -2 | 4 | 20 | 28 | 16 | 2 | -2 |
| 15 | -2 | -2 | 3 | 19 | 29 | 17 | 2 | -2 |

Table 3: Coefficients for 1.5:1 downsampling

| Fractional sample position p | coefficients for 1.5:1 downsampling |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 | p4 | p5 | p6 | p7 |
| 0 | 0 | -4 | 17 | 37 | 17 | -4 | 0 | 1 |
| 1 | 1 | -4 | 15 | 37 | 19 | -4 | 0 | 0 |
| 2 | 1 | -4 | 13 | 37 | 21 | -3 | -1 | 0 |
| 3 | 1 | -4 | 11 | 37 | 24 | -3 | -1 | -1 |
| 4 | 1 | -4 | 9 | 37 | 25 | -2 | -1 | -1 |
| 5 | 1 | -4 | 7 | 36 | 28 | -1 | -2 | -1 |
| 6 | 1 | -4 | 5 | 36 | 30 | 0 | -2 | -2 |
| 7 | 1 | -4 | 4 | 35 | 32 | 1 | -3 | -2 |
| 8 | 1 | -3 | 2 | 32 | 32 | 2 | -3 | 1 |
| 9 | -2 | -3 | 1 | 32 | 35 | 4 | -4 | 1 |
| 10 | -2 | -2 | 0 | 30 | 36 | 5 | -4 | 1 |
| 11 | -1 | -2 | -1 | 28 | 36 | 7 | -4 | 1 |
| 12 | -1 | -1 | -2 | 25 | 37 | 9 | -4 | 1 |
| 13 | -1 | -1 | -3 | 24 | 37 | 11 | -4 | 1 |
| 14 | 0 | -1 | -3 | 21 | 37 | 13 | -4 | 1 |
| 15 | 0 | 0 | -4 | 19 | 37 | 15 | -4 | 1 |

## Simulation Results

The filters were implemented on VTM-6.1 modified with the affine patch. Common test conditions (CTC) detailed in JVET-O2021 [6] were used. Tables 4,5,6 & 7 lists the BDRATE summary results (PSNR1, PSNR2) for Class B, C & E sequences for 2x and 1.5x scaling ratios. From the tables it is evident that BDRATE gains of 2.52% and 0.03% ( for PSNR2); 4.72% and 1.03% (for PSNR1) can be achieved for luma channels of Class C and B sequences respectively for 2x scaling ratio using the new filters. Meanwhile, BDRATE gains of 1.33% and 1.50% ( for PSNR1); 0.72% and 4.65% (for PSNR2) can be achieved for luma channels of Class C and F sequences respectively for 1.5x scaling ratio using the new filters.

Table 4: PSNR2 for 2x scaling ratio

Table 5: PSNR1 for 2x scaling ratio

Table 6: PSNR1 for 1.5x scaling ratio

Table 7: PSNR2 for 1.5x scaling ratio

# Patent rights declaration(s)

FastVDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] ITU-T H.263 (01/2005): Video coding for low bit rate communication, Annex O “Temporal, SNR and Spatial Scalability mode” and Annex P “Reference Picture Resampling”

[2] JCTVC-F158, “Resolution switching for coding efficiency and resilience”, T. Davies (Cisco), JCTVC 6th Meeting, Turin, IT, 14-22 July 2011

[3] JCTVC-G264, “AHG18: Adaptive Resolution Coding,” T. Davies (Cisco), P. Topiwala (FastVDO), JCT-VC 7th Meeting, Geneva, CH,  21-30 November, 2011.

[4] JVT-V030, “Advanced Extended Spatial Re-sampling Filters For SVC”, Trac D. Tran, L. Liu, P. Topiwala (FastVDO), JVT 22nd Meeting: Marrakech, Morocco, 13–19 January, 2007

[5] JVET-O0319, “Adaptive resolution change (ARC) in VVC”, W. Dai, M.Krihnan, P. Topiwala (FastVDO), JVET 15th Meeting, Gothenburg, SE, 3–12 July 2019.

[6] JVET-O2021, “Description of Core Experiment 1 (CE1): Reference picture resampling filters”, D. Luo, V. Seregin, W. Wan, JVET 15th Meeting, Gothenburg, SE, 3–12 July 2019.