| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0639 |
| --- | --- |

| Title: | CE5: Crosscheck of JVET-P0045 (CE5-3.3: Signalling EO signs in SAO) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET or Output document approved by JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, A.M. Kotra Riesstr. 25, Munich, 80992,Germany | Tel: Email: | +491624302163 elena.alshina@huawei.com anand.meher.kotra@huawei.com |
| Source: | Huawei Technologies Düsseldorf, GmbH |  |  |

_____________________________

# Abstract

Performance and algorithm tested in CE5-3.3 is verified in this report. Visual quality improvement examples provided by proponent have been investigated and quality improvement is confirmed. Additionally SAO performance was evaluated in different scenarios tested in CE5. Supplementary test results for EO with sign coded as equal probability are provided. Report concludes in target use case (real-time encoder) SAO provide substantial objective and subjective performance benefits in VTM6.0. This justifies inclusion SAO inti VVC standard.

# Introduction

Edge Offset (EO) in HEVC SAO is disallowed to have sign. So only smoothing effect can be provided by EO. Contribution which is cross-checked in this report investigates possibilities of usage sign in EO.

Code provided perfectly matches specification in JVET-P0045. Syntax modification is straightforward, sample-processing isn’t changed.

One context model is introduced by CE-5.3-3 for EO sign encoding. In this report we alternatively tested by-pass coding for EO sign.

# Test results

Our test results perfectly match results reported by proponent of CE5-3.3. Test results are shown in Table 1. Encoder and decoder run-time deviation for anchor are within normal measurement error, both encoder and decoder run time shall be interpreted as the same with anchor.

It appears that EO sign is the most useful for Low Delay P scenario, class F (and screen content). If EO sign is coded as by-pass then -0.1% BD-rate gain for class F in LDP test is observed (over VTM6.0)

In Table 2 we present supplementary results when EO sign is coded w/o ctx (as by-pass).

Table 1. Verification test for CE5.3-3: Anchor VTM6.0, test – EO with sign coded with ctx.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.04% | 101% | 100% |
| Class A2 | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.01% | 0.00% | 0.03% | 100% | 101% |
| Class C | 0.02% | 0.01% | 0.03% | 100% | 102% |
| Class E | -0.01% | 0.00% | 0.01% | 100% | 102% |
| Overall | 0.00% | 0.01% | 0.02% | 100% | 101% |
| Class D | 0.02% | 0.04% | 0.05% | 100% | 105% |
| Class F | 0.01% | 0.01% | 0.03% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.04% | 100% | 100% |
| Class A2 | 0.03% | 0.05% | 0.06% | 100% | 100% |
| Class B | -0.01% | 0.00% | -0.03% | 100% | 100% |
| Class C | 0.00% | 0.02% | 0.04% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.00% | 100% | 100% |
| Class D | 0.01% | -0.08% | 0.00% | 101% | 103% |
| Class F | -0.02% | 0.06% | 0.03% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.11% | -0.06% | 100% | 101% |
| Class C | 0.00% | 0.04% | -0.07% | 100% | 99% |
| Class E | -0.10% | 0.06% | 0.50% | 99% | 100% |
| Overall | -0.03% | -0.02% | 0.07% | 100% | 100% |
| Class D | 0.04% | -0.09% | 0.59% | 100% | 100% |
| Class F | 0.09% | 0.28% | 0.60% | 100% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.24% | 0.11% | 100% | 100% |
| Class C | 0.06% | 0.01% | 0.17% | 100% | 99% |
| Class E | 0.00% | 0.10% | 0.75% | 99% | 100% |
| Overall | 0.02% | 0.13% | 0.29% | 100% | 100% |
| Class D | 0.06% | -0.07% | -0.53% | 100% | 99% |
| Class F | 0.00% | 0.09% | 0.15% | 99% | 100% |

Table 2. Modified for CE5.3-3 test: Anchor VTM6.0, test – EO with sign coded as EP.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% | 0.05% | 100% | 100% |
| Class A2 | 0.01% | 0.03% | 0.02% | 100% | 100% |
| Class B | 0.00% | 0.02% | 0.05% | 100% | 101% |
| Class C | 0.01% | 0.03% | 0.05% | 100% | 102% |
| Class E | -0.02% | 0.01% | 0.02% | 100% | 102% |
| Overall | 0.00% | 0.03% | 0.04% | 100% | 101% |
| Class D | 0.01% | 0.03% | 0.03% | 100% | 105% |
| Class F | 0.00% | 0.02% | 0.04% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.01% | 100% | 100% |
| Class A2 | 0.03% | 0.02% | 0.03% | 100% | 100% |
| Class B | -0.03% | 0.11% | 0.06% | 100% | 100% |
| Class C | 0.02% | 0.10% | -0.02% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.07% | 0.02% | 100% | 100% |
| Class D | -0.03% | 0.05% | 0.05% | 100% | 104% |
| Class F | -0.01% | 0.05% | 0.02% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.09% | -0.02% | 100% | 101% |
| Class C | 0.01% | 0.18% | -0.16% | 99% | 100% |
| Class E | -0.04% | 0.52% | 0.34% | 99% | 101% |
| Overall | -0.01% | 0.23% | 0.02% | 99% | 101% |
| Class D | 0.02% | -0.10% | 0.20% | 100% | 100% |
| Class F | 0.05% | 0.00% | 0.53% | 99% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.13% | 0.45% | 100% | 100% |
| Class C | 0.03% | 0.20% | -0.10% | 100% | 100% |
| Class E | 0.03% | -0.01% | 0.43% | 99% | 100% |
| Overall | 0.00% | 0.12% | 0.26% | 100% | 100% |
| Class D | 0.03% | -0.01% | -0.12% | 100% | 100% |
| Class F | -0.11% | -0.03% | 0.46% | 99% | 100% |

# On SAO performance and usage

VVC draft contains number of powerful tools all together provided impressive compression benefits over HEVC. Gain never comes for free. Not all encoders will be capable to use full range of VVC tools. For instance mobile devices, CCTV and teleconferencing operate in real-time encoding scenario and have very limited capability for encoder search.

SAO was designed to provide no more than one CTU delay on encoder side, so among all in-loop filters in VVC draft spec and consider in CE5 SAO is the least complicated and the friendliest to real-time encoders. Typical use case of SAO is LDP and ALF off scenario. Table 3 shows SAO gain in target use case.

Under common test conditions SAO gain in VVC is not substantial, but for target use case (real time encoding) performance justifies keeping SAO in VVC specification.

Table 3. Compression gain of SAO in target use case.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | SAO over VTM-6.0,SAO off,ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.60% | -1.84% | 100% | 104% |
| Class A2 | -0.48% | -0.56% | -0.44% | 100% | 105% |
| Class B | -0.29% | -0.21% | -0.42% | 100% | 104% |
| Class C | -0.30% | -0.58% | -0.92% | 100% | 103% |
| Class E | -0.38% | 0.05% | -0.01% | 100% | 103% |
| Overall | -0.32% | -0.37% | -0.70% | 100% | 104% |
| Class D | -0.21% | -0.29% | -0.45% | 101% | 102% |
| Class F | -0.97% | -0.78% | -1.09% | 100% | 103% |
|  | Random Access |  |  |  |  |
|  | SAO over VTM-6.0,SAO off,ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | -0.46% | -0.54% | 100% | 102% |
| Class A2 | -1.39% | -1.28% | -0.91% | 100% | 102% |
| Class B | -1.00% | -0.89% | -0.69% | 100% | 102% |
| Class C | -0.40% | -0.58% | -0.74% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.79% | -0.80% | -0.72% | 100% | 102% |
| Class D | 0.05% | 0.10% | 0.08% | 100% | 101% |
| Class F | -1.27% | -0.89% | -1.32% | 100% | 101% |
|  | Low delay B |  |  |  |  |
|  | SAO over VTM-6.0,SAO off,ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.99% | -1.94% | -2.65% | 99% | 103% |
| Class C | -0.97% | -2.52% | -4.46% | 100% | 103% |
| Class E | -0.87% | -0.26% | -0.91% | 100% | 101% |
| Overall | -0.95% | -1.71% | -2.82% | 100% | 102% |
| Class D | -0.15% | -1.31% | -1.37% | 100% | 103% |
| Class F | -2.47% | -3.32% | -3.74% | 99% | 102% |
|  | Low delay P |  |  |  |  |
|  | SAO over VTM-6.0,SAO off,ALF off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.12% | -3.44% | -3.55% | 99% | 103% |
| Class C | -2.08% | -3.36% | -4.84% | 100% | 103% |
| Class E | -2.36% | -2.03% | -2.23% | 100% | 102% |
| Overall | -2.59% | -3.06% | -3.65% | 100% | 102% |
| Class D | -0.89% | -1.16% | -1.44% | 100% | 103% |
| Class F | -2.62% | -3.38% | -3.83% | 99% | 103% |

# Visual quality observation

Objective coding gain provided by allowing sign of Edge Offset in SAO is negligible, but sharpening the edges is observed and visual quality is improved in some cases.

Subjective benefits SAO provided for target use case (real time encoder, LDP w/o ALF is good emulation for that) are knows from HEVC development. And still clear, Fig. 1 (a,b) illustrates visual quality benefits of SAO in target use case. Left part for all pictures is SAO on, right part – SAO off.

Also please watch SAO demo attached to this contribution, lose less compression as used to create mp4. CE4 bit streams (CE5-3_3_2_SDR_15_QP32_LDP_NO_ALF.bin and SDR_15_QP37_LDP_NO_ALF.bin) been used for this demo preparation.

Fig. 1. SAO on (left part of the picture) vs SAO off (right part of the picture): a) POC 242, b) POC 342.

# Conclusions

Under common test conditions SAO gain in VVC is not substantial, but for target use case (real time encoding) performance justifies inclusion SAO in VVC standard. Also visual quality benefits of SAO for target use case are known since HEVC development and still clear now. Allowing sing for Edge Offset provide minor objective gain but sharpening effect for several cases. If sign is allowed then it is recommended to use by-pass coding (not context coded bin).

# Patent rights declaration(s)

Huawei Technologies does not have any current or pending patent rights relating to the technology described in this contribution.