| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0641_v3 |
| --- | --- |

| Title: | Non-CE3: Combination of JVET-P0596 and JVET-P0531 on removal of 2xN chroma intra blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van, Geert Van der Auwera, Ted Hsieh, Adarsh K. Ramasubramonian, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 Zhipin Deng, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Email: | {lphamvan, geertv, thsieh, aramasub, vseregin, martak}@qti.qualcomm.com {zhipin.deng, lizhang.idm}@bytedance.com |
| Source: | Qualcomm Inc. and Bytedance Inc. |  |  |

_____________________________

# Abstract

This document presents the result of a combination of JVET-P0596 Test 2 and JVET-P0531 Test 2. In the combination test, the 2xN intra chroma blocks in dual tree are removed by disabling binary and ternary splits for 4xN and 8xN chroma partitions, respectively. In the single tree case, the regular 2xN chroma intra blocks are removed by extending the local dual tree as proposed in JVET-P0531 Test 2. For 4xN CIIP blocks, the chroma block uses only inter prediction as proposed in JVET-P0596 Test 2. The average (Y, U, V) BD-rate change is reported as follows with a negligible runtime change: (Version 2: Update the running time) (Version 3: Update the LDB results)

Dual tree off: AI: (-0.02%, -0.18%, -0.22%) with 3% encoding time reduction.

Dual tree on: AI: (0.01%, 0.35%, 0.37%), RA: (0.11%, 0.50%, 0.49%), and LDB: (0.12%, 0.30%, 0.51%).

# Introduction

Current VVC supports a flexible partitioning mechanism including quad tree, binary tree and ternary tree splits. In this split structure, the block size may range from 128 to 4 for the luma component or from 64 to 2 for the chroma components. The introduction of small block sizes, i.e., 2xN, leads to an inefficient hardware implementation, because it causes pipeline delay and requires 2xN pixels process in the hardware architecture. In most hardware implementations, 4x1 pixel per 1 CPU (or GPU) clock is used for luma and chroma. However, it is asserted that an extra 2x2 pixel per 1 clock processing is needed for 2xN blocks. This makes the hardware implementation not clean due to the 2xN block size. In addition, the memory access (reading and writing) is inefficient with 2xN, because in each access only 2x1 pixels are fetched.

In the last JVET meeting in Gothenburg, Sweden, the 2x2, 2x4 and4x2 chroma intra blocks were removed by the adoption of local dual tree (JVET-O0050). In addition, 2xN luma intra blocks resulting from ISP splits were also restricted in the last meeting. Also, in the last meeting, the restriction of 2xN chroma intra blocks was proposed for the dual-tree case in JVET-O0398. It should be noted that JVET-O0398 was not adopted at that time, because it restricts 2xN in only the dual tree case and it was suggested to further study the single tree case. It is obvious that, all above solutions do not completely remove 2xN chroma intra blocks.

# Proposed method

This document presents the results of a combination of JVET-P0596 and JVET-P0531 with the purpose of removing 2xN chroma intra blocks in both dual tree and single tree. The proposed scheme is presented as follows:

- Restriction of 2xN in dual tree

In dual tree, 2xN intra chroma is restricted by disabling some splits as proposed in JVET-O0398. Particularly, the binary tree and ternary tree splits are prohibited for blocks with width of 4 and 8, respectively.

- Restriction of 2xN in single tree

In order to remove 2xN in single tree, two restrictions are proposed including an extension of local dual tree and restriction of CIIP for chroma 2xN.

In the first restriction, if a partition with the width of 4 and the split is binary vertical split or the width of 8 and the split is ternary vertical split, it is treated as a SCIPU. Following the principle restriction of SCIPU, the chroma components are not split in intra-SCIPU (all luma blocks are encoded using non-inter mode and the non-split chroma block is coded using intra mode). For inter-SCIPU (all luma and chroma blocks are encoded using inter mode), the split of the chroma components is inherited from the luma component.

In the second restriction, for 4xN CIIP blocks, the combination intra-inter prediction is only used for the luma component while only inter prediction is used for the chroma components.

# Experimental results

The proposed method has been implemented on top of VTM6.0 and has been tested under the test conditions defined in JVET-N1010. The experimental results under AI with dual tree disabled are presented in Table 1. Table 2 shows the test results under AI and RA configuration with dual tree enabled.

With a small loss, this restriction ensures that the width of intra chroma blocks is always larger than or equal to 4, and thus 2xN pixels intra process is removed. This restriction makes the implementation of the video codec friendly for hardware implementations in terms of pipeline management and latency.

Table 1 Experimental results (Dual tree off)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.13% | -0.12% | 94% | 99% |
| Class A2 | 0.00% | -0.03% | 0.45% | 97% | 100% |
| Class B | -0.03% | 0.10% | -0.08% | 97% | 100% |
| Class C | 0.00% | -0.21% | -0.28% | 98% | 101% |
| Class E | -0.07% | -1.10% | -1.13% | 99% | 100% |
| Overall | -0.02% | -0.18% | -0.22% | 97% | 100% |
| Class D | 0.02% | 0.22% | 0.30% | 97% | 99% |
| Class F | 0.02% | 0.12% | 0.07% | 100% | 100% |

Table 2 Experimental results (Dual tree on)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.37% | 0.15% | 99% | 99% |
| Class A2 | 0.02% | 0.61% | 0.55% | 98% | 99% |
| Class B | 0.01% | 0.37% | 0.45% | 100% | 99% |
| Class C | 0.04% | 0.47% | 0.59% | 99% | 100% |
| Class E | -0.02% | -0.16% | -0.02% | 100% | 99% |
| Overall | 0.01% | 0.35% | 0.37% | 99% | 100% |
| Class D | 0.05% | 0.88% | 0.82% | 100% | 100% |
| Class F | 0.03% | 0.45% | 0.50% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.22% | 0.15% | 100% | 100% |
| Class A2 | 0.15% | 0.70% | 0.56% | 100% | 100% |
| Class B | 0.06% | 0.44% | 0.53% | 100% | 100% |
| Class C | 0.19% | 0.64% | 0.63% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.50% | 0.49% | 100% | 100% |
| Class D | 0.23% | 0.88% | 0.80% | 100% | 100% |
| Class F | 0.20% | 0.51% | 0.76% | 100% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | 0.32% | 0.35% | 99% | 98% |
| Class C | 0.24% | 0.54% | 0.47% | 100% | 96% |
| Class E | 0.03% | -0.04% | 0.83% | 100% | 100% |
| Overall | 0.12% | 0.30% | 0.51% | 100% | 98% |
| Class D | 0.25% | 1.21% | 1.35% | 101% | 99% |
| Class F | 0.46% | 0.57% | 0.87% | 99% | 98% |

# Conclusion

This document presents the results of a combination of JVET-P0596 and JVET-P0531 with the purpose of removing 2xN chroma intra blocks for both dual tree and single tree. It is recommended to adopt the proposed combination test into the next version of VTM.

# Patent rights declarations

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).