| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0644 |
| --- | --- |

| Title: | Cross-check of JVET-P0421: Non-CE8: Palette mode with 8-bit entries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Zhao 10225 Willow Creek Rd, San Diego, CA 92131, U.S.A | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

This is cross-check report for JVET-P0421 Non-CE8: Palette mode with 8-bit entries. The cross-check result match with what is provided by proponent in JVET-P0421.

# Introduction

JVET-P0421 proposes to set the bit depth of palette entries to 8 instead of the internal coding bit depth.

The software package was provided by proponent. We examined the software, and code changes are as described by the JVET-P0421 document.

# Experimental Results

Code is compiled and tested following JVET CE8 test condition using SCC444 sequences [1][2], and compared with VTM6 CE8 anchor. Results match with what is provided by proponent.

Table 1 Result of Dual Tree Off

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.46% | 0.38% | 0.43% | 101% | 100% |
| TGM 720p | 0.70% | 0.30% | 0.44% | 101% | 100% |
| Animation | 0.41% | 0.97% | 0.85% | 100% | 99% |
| Mixed content | 0.29% | 0.79% | 0.91% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.42% | 0.50% | 0.55% | 101% | 100% |

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.09% | 0.23% | 100% | 100% | 0.33% | 0.24% | 0.21% | 100% | 101% |
| Class A2 | 0.27% | 0.26% | 0.46% | 100% | 100% | 0.21% | 0.03% | 0.79% | 100% | 99% |
| Class B | 0.34% | 0.68% | 0.65% | 100% | 100% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.17% | 0.37% | 0.38% | 100% | 101% | 0.24% | 1.50% | 1.35% | 100% | 100% |
| Class E | -0.01% | 0.00% | 0.01% | 100% | 101% | 0.02% | -0.02% | -0.08% | 100% | 100% |
| Overall | 0.20% | 0.28% | 0.37% | 100% | 100% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.15% | 0.09% | 0.23% | 100% | 100% | 0.33% | 0.24% | 0.21% | 100% | 101% |
| Class F | 0.27% | 0.26% | 0.46% | 100% | 100% | 0.21% | 0.03% | 0.79% | 100% | 99% |

Table 1 Result of Dual Tree On

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.08% | 0.35% | 0.35% | 102% | 102% |
| TGM 720p | 0.16% | 0.19% | 0.26% | 102% | 102% |
| Animation | 0.11% | 0.22% | 0.21% | 101% | 101% |
| Mixed content | 0.02% | 0.18% | 0.35% | 101% | 102% |
| Camera-Captured content | 0.01% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.05% | 0.21% | 0.26% | 101% | 101% |

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.16% | 0.26% | 100% | 100% | 0.25% | 0.00% | 0.29% | 100% | 100% |
| Class A2 | 0.24% | 0.07% | 0.01% | 100% | 100% | -0.08% | -0.13% | 0.25% | 100% | 99% |
| Class B | 0.29% | 0.28% | 0.33% | 100% | 100% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.07% | 0.48% | 0.37% | 100% | 100% | -0.23% | 0.84% | 0.33% | 100% | 100% |
| Class E | -0.01% | 0.03% | -0.02% | 99% | 100% | -0.02% | 0.00% | -0.02% | 100% | 101% |
| Overall | 0.14% | 0.20% | 0.20% | 100% | 100% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.06% | 0.16% | 0.26% | 100% | 100% | 0.25% | 0.00% | 0.29% | 100% | 100% |
| Class F | 0.24% | 0.07% | 0.01% | 100% | 100% | -0.08% | -0.13% | 0.25% | 100% | 99% |

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-O0010, Gothenburg, SE, July 2019.

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” Document of Joint Video Experts Team (JVET), JVET-O2028, Jul. 2019