| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0657 |
| --- | --- |

| Title: | Crosscheck of JVET-P0292 (AHG15: Scaling process for LFNST case) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Yang Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | wangyang.cs@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-P0292 [1], which proposes to apply scaling matrices in the coefficients of LFNST output as a second scaling process.. The simulation results match with those provided in JVET-P0292.

Experimental results

JVET-P0292 was implemented on top of VTM software (version 7977eb8e). The simulations are conducted in the condition of ‘All Intra’, ‘Random Access’ with scalingLists.txt as attached in JVET-P0292. The crosscheck results of test #1 and test #2 are shown in Table 1 and Table 2.

Table 1. Result of scaling matrix on case with proposed method and LFNST enabled (Test #1)
(Test: Proposed, Anchor: VTM-6.0 CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.07% | 1.25% | -0.42% | 101% | 97% |
| Class A2 | 2.90% | 0.24% | -0.03% | 99% | 101% |
| Class B | 1.80% | -1.10% | -1.21% | 103% | 101% |
| Class C | 2.59% | 0.76% | 0.82% | 98% | 116% |
| Class E | 1.55% | 0.19% | 0.09% | 102% | 104% |
| Overall | 2.16% | 0.14% | -0.21% | 101% | 104% |
| Class D | 4.19% | 2.08% | 2.03% | 98% | 145% |
| Class F | 1.24% | 1.74% | 1.92% | 100% | 107% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.41% | 0.62% | -0.87% | 102% | 97% |
| Class A2 | -0.04% | -0.17% | -0.52% | 99% | 96% |
| Class B | 0.65% | -1.00% | -1.36% | 102% | 101% |
| Class C | 1.54% | 0.30% | -0.09% | 102% | 124% |
| Class E |  |  |  |  |  |
| Overall | 0.70% | -0.17% | -0.75% | 101% | 105% |
| Class D | 2.06% | 0.08% | 0.06% | 102% | 176% |
| Class F | 0.83% | 1.97% | 2.05% | 102% | 118% |

Table 2. Result of scaling matrix on case with proposed method and LFNST enabled (Test #2)
(Test: Proposed, Anchor: VTM60 (7977eb8e) with scaling list)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 (7977eb8e) scaling matrix enabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.12% | 102% | 100% |
| Class A2 | -0.02% | 0.14% | 0.23% | 103% | 103% |
| Class B | 0.01% | 0.08% | 0.16% | 101% | 102% |
| Class C | 0.01% | 0.15% | 0.22% | 101% | 104% |
| Class E | 0.01% | 0.23% | -0.06% | 99% | 100% |
| Overall | 0.00% | 0.11% | 0.14% | 101% | 102% |
| Class D | 0.00% | 0.17% | -0.04% | 101% | 111% |
| Class F | 0.03% | 0.08% | 0.23% | 100% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 (7977eb8e) scaling matrix enabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.06% | 0.00% | 101% | 101% |
| Class A2 | -0.03% | -0.05% | 0.05% | 101% | 101% |
| Class B | 0.00% | 0.24% | 0.15% | 101% | 103% |
| Class C | 0.01% | 0.26% | 0.25% | 101% | 109% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.15% | 0.13% | 101% | 104% |
| Class D | 0.00% | 0.21% | -0.02% | 101% | 121% |
| Class F | 0.01% | 0.10% | 0.07% | 100% | 107% |

References

T. Hashimoto, E. Sasaki, T. Ikai (Sharp), “AHG15: Scaling process for LFNST case”, JVET-P0292, October 2019, Geneva, CH.