| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0673-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-P0090 (Non-CE4: 32 bits coding for abs_mvd_minus2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 jenny.lai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-P0090 proposed by Qualcomm. JVET-P0090 proposes two methods to limit the worst-case code length to 32 bits for coding the syntax element abs_mvd_minus2 in VVC. The verification task was done, and the BD-rate results match exactly with those in JVET-P0090.

# Verification

Qualcomm proposed to limit the worst-case code length to 32 bits for coding the syntax element abs_mvd_minus2 by two methods [1]. In method 1, the MVD resolution of 1/16 in affine AMVR is replaced with 1/8, then the MVD range is changed to [-216, 216-1]. Therefore, EG1 coding of abs_mvd_minus2 is within 32 bits. And in method 2, a new binarization scheme is applied for abs_mvd_minus2. For the value range from 0 to 131069, the EG1 is applied as in current VTM. For the value of 131070, it’s code by truncated unary prefix with 16 bit suffix, i.e. 1111 1111 1111 1111 0000 0000 0000 0000.

Two tests were conducted under the common test conditions (CTC) defined in [2] using the VTM6.0 software [3]. Table 1 shows the results using VTM6.0 as the anchor and enabling the proposed Method 1. Table 2 shows the results using VTM6.0 as the anchor and enabling the proposed Method 2. The BD-rates of the test from our experimental results match exactly those in JVET-P0090. Please note that the runtime comparison may be not accurate.

Table 1. The results of Method 1 in JVET-P0090 using VTM6.0 as anchor

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.07% | -0.06% | 102% | 105% |
| Class A2 | -0.01% | 0.00% | 0.03% | 102% | 102% |
| Class B | -0.01% | 0.08% | -0.06% | 102% | 99% |
| Class C | -0.01% | 0.03% | 0.02% | 100% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.05% | -0.02% | 101% | 102% |
| Class D | -0.03% | 0.02% | 0.02% | 99% | 103% |
| Class F | -0.03% | -0.07% | 0.01% | 100% | 103% |

Table 2. The results of Method 2 in JVET-P0090 using VTM6.0 as anchor

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 105% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 104% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 97% | 94% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 100% |

# References

[1] H. Huang, M. Coban, H. Wang, V. Seregin, W.-J. Chien, M. Karczewicz, “Non-CE4: 32 bits coding for abs_mvd_minus2,” Document of Joint Video Experts Team, JVET-P0090.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[3] VTM6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).