| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0676-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-P0457 (Non-CE8: On IBC merge list size signalling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 yuling.hsiao@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-P0457 proposed by Tencent. JVET-P0457 proposes three methods to address the mismatch between spec and software related to merge_idx signalling. The verification task was done, and the BD-rate results match exactly with those in JVET-P0457.

# Verification

Tencent provided the source code of their proposed methods described in JVET-P0457 [1]. Two verifications were carried out by setting the following parameters in the provided code:

Test 1 (with MaxNumMergeCand=6 and MaxNumIBCMergeCand=5):

#define IBC_MERGE_SIZE_FIX       1 // fix ibc merge size issue

#if IBC_MERGE_SIZE_FIX

#define IBC_MERGE_SIZE_FIX_S1    1 // solution 1: constraint ibc merge size <= inter merge size

#define IBC_MERGE_SIZE_FIX_S2    0 // solution 2: set merge idx range = max(ibc merge size, inter merge size)

#endif

Test 2 (with MaxNumMergeCand=5 and MaxNumIBCMergeCand=6):

#define IBC_MERGE_SIZE_FIX       1 // fix ibc merge size issue

#if IBC_MERGE_SIZE_FIX

#define IBC_MERGE_SIZE_FIX_S1    0 // solution 1: constraint ibc merge size <= inter merge size

#define IBC_MERGE_SIZE_FIX_S2    1 // solution 2: set merge idx range = max(ibc merge size, inter merge size)

#endif

Test 3:

Align spec to software. No BD-rate changes.

The tests are conducted under the common test conditions (CTC) defined in [2] and are carried out by using the VTM6.0 software [3]. Table 1 and Table 2 show the results using VTM6.0 as the anchor and enabling the proposed methods in the tests. The BD-rates of the test from our experimental results match exactly those in JVET-P0457. Please note that the runtime comparison may be not accurate.

Table 1. The results of test 1 in JVET-P0457 using VTM6.0 as anchor and setting MaxNumMergeCand=6 and MaxNumIBCMergeCand=5

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0, Merge=6, IBC=5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.01% | -0.13% | -0.01% | 100% | 101% |
| Class SCC | 0.01% | 0.06% | 0.04% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0, Merge=6, IBC=5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.07% | -0.08% | -0.17% | 102% | 102% |
| Class SCC | -0.07% | -0.09% | -0.02% | 101% | 99% |

Table 2. The results of test 2 in JVET-P0457 using VTM6.0 as anchor and setting MaxNumMergeCand=5 and MaxNumIBCMergeCand=6

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0, Merge=5, IBC=6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.06% | -0.03% | 0.05% | 100% | 98% |
| Class SCC | -0.01% | 0.00% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0, Merge=5, IBC=6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.01% | -0.01% | -0.24% | 99% | 100% |
| Class SCC | 0.09% | 0.08% | 0.13% | 100% | 97% |

# References

[1] X. Xu, X. Li, and S. Liu, “Non-CE8: On IBC merge list size signaling,” Document of Joint Video Experts Team, JVET-P0457.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[3] VTM6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).