| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0687-v5 |
| --- | --- |

| Title: | AHG17: A summary of HLS proposals on access unit delimiter, picture header, and slice header parameters signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hendry San Diego, CA, USA | Email: | fhendry@futurewei.com |
| Source: | Futurewei Technologies Inc. |  |  |

_____________________________

# Abstract

This contribution provides a summary of high-level syntax proposals submitted to this JVET meeting related to access unit delimiter, picture header, and slice header parameters signalling. A list of design questions is provided to help structure the review of the proposals.

In v2 version of this contribution, JVET-P0152 and JVET-P0605 were included. These two contributions were not registered to be part of AHG17 but upon review by CE4 coordinator, they were suggested to be included in this category as the content of the contributions are about signalling of constant slice header parameters.

In v3 version of this contribution, JVET-P0146 and JVET-P0420 were included as they are asserted to contain aspects related to AUD.

In v5 version of this contribution, JVET-P0380 was considered and some bullets were added.

# List of questions

The contributions in this category can be classified into two sub-categories:

- AUD and Picture Header

- Modifications to constant slice header paramater signalling (related to constant_slice_header_params_enabled_flag)

General questions:

- Do we want to avoid signalling syntax elements in slice header that are constrained to have same values for all slices of a picture?

  - If yes. Where to signal them? AUD? New NAL Picture Header? Use the constant slice header params signalling in PPS?

- For constant slice header parameters, should some of the syntax elements be in PPS? or in AUD? or in Picture Header, if exists?

On AUD and Picture Header.

- Do we want to have Picture Header?

  - Yes. JVET-P0095, JVET-P0120, JVET-P0239

Why? 
JVET-P0092 & JVET-P0120: AUD is AU specific. An AU may contain multiple pictures.

JVET-P0120: Order of AUD is supposed to the first NAL unit in an AU. If AUD preceeds SPS & PPS, some picture-level syntax elements in SH cannot be moved to AUD. Having PH is asserted would solve this problem.

- If yes, what to signal in PH?

  - Move some signalling from PPS to PH

    - JVET-P0095: Slice Id (and Subpicture Id, if it is signalled in PPS)

  - Signal syntax element both in PPS and PH. Present in PH if not present in PPS

    - JVET-P0095: Tile/Brick/Rect Slice signalling

  - Move some signalling from SH to PH

    - JVET-P0095, P0120: no_output_of_prior_pics_flag

    - JVET-P0120, P0239: slice_pic_parameter_set_id, non_reference_picture_flag, colour_plane_id, slice_pic_order_cnt_lsb, recovery_poc_cnt, pic_output_flag, slice_temporal_mvp_enabled_flag.

    - JVET-P0239: slice_lmcs_aps_id and slice_scaling_list_aps_id

    - JVET-P0239 move some syntax elements that are currently not constrained to be the same for all slices of a picture, but asserted having no / minimal benefit and coding loss to transmitting them in every slice header as their anticipated usage would change at the picture level.

  - Signal some syntax element that are not constrained to be the same for all slices of a picture both in PH and SH. Present in SH if not present in PH

    - JVET-P0120: RPL, joint cb/cr sign flag, SAO, ALF, LMCS, Scaling list

  - New syntax elements:

    - JVET-P0095: A flag to specify whether associated picture contains mixed NAL unit types (i.e, mixed_nalu_types_in_pic_flag). See JVET-P0124, JVET-P0146, and JVET-P0222.

    - JVET-P0120: Picture type (whether picture is IDR, CRA, GDR, contains only I-slices, only P- and I- slices, any type of slices)

- If yes, what is property of PH?

  - JVET-P0095, JVET-P0120, JVET-P0239: Mandating one PH one picture

  - JVET-P0120: Specifies layer Id, temporal Id, order of PH in access unit, etc.

  - JVET-P0095, JVET-P0120: PH is a non-VCL NAL unit.

  - JVET-P0239: PH is a VCL NAL unit.

- Do we want to keep AUD?

  - No: JVET-P0095, JVET-P0120

- Do we want to keep mandating the presence of AUD?

  - No.

JVET-P0218. Signal single_slice_in_pic_flag in SPS. When single_slice_in_pic_flag is equal to 1, AUD is not mandated to be present. single_slice_in_pic_flag can be used to condition the presence of other syntax elements such as num_bricks_in_slice_minus1.

JVET-P0367: Signal a flag aud_in_next_access_unit_present_flag in AUD to specify if AUD is present for next AU.

JVET-P0480: AUD is mandated only when there are more than 1 subpicture. Signal first_slice_in_sub_pic_flag in SH. This flag can be used for detecting first slice of each picture when there is only one subpicture in a picture.

- Do we want to signal more syntax elements in AUD?

  - JVET-P0218: Optionally moves PPS Id, non_reference_picture_flag, colour_plane_id, slice_pic_order_cnt_lsb, and pic_output_flag to AUD, if present. Create a syntax structure pic_header(). NOTE: pic_header is not in its own NAL unit.

  - JVET-P0146: Signal syntax elements which specifies types of subpictures of a picture (non-IRAP, IDR, CRA). This would allow a picture to contain mixed subpicture types.

  - JVET-P0222: Signal rap_type and au_order_cnt. Syntax element rap_type specifies which specifies whether the coded pictures in the access unit contain non-IRAP NAL unit only, IRAP NAL unit only, or both IRAP and non-IRAP.

Comments: JVET-P0146 and first aspect of JVET-P0222 suggest to allow mixed of IRAP and non-IRAP in a picture. They should be discussed together with JVET-P0095 and JVET-P0124.

- AUD properties:

  - JVET-P0218: The AUD shall have nuh_layer_id value equal to vps_layer_id[ 0 ].

  - JVET-P0380: The AUD shall have nuh_layer_id value equal to 0.

  - JVET-P0218: The AUD shall have TemporalId equal to the TemporalId of the access unit containing the NAL unit

On signalling constant slice header params.

- Some bugs are asserted to be present in the signalling of syntax element when constant_slice_header_params_enabled_flag?

  - JVET-P0152:

    - Update semantics of pps_five-minus_max_mun_subblock_merge_cand_plus1 to incorporate the value of sps_affine_enabled_flag.

Comment: The proposed changes seem to be more appropriate to be expressed as constraints.

    - Update the inference of the value five_minus_max_num_subblock_merge_cand in SH when it is not present taking into account the proposed change above.

  - JVET-P0427 proposal 1: Condition the signalling of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 on the value of pps_six_minus_max_num_merge_cand_plus1

  - JVET-P0427 proposal 2: Update the range value of pps_max_num_merge_cand_minus_max_triangle_cand_plus1 to not depend on the value of MaxNumMergeCand

  - JVET-P0427 proposal 3: MaxNumTriangleMergeCand is inferred to be equal to 2 in slice header if MaxNumMergeCand is equal to 2 without signaling in either PPS or slice headers

  - JVET-P0605: proposed to modify pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 to pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1

This has been addressed by editor between last meeting and this meeting. The update is reflected in JVET-P0113

- Adding more syntax element to be signalled in PPS when constant_slice_header_params_enabled_flag is equal to 1:

  - JVET-P0334 proposal / modification 1

  - JVET-P0368

  - JVET-P0428 proposal 1

- Improvement for signalling mechanism of constant slice header params:

  - JVET-P0334 proposal 2 & 3:

    - Instead of signalling 1 set of constant slice header params in PPS, allow signalling up to 7 sets of constant slice header params in PPS. In SH, signal index of the set to be used.

    - When the number of constant slice header params set in PPS is equal to 0, signal the syntax elements in SH

  - JVET-P0428 proposal 2: signal conformance_constant_params_enabled_flag in SH to specify if the slice uses the constant syntax elements signalled in PPS or not. This allow constant slice params to be present in the PPS but also present in SH (i.e., when the value of conformance_constant_params_enabled_flag is equal to 0)

# List of contributions (14)

JVET-P0095 AHG12/AHG17: On signalling of picture-specific syntax elements in access unit delimiter [M. M. Hannuksela (Nokia)]

JVET-P0120 AHG17: On access unit delimiter and picture header [Hendry, Y.-K. Wang (Futurewei)]

JVET-P0146 AHG17/AHG12: On AUD as a picture header [L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0218 AHG8/AHG17: On Access Unit Delimiter and Picture Detection [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

JVET-P0222 AHG17: Parameters in AUD [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-P0239 AHG17: Picture Header [W. Wan, T. Hellman, B. Heng (Broadcom)]

JVET-P0334 AHG17: On constant slice header parameter set in PPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

JVET-P0367 AHG17: On AUD signaling [M. Pettersson, R. SjÃ¶berg, M. Damghanian (Ericsson)]

JVET-P0368 AHG17: On selectively signal slice header parameters in PPS [M. Pettersson, R. SjÃ¶berg, M. Damghanian (Ericsson)]

JVET-P0380 AHG8/AHG17: On the role of nuh_layer_id in the layer design [E. Thomas (TNO), M. Hannuksela (Nokia), L. Chen (MediaTek), Yong.He@InterDigital.com, S. Wenger, B. Choi (Tencent)]

JVET-P0480 AHG17: On simplification of subpicture design [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI]

Aspect 4 of the contribution is related to the presence of access unit delimiter in access unit.

JVET-P0427 AHG17: Bugfixes on constant slice header parameters [Y.-J. Chang, V. Seregin, M. Coban, A. Ramasubramonian, M. Karczewicz (Qualcomm)]

JVET-P0428 AHG17: Improvement on signaling of constant slice header parameters [Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-P0152 CE4-related: On maximum number of subblock-based merging candidates [O. Chubach, C.-C. Chen, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-P0605 Non-CE4: On triangular merge list size signalling [T. Solovyev, S. Ikonin, A. Karabutov, R. Chernyak (Huawei)]